3GPP TSG-RAN4 Meeting #116	R4-2510297

# Bangalore, India, 25 Aug - 29 Aug, 2025

Agenda item:	 7.24.3.4

Source:	 vivo

Title:	Proposals on concluding LP-WUS Testability

Document for:	 Approval

- Introduction

Issue 4-1-1: How to verify LP-WUR

Agreement:

Verify LP-WUR based on 1% MDR of LP-WUS which can be tested based on UE’s response to the NW/TE upon successfully detecting the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or other methods)

- The ultimate test method including the potential test mode will be decided by RAN5

Issue 4-1-2: FR2 skipping rule

WF:

- Further check whether FR2 LP-WUR spherical coverage can be skipped.

Issue 4-1-3: skipping rule for LP-WUR different mode

Agreements:

- For idle-mode only or connected-mode only LP-WUR, the test should be done accordingly.

- FFS For LP-WUR support both.

Last meeting, RAN4 agreed the framework on how to verify LP-WUR RF requirements, but the detailed test method and potential test mode will be decided by RAN5. Besides, from RAN4 perspective some skipping rule of RF requirement testing is needed, but no further consensus reached [1].

Given this is the last meeting of this Work Item, potential conformance testing WI will be introduced in RAN5#119. To provide necessary information to specify the conformance test case, it is valuable to send an LS to RAN5 on RAN4 testability agreements.

In this contribution, we share our views on concluding LP-WUS UE RF testability and a draft LS is prepared.

- LP-WUS UE RF testability

For FR2 spherical coverage, most likely the LR and MR will share the same antenna module and RF chain, it is reasonable to skip the spherical coverage testing for LP-WUS if the LR reference sensitivity requirement can meet.

Proposal 1: FR2 LP-WUR spherical coverage can be skipped if the reference sensitivity requirements can meet.

For the LR support both idle-mode and connected-mode operation, single test case is sufficient to verify the LR UE RF chain performance.

Proposal 2: For LP-WUR support both idle-mode and connected-mode, RF requirements testing under connected-mode operation is sufficient.

Regarding the testing of type-1 requirement or type-2 requirements, this is based on UE declaration whether it is type 1 UE or type 2 UE.

Proposal 3: Type-1 or Type-2 conformance test case is based on UE declaration of corresponding requirements (LR type).

More important, the framework on how to verify LP-WUS UE RF requirements has been concluded last meeting, but the detailed test procedure and UE test mode is left for RAN5 discussion. A draft LS is prepared in the Annex part, to provide RAN4 agreements to RAN5.

Proposal 4: Send an LS to RAN5 on LP-WUS UE RF testability conclusions.

- Conclusion

In this contribution, we prepare a draft LS to RAN5 in the Annex part.

Proposal 1: FR2 LP-WUR spherical coverage can be skipped if the reference sensitivity requirements can meet.

Proposal 2: For LP-WUR support both idle-mode and connected-mode, RF requirements testing under connected-mode operation is sufficient.

Proposal 3: Type-1 or Type-2 conformance test case is based on UE declaration of corresponding requirements (LR type).

Proposal 4: Send an LS to RAN5 on LP-WUS UE RF testability conclusions.

Draft LS in the Annex.

# References

- R4-2507936, WF for [115][136] NR_LPWUS_UERF, vivo, RAN4#115.

# Annex: draft LS to RAN5 on LP-WUS testing

3GPP TSG-RAN WG4 Meeting #116	R4-250xxxx

Bengaluru, India, August 25th – 29th, 2025

Title:	LS to RAN5 on LP-WUS UE RF testability

Response to:

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	RAN4

To:	RAN5

Cc:

Contact Person:

#### Name:	Ruixin Wang

#### E-mail Address:	ruixin.wang@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN4 discussed the testability of LP-WUS UE RF requirements and reached the following agreements:

- Verify LP-WUR based on 1% MDR of LP-WUS which can be tested based on UE’s response to the NW/TE upon successfully detecting the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or other methods)

  - The ultimate test method including the potential test mode will be decided by RAN5

- For FR1 LR, whether UE is type-1 or type-2 is based on UE declaration, the corresponding test case should be aligned with the RF requirements (UE type) declaration.

- FR2 LP-WUR spherical coverage can be skipped if the LR reference sensitivity requirement is met.

- For idle-mode only or connected-mode only LP-WUR, the test should be done accordingly. For LP-WUR support both idle-mode and connected-mode, RF requirements testing under connected-mode operation is sufficient

- Consider testing time reduction when specify LP-WUS conformance test case.

RAN4 would like to ask RAN5 to take above agreements into account when develop conformance test case of LP-WUS.

2. Actions:

To RAN5

ACTION: 	RAN4 respectfully asks RAN5 to take above information into consideration when specify conformance test case.

3. Date of Next TSG-RAN4 Meetings:

3GPP TSG RAN WG2#116bis	13-17 October 2025	Prague, Czech Republic