3GPP TSG-RAN WG4 Meeting #116	R4-2510300

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Fundamental RX requirements for LP-WUR

Agenda item:	7.24.3.2

Document for:	Discussion

Introduction

In this document we present our views on open issues regarding REFSENS, ACS, and ASCS requirements. Following agreements were made in the last RAN4 meeting [1].

# Topic #2: REFSENS, ASCS and ACS requirements ### Sub-topic 2-1 FR1 SNR and REFSENS requirements Issue 2-1-1: SNR for LP-WUR two types Agreements: - There is no repetition of information bits to derive OFDM-based receiver SNR. - SNR=-4.5dB for envelope detection and OFDM-based receiver. Issue 2-1-2: Whether update the equation for REFSENS Agreements: - Use legacy REFSENS approach to derive requirements in spec. For TR, some statement of this issue may be presented. Issue 2-1-3: RMC for LP-WUR requirements Agreements: - In the FR1 spec, there should be two tables for 15kHz and 30kHz; - The following parameter is agreed. Further adding other parameter (if necessary) is not precluded. - Table A.3.4.2-1 Fixed reference channel for LP-WUR requirements Parameter; Unit; Value. MR Channel bandwidth; MHz; All CBW. LP-WUS bandwidth; RB; 11. Subcarrier spacing; kHz; 15/30kHz. RM coding; bits; 16bits. CRC; No CRC. Chip rate; M=4 (4 chips in an OFDM symbol). overlaid OFDM sequence; Length 33: generated by 31-length ZC sequence with extension. Number of overlaid OFDM sequence per chip to carry information; 4. WUS duration for OOK; 8 OFDM symbols. WUS duration for OFDM; 2 OFDM symbols. Manchester coding for OOK; 1/2. Number of information bits; 5bits. Issue 2-1-6: UE feature for LP-WUS WF: - FFS if RAN4 will specify capability (or capabilities) for the two different types of LP-WUR. ### Sub-topic 2-2 FR2 SNR and REFSENS requirements Issue 2-2-1: Combining strategy for FR2 LP-WUR Agreements: - The combining strategy of FR2 LP-WUR is selection-based diversity for the two Rx chains with orthogonally polarized antennas, just for requirement development perspective. Issue 2-2-2: Diversity gain value for FR2 LP-WUR Agreements: - -3dB for diversity gain. - FFS: 1% MDR REFSENS requirement should be satisfied for each orthogonal link polarization respectively. Issue 2-2-3: Peak direction for FR2 LP-WUR Agreements: - All FR2 directional LPWUS requirements apply in the same direction as the corresponding legacy Rx requirement. Issue 2-2-4: NF+IM for FR2 LP-WUR Agreements: - For FR2, NF+IM is [12] dB for at least type-2 LP-WUR. - FFS two values for two types. Issue 2-2-5: Spherical coverage for FR2 LP-WUR WF: - FFS FR2 LP-WUR spherical coverage whether can be skipped. Issue 2-2-6: other Rx requirements for FR2 LP-WUR WF: - FFS whether skip out-of-band blocking and spurious response requirements for FR2 LP-WUR. ### Sub-topic 2-3 ASCS requirements Issue 2-3-1: proposals to update the agreed ASCS requirements for LP-WUR last meeting WF: - Prepare Draft CR directly based on last meeting agreement of ASCS. Issue 2-3-2: whether high in-band signal (both LP-WUS and NR) case is considered for ASCS requirements Agreements: - Keep using low power level for ASCS, but use Max input power requirement to verify CBW with full NR+LP-WUS RBs. ### Sub-topic 2-4 ACS simulation and requirements Issue 2-4-1: Baseline ACS requirements (same interference) and required guard RB for LP-WUR Agreement: - 0 guard RB is assumed for deriving both type-1 and type-2 ACS requirement. Issue 2-4-2: case 1 ACS test parameter WF: - Finalize detailed test parameters next meeting. endorse the definition and clause structure, requirement table (with requirement TBD) this meeting. Issue 2-4-3: case 2 ACS test parameter WF: - Finalize detailed test parameters next meeting. endorse the definition and clause structure, requirement table (with requirement TBD) this meeting.

Discussion

REFSENS value

During the last meeting, both the SNR and NF values were agreed upon, however the final REFSENS numbers were not agreed and left as TDB in the draft big CR. The contention is related to the NF values for bands with frequencies higher than 3.3 GHz as till now RAN4 has discussed NF and SNR values assuming a maximum operating frequencies of 2.6 GHz.

In order to define the NF for bands with higher than 3.3GHz frequency, one possible way is to have a frequency range specific delta. The difference between the worst REFSENS for bands with frequency less than 3.3 GHz can be compared with the worst case REFSENS for bands with higher than 3.3 GHz frequency and that can be used as a frequency range specific delta. Based on the current specification after removing the outliers, this number is 0.5 dB.

The delta between the worst case REFSENS for the bands with frequency higher and lower than 3.3 GHz is 0.5 dB.

Agree to split the REFSENS table for LR into two based on the frequency of operation i.e., one set of REFSENS for LR operating with FDL_high < 2700 MHz and another set of REFSENS for LR operating with FDL_high > 3300 MHz.

Agree to relax the REFSENS for LR operating with FDL_high > 3300 MHz by 0.5 dB.

Conclusion

The following Observations and Proposals were made:

Observation 1: The delta between the worst case REFSENS for the bands with frequency higher and lower than 3.3 GHz is 0.5 dB.

Proposal 1: Agree to split the REFSENS table for LR into two based on the frequency of operation i.e., one set of REFSENS for LR operating with FDL_high < 2700 MHz and another set of REFSENS for LR operating with FDL_high > 3300 MHz.

Proposal 2: Agree to relax the REFSENS for LR operating with FDL_high > 3300 MHz by 0.5 dB.

- References

- R4-2507936, WF for [115][136] NR_LPWUS_UERF, Vivo, RAN4#115, Malta, MT, 19th – 23rd May, 2025.

- R4-2505234, WF for [114bis][131] NR_LPWUS_UERF, Vivo, RAN4#114bis, Wuhan, China, 7th – 11th April, 2025.

- R4-2503028, WF on UE RF requirements for LP-WUS, Vivo, RAN4#114, Athens, Greece, 17th – 21st Feb, 2025.

- R4-2415707, Fundamental RX requirements for LP-WUR, Nokia, RAN4#112bis, Hefei, China, 14th – 18th October, 2024.