3GPP TSG-RAN WG4 Meeting #116	R4-2510302

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Testability aspects of LP-WUR

Agenda item:	7.24.3.4

Document for:	Discussion

Introduction

In this document we present our views on testability aspects of the LP-WUR. Following agreements were made in the last RAN4 meeting [1].

### Sub-topic 4-1 Testability for UE RF requirements Issue 4-1-1: How to verify LP-WUR Agreement: Verify LP-WUR based on 1% MDR of LP-WUS which can be tested based on UE’s response to the NW/TE upon successfully detecting the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or other methods) - The ultimate test method including the potential test mode will be decided by RAN5 Issue 4-1-2: FR2 skipping rule WF: - Further check whether FR2 LP-WUR spherical coverage can be skipped. Issue 4-1-3: skipping rule for LP-WUR different mode Agreements: - For idle-mode only or connected-mode only LP-WUR, the test should be done accordingly. - FFS For LP-WUR support both.

Discussion

RF Test cases to be executed

For the RF performance testing there are the following test cases to be considered:

- RF Sensitivity test defined by the missed detection Rate of 1%

- RF blocking test (IBB and OBB)

- Adjacent channel suppression performance

- Frequency and time stability

- All/ some test should be tested in nominal and extreme temperature

As it can be seen there are quite many test cases and it is therefore important to be able to execute the test in reasonable test time.

There are many RF test cases, and it is therefore important to have a short test time/ fast test.

Test procedure for the LP-WUR RF performance parameters

The test procedure must make sure that it is testing only the LP-WUR performance and not the combined performance of MR and LP-WUR. This might require using high SINR for the MR whenever there is communication between TE and MR. However, designing detailed test cases comes under RAN5 responsibility.

Test time in idle based on msg. 1/ 3 to test equipment

To verify the performance of the LP-WUR in idle based on 1% MDR the number of LP-WUS detections/ responses should be in the range of 1000. (factor of 10x to achieve good accuracy of the MDR). The steps for responding to a received LP-WUS dedicated to the device is as follows:

- Perform decoding of the WUS

- Wakeup the Main Radio (MR)

- Send MSG. 1(PRACH), receive msg. 2 for time sync, transmit msg3, establish connection

- Terminate connection

- Enter LP-WUS/WUR connection

- Goto sleep

The time for this procedure can be in the range of app. 10 sec and should be repeated app. 1000 times. This gives a test time of almost 3 hours.

The similar procedure can be followed when the device is under test for FAR. In this case a failing scenario is when the device is responding when not expected.

Assuming a FAR of max 1% as with MDR, this gives a total test time of app. 6 hour. As this test is containing both LR and MR activity any given error can be due to other actions than the actual test purpose – test and verification of the demodulation of the LP-WUS, it would be beneficial to have a test mode and thereby minimizing the test time and reduce the activity and errors there could come from MR activity.

RAN5 defines the detailed test procedure for the conformance tests.

A higher SNR for the main radio signals can be used, so that just the performance of the LP-WUR is tested.

Introduce a test mode to have test time in few minutes instead of several hours for test without test mode.

Test mode

As the LP-WUR is going to be a receiver which has no direct external interface to the test equipment (TE) as it can only talk with TE via the main radio (MR) unless there is a test mode to enable LP-WUR to enable such functionality. A test mode will help in simplifying the test procedure and will facilitate faster testing. In test mode the DUT can count the received and decoded LP-WUS sequences and return the result via the MR to the TE. This means the test containing 1000 measurements can be executed in 1000 x 312 ms= 5.2 minutes instead of 6 hours test time without test mode. Further, as the WUS functionality will be presented in all RRC modes, the test mode should support all RRC states. This will also help in testing the efficiency of LP-SS. The RRM test mode can be further discussed in the RRM session. However, how to design the test mode and how that will work in RRC_IDLE/RRC_INACTIVE and RRC_CONNECTED mode can be left for RAN5 to define with RAN4 input.

Figure 1 Illustration of RF performance test setup

RAN5 can design the details of Test mode required to test the LP_WUR with RAN4 input.

Test mode shall support RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED states.

Testing time

To achieve a ≤1% miss detection rate (MDR) of LP-WUS in a statistically confident way will take long time. The total time taken to have a statistically significant value depends on multiple factors such as test case execution time, TE setup and response time, UE feedback time, repetition cycle, waiting time between repetition, etc. As the motivation to have a higher MDR (≥1%) is to save on conformance test time, it is best for RAN5 to decide the confidence interval for the requirements in order to test them in a reasonable amount of time.

Only motivation to have a higher (≥1%) MDR is to save conformance test time.

Lot of factors impacting the time taken for test case execution are under the scope of RAN5 work.

RAN5 can decide the confidence level to be used for testing the LP-WUR requirements in a reasonable amount of time.

Conclusion

The following Observations and Proposals were made:

Observation 1: There are many RF test cases, and it is therefore important to have a short test time/ fast test.

Proposal 1: RAN5 defines the detailed test procedure for the conformance tests.

Proposal 2: A higher SNR for the main radio signals can be used, so that just the performance of the LP-WUR is tested.

Proposal 3: Introduce a test mode to have test time in few minutes instead of several hours for test without test mode.

Proposal 4: RAN5 can design the details of Test mode required to test the LP_WUR with RAN4 input.

Proposal 5: Test mode shall support RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED states.

Observation 2: Only motivation to have a higher (≥1%) MDR is to save conformance test time.

Observation 3: Lot of factors impacting the time taken for test case execution are under the scope of RAN5 work.

Proposal 6: RAN5 can decide the confidence level to be used for testing the LP-WUR requirements in a reasonable amount of time.

- References

- R4-2507936, WF for [115][136] NR_LPWUS_UERF, Vivo, RAN4#115, Malta, 19th – 23rd May, 2025.