3GPP TSG-RAN WG4 Meeting #116	R4-2510306

Bengaluru, India, 25th August 2025 – 29th August 2025

Source:	Nokia

Title:	RedCap NTN UEs

Agenda item:	7.27.3.1

Document for:	Discussion

Introduction

Following agreements were made in the last RAN4 meeting [1].

## Issue 1-1: Output power enhancement for NR NTN PC3 Agreements: - If defined, additional output power enhancement will be optional for UE to support. - FFS on need of new UE capability - Evaluate potential to further enhance output power for RAN4#115: - Amount of output power enhancement [0.5 to 1 dB] - Less than 0.5 dB improvement is not specified - Initially consider all RB allocations and modulation orders - Consider both MPR reduction and MOP increase - FFS how to capture MOP increase in the specification - Choice between MPR reduction and MOP increase can be different for inner/outer allocations and modulation orders - Evaluate both HD-FDD and FD-FDD - Consider at least insertion loss impact - FFS whether to apply same enhancement to both HD-FDD and FD-FDD - Evaluate the option to support both rel-18 and rel-19 output power enhancement schemes simultaneously - FFS whether simultaneous support is specified - Consider whether PC3 or PC2 ACLR requirement should apply - FFS Applicability to PC3 non-RedCap NR NTN ## Issue 1-3: draft CR Agreement: - Consider draftCR in R4-2504551 as baseline for further improvements and possible additions

Further, RAN4 has received a reply LS from RAN2 regarding simultaneous GNSS operation with NR, with the contents copied below.

1. Overall Description: RAN2 would like to thank RAN4 for the original LS in R4-2419898. RAN2 has considered the simultaneous operation between GNSS and UL transmission in NR NTN and concluded not to work on the enhancements to the existing IDC solutions in Release 18. RAN2 might consider the extension to IDC TDM solution or autonomous denial solution in Release 19, and would like to receive more information on requirements, e.g. on the periodicity and duration of IDC free time to help the UE read the GNSS in NTN. 2. Actions: To RAN4: ACTION: RAN2 respectfully asks RAN4 to take the aforementioned RAN2 decisions into account and provide the corresponding requirements, e.g. the periodicity and duration of IDC free time to help the UE read the GNSS.

In this paper we provide our views on the simultaneous operation of the RedCap NTN UE with GNSS.

Discussion

Simultaneous operation with GNSS

As discussed during several previous meetings, it is agreeable that there are technical challenges regarding parallel operation of some GNSS variants with NR NTN in some bands. Neither all GNSS bands, nor all UEs suffer i.e., depending on UE implementation, and the GNSS signals (and frequencies) available in a given location, not all UEs will be harmed by this collision all the time.

Not all UEs will be harmed by the potential interference between UL and GNSS frequencies.

According to the RAN2 reply, they can consider extension to the IDC TDM or autonomous denial solution in Release 19 only and will not consider any enhancements in Release 18. Given RAN2 stand on older release, it makes sense to not do any further work regarding this issue in Release 18 in RAN4 too.

RAN2 will consider enhancements to the IDC framework in Release 19 only. No changes will be done for Release 18.

RAN4 shall only consider any changes regarding this issue in Release 19.

RAN2 also indicates that for although they are willing to enhance the IDC features for NTN UEs in Release 19, they will only do so if RAN4 provides proper guidance about the values to be used. Given that FDM IDC solution does not help in this particular co-existence issue, we will not discuss this. The IDC feature is composed by two important IEs as described by TS 38.331:

- AutonomousDenialParameters

- IDC-TDM-Assistance-r18

The first IE configures the autonomous denial at the UE side. It contains the following parameters and allows the following values:

AutonomousDenialParameters-r18 ::= SEQUENCE { autonomousDenialSlots-r18 ENUMERATED {n2, n5, n10, n15, n20, n30, spare2, spare1}, autonomousDenialValidity-r18 ENUMERATED {n200, n500, n1000, n2000}

The UE is allowed to deny UL transmissions according to the values above, when configured. The procedure goes as follows:

1> if the CellGroupConfig contains the autonomousDenialParameters: 2> consider itself to be allowed to deny any transmission in a particular UL slot if during the number of slots indicated by autonomousDenialValidity, preceding and including this particular slot, it autonomously denied fewer UL slots than indicated by autonomousDenialSlots within the same cell group;

As can be seen above, the UE is allowed to drop up to 30 slots (30 ms in 15 kHz and 15 ms in 30 kHz). This is a significantly limited span of time if compared to the total time needed to update a GNSS signal.

Also, from the LTE study TR 36.816, it is understood that Autonomous denial solution is more suitable for successful reception of LTE signalling but its applicability and acceptability is questionable for steady state events. Thus, we propose not to increase the number of denied slots.

Agree not to increase the value of autonomousDenialSlots-r18.

From TR 36.816:

##### 5.2.2.1.1 LTE denials for infrequent short-term events UE can autonomously deny LTE resources due to some critical short-term events of ISM side, e.g. some events during BT/WiFi connection-setup or other important signalling. Otherwise, large delay or failure of connection-setup could happen if these events are not prioritized over LTE. This solution is assumed to be used for the event that rarely takes place. Potentially, requirements on the frequency and duration of denials would need to be defined if such a solution would be adopted. The analysis indicates that autonomous LTE denial at the UE, i.e. UE occasionally skipping an LTE UL transmission without any limitation is not acceptable due to its impact on LTE performance, especially on PDCCH link adaptation accuracy and PDCCH capacity [19]. It is FFS whether autonomous LTE denial with further enhancement, e.g. the UE would have to provide additional assistant information to the network, is needed to handle rare periodic or non-periodic events. Editor’s note: The feasibility and usefulness of this solution need further study. ##### 5.2.2.1.2 LTE denials for ISM data packets During stable situation of ISM operation, some LTE resources can be denied by UE autonomously to protect ISM data packets, so e.g. the BT eSCO connection or WiFi connection with PS-Poll can be maintained. The UE can feedback the denial pattern to the eNB, or the eNB can learn the pattern used by the UE based on DTX and other implementation specific solutions. An example of this solution is shown in Figure 5.2.2.1.2-1. Figure 5.2.2.1.2-1: Example of LTE denials in case of LTE in Band40 coexisting with BT slave The analysis indicates that without eNB knowing the denial resources, the UL throughput loss is up to 41.6% [16]. Therefore, autonomous LTE denials for ISM data packets seem not an acceptable solution for solving steady state situations e.g. voice call. ##### 5.2.2.1.3 ISM denials for LTE important reception UE can autonomously deny ISM transmissions to ensure successful reception of important LTE signalling, e.g. system information, paging, synchronization signal, critical dedicated signalling, etc. The details are up to UE implementation and will not be specified in 3GPP. ## 5.3 Applicability of interference avoidance solutions The applicability of TDM solutions for each usage scenario is summarized in Table 5.3-1. Table 5.3-1: Applicability of different TDM solutions TDM solution; Usage scenario. LTE+BT earphone (VoIP service); LTE+BT earphone (Multimedia service); LTE+WiFi portable router; LTE+WiFi offload; LTE+GNSS Receiver. HARQ process reservation based solution; Applicable; Applicable for BT Master, but not applicable for BT Slave; FFS; FFS; Applicable. DRX based solution; Applicable; Applicable; Applicable; Applicable; Applicable. Uplink scheduling restriction based solution; Not applicable; Not applicable; Not applicable; Not applicable; Applicable. Autonomous denial solution; Complementary solution for receiving important signalling.

But this is just one of the aspects around the feature. This feature also allows the UE to indicate to the network, how much time it needs to solve the IDC problems (by indicating an active cycle and cycle length) as follows:

In the IDC TDM solution, the UE notifies the amount of time it needs to sort the IDC and the network might react by avoiding scheduling UL grants in the interval proposed by the UE. In this sense, there is no need to undergo autonomous denial, as much of the UL resources will not be used to schedule this UE. However, as of Rel-18, the maximum active duration of the IDC is 1600 ms within a maximum cycle length of 10240 ms. Assuming that in NTN, the largest IDC cycles are the ones to be used when there is a conflict between the UL band and one of the GNSS bands, this means that the UE requests to avoid UL transmissions at 15% of the time. In a scenario where multiple UEs request IDC, this imposes extra complexity on the NW scheduler.

On the other hand, we understand the concerns raised by other companies that 1600 ms might not be enough time to fully acquire GNSS information. As GNSS is a paramount feature of NTN from Release 17 to Release 19, both, NW and UE should strive for guaranteed timely GNSS information at the UE side. But, if the active duration is expanded, the UEs requesting the largest active duration will be compromising close to 20% of the UL duty cycle. In several cases, this is not in the best interest of the UE that in low mobility scenarios might only need to update the GNSS seldom.

As a compromise between the complexity added to the UL scheduler and the need of NTN UEs to acquire the GNSS information, we propose the following:

Increase the maximum activeDuration-r18 to 2000ms and cycleLength-r18 to 40960 ms, provided that RRM requirements are not affected.

Agree to a maximum IDC free duration of 2000ms.

Agree to the attached LS provided in the Annex.

Conclusion

The following Observations and Proposals were made:

Observation 1: Not all UEs will be harmed by the potential interference between UL and GNSS frequencies.

Observation 2: RAN2 will consider enhancements to the IDC framework in Release 19 only. No changes will be done for Release 18.

Proposal 1: RAN4 shall only consider any changes regarding this issue in Release 19.

Proposal 2: Agree not to increase the value of autonomousDenialSlots-r18

Proposal 3: Increase the maximum activeDuration-r18 to 2000ms and cycleLength-r18 to 40960 ms, provided that RRM requirements are not affected.

Proposal 4: Agree to a maximum IDC free duration of 2000ms.

Proposal 5: Agree to the attached LS provided in the Annex.

- References

- R4-2504708, Way Forward for [114bis][309] NR_NTN_Ph3_General_UE_SAN_RF, Qualcomm, RAN4#114bis, Wuhan, China, 7th – 11th Apr, 2025.

- R4-2503119, Response LS on simultaneous operation between GNSS and NR NTN, Nokia, RAN4#114bis, Wuhan, China, 07th – 11st April, 2025.

- Annex

Title:	draft LS on NR NTN co-existence with GNSS

Response to:

Release:	Rel-19

Work Item:	NR_NTN_Ph3-Core

Source:	RAN WG4

To:	RAN WG2

Cc:	RAN WG1

Contact person:

Name:	Alok Sethi

E-mail Address:	alok.sethi@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# Overall description

RAN4 would like to thank RAN2 for the LS reply in R2-2501568. RAN4 has discussed possible values for both TDM solution and autonomous denial solution.

According to RAN4, in worst case UE may need IDC free duration of up to 2s with a periodicity of approximately 2s to 40s.

Regarding autonomusDenial solution, for some UEs up to 50% of duty cycle is possible provided long mute times are avoided. As an example, a UE can avoid transmitting 10ms out of every 20ms. However, given the extensive performance penalty of long mute times, it is not advisable to increase the value of autonomousDenialSlots-r18.

# Actions

ACTIONS:

To RAN WG2 and WG1: RAN4 respectfully asks RAN2 to consider the above mentioned values regarding the IDC related signalling.

# Dates of next TSG RAN WG4 meetings

TSG-RAN4 Meeting #116	Aug 25 - 29, 2025	Bangalore, India

TSG-RAN4 Meeting #116-bis	Oct 10 - 13, 2025	Prague, CZ