3GPP TSG-RAN WG4 Meeting#116	R4-2510315

# Bengaluru, India, 25 – 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | <CR#> | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-1: power domain enhancement for a UE-specific bandwidth within a wider carrier |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce a power domain enhancement for UL transmissions within a UE-specific channel bandwidth configured within a wider carrier subject to UE capability. The feature must be enabled by the network upon configuration of the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.2.2: the MPR is enhanced (reduced) for UL transmissions within the UE-specific channel bandwidth for UEs supporting this feature when a fixed extension ceil(N’RB/2) of the transmission bandwidth configuration N’RB of the said UE-specific channel bandwidth fits within the carrier as detemined by the UE using existing cell-specific and dedicated signaling (offset to carrier): then outer allocations is converted to inner allocations. Clause 6.5.2.2: the MPR enhancement is enabled by relaxing the emission limit for the first MHz of the spectrum emission mask by a tentative 3 dB for the UE-specific channel bandwidth within the wider carrier under the conditions introduced for the feature in clause 6.2.2. Clause 6.5.2.4.1: the ACLR requirement for the UE-specific CHBW is relaxed by a tentative 3 dB under the conditions introduced in clause 6.2.2. The UE capability and higher-layer parameter notions introduced are tentative. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MPR for transmissions within a UE-specific channel bandwidth configured within a wider carrier bandwidth may be unnecessarily large for some allocations for which UE power reduction is not needed for ensuring in-channel and out-of-band coexistence. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2, 6.5.2.2, 6.5.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes

Table 6.2.2-5 Maximum power reduction (MPR) for power class 1 for Band n14

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 0.5 | ≤ 0.5 | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.5 | 0 | 0 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1. RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

An Edge RB allocation is the one for which the RB(s) is (are) allocated at the lowermost or uppermost edge of the channel LCRB ≤ 2 RBs, except for PC1 UE supporting other bands than n14.

For a UE supporting [mpr-UE-chbw-capability] configured with a UE channel bandwidth by ServingCellConfig and configured with [mpr-UE-chbw-enabled], the above conditions are modified as follows only under the following conditions

-	if OC + ceil(N’RB/2) ≤  OBW, an RB allocation is an inner allocation within the configured UE channel bandwidth for

RBStart  ≤  RBStart,High, and LCRB  ≤  ceil(2/3 *N’RB)

and the condition for an edge RB allocation is not applicable at the lowermost edge

-	if OBW + N’RB + ceil(N’RB/2) ≤  OC + NRB, an RB allocation is an inner allocation within the configured UE channel bandwidth for

RBStart_Low  ≤  RBStart  ≤  NRB – LCRB, and LCRB  ≤  ceil(2/3 *N’RB)

and the condition for an edge RB allocation is not applicable at the uppermost edge

-	if OC + ceil(N’RB/2) ≤  OBW and OBW + N’RB + ceil(N’RB/2) ≤  OC + NRB, an RB allocation is an inner allocation within the configured UE channel bandwidth and the condition for an edge RB allocation is not applicable for

RBStart  ≥ 0 and LCRB  ≤  N’RB

and the condition for an edge RB allocation is not applicable,

where OBW and N’RB are the offset and transmission bandwidth configuration configured by the respective offsetToCarrier and carrierBandwidth in the IE ServingCellConfig and, while OC and NRB are the corresponding configured by the respective offsetToCarrier and carrierBandwidth in the IE DownlinkConfigCommon/DownlinkConfigCommonSIB. RBStart  = 0 coincides with the first common RB index within the transmission bandwidth configuration N’RB of the UE channel bandwidth.

And for PC1 UE supporting other bands than n14 RB allocation is an Edge RB allocation if

< text omitted >

#### 6.5.2.2		Spectrum emission mask

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR channel bandwidth. For frequencies offset greater than ΔfOOB, the spurious requirements in clause 6.5.3 are applicable.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

The power of any UE emission shall not exceed the levels specified in Table 6.5.2.2-1 for the specified channel bandwidth.

Table 6.5.2.2-1: General NR spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 10, 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 | -13 | -13 |  | 1 % of channel BW |
| ± 0-1 |  |  |  | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 |  | 1 MHz |
| ± 5-6 | -25 | -13 |  |  |  |
| ± 6-10 |  | -25 |  |  |  |
| ± 5-BWChannel |  |  | -13 |  |  |
| ± BWChannel-(BWChannel+5) |  |  | -25 |  |  |

The following exceptions apply for a UE supporting [mpr-UE-chbw-capability] configured with a UE channel bandwidth BWChannel by ServingCellConfig and configured with [mpr-UE-chbw-enabled],

-	if OC + ceil(N’RB/2) ≤  OBW, the spectrum emission mask in Table 6.5.2.2-1 with the emission limit in |ΔfOOB | ≤ 1 MHz modified to [-10] dBm applies immediately below the channel bandwidth

-	if OBW + N’RB + ceil(N’RB/2) ≤  OC + NRB, by the spectrum emission mask in Table 6.5.2.2-1 with the emission limit in |ΔfOOB | ≤ 1 MHz modified to [-10] dBm applies immediately above the channel bandwidth

-	if OC + ceil(N’RB/2) ≤  OBW and OBW + N’RB + ceil(N’RB/2) ≤  OC + NRB, the spectrum emission mask in Table 6.5.2.2.1-1 with the emission limit in |ΔfOOB | ≤ 1 MHz modified to [-10] dBm applies immediately above and below the channel bandwidth

where OBW and N’RB are the offset and transmission bandwidth configuration configured by the respective offsetToCarrier and carrierBandwidth in the IE ServingCellConfig and, while OC and NRB are the corresponding configured by the respective offsetToCarrier and carrierBandwidth in the IE DownlinkConfigCommon/DownlinkConfigCommonSIB.

#### 6.5.2.3		Additional spectrum emission mask

< text omitted >

##### 6.5.2.4.1	NR ACLR

NR Adjacent Channel Leakage power Ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5.2.4.1-1.

If the measured adjacent channel power is greater than –50 dBm then the NRACLR shall be higher than the value specified in Table 6.5.2.4.1-2.

When the IE powerBoostPi2BPSK-r18 or powerBoostQPSK-r18 is set to 1 for a UE supporting the capability of powerBoosting-pi2BPSK-QPSK-r18 or capability of powerBoosting-pi2BPSK-QPSK-Modified-r18, for power class 2 UE, the ACLR requirement of PC2 applies. For power class 3 UE, the ACLR requirement of PC3 applies.

Table 6.5.2.4.1-1: NR ACLR measurement bandwidth

| Channel bandwidth | (MHz) | 3,5,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| NR ACLR measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |
| NOTE : “NRB” in the formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |

Table 6.5.2.4.1-2: NR ACLR requirement

|  | Power class 1 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR | 37 dB | 31 dB | 31 dB | 30 dB |
| NOTE 1: Void |  |  |  |  |

The following exceptions apply for a UE supporting [mpr-UE-chbw-capability] configured with a UE channel bandwidth BWChannel by ServingCellConfig and configured with [mpr-UE-chbw-enabled],

-	if OC + ceil(N’RB/2) ≤  OBW, the NR ACLR requirement in Table 6.5.2.4.1-2 is reduced by [3] dB below the channel bandwidth

-	if OBW + N’RB + ceil(N’RB/2) ≤  OC + NRB, the NR ACLR requirement in Table 6.5.2.4.1-2 is reduced by [3] dB  above the channel bandwidth

-	if OC + ceil(N’RB/2) ≤  OBW and OBW + N’RB + ceil(N’RB/2) ≤  OC + NRB, the NR ACLR requirement in Table 6.5.2.4.1-2 is reduced by [3] dB above and below the channel bandwidth

where OBW and N’RB are the offset and transmission bandwidth configuration configured by the respective offsetToCarrier and carrierBandwidth in the IE ServingCellConfig and, while OC and NRB are the corresponding configured by the respective offsetToCarrier and carrierBandwidth in the IE DownlinkConfigCommon/DownlinkConfigCommonSIB. The measurement bandwidth is according to the BWChannel configured.

##### 6.5.2.4.2	UTRA ACLR

< end of changes >