3GPP TSG-RAN WG4 Meeting#116	R4-2510317

# Bengaluru, India, 25 – 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | <CR#> | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101.1: time masks for DL-only CA via switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LBCA_Sw-Core |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce transmission masks for low-band DL CA via switching with the UL configured in one UL band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The transmission time masks specify the locations of transient periods for UL slots paired with DL slots that are adjacent to switching gaps, or containing switching gaps, for the cases UL signals are not dropped and cases for which the network schedules transmissions as expected by the UE. These time masks also specify the location of a switching period (subject to UE capability) and its timing relation to trailing/leading transient periods of the timing advanced UL within the switching gap configured by the NW. Clause 6.3A.3.3.1 (general requirements for inter-band CA): reference to a new clause Clause 6.3A.3.3.7 (new): time masks for the DL introduced for transmissions - configured by higher layers assuming NTA = 0 µs that defines collision cases for these transmissions (these masks also applicable for scheduled transmissions when NTA = 0 µs) - scheduled/triggered and overlapping with the switching gap for NTA > 0 µs. The switching period must occur before the first symbol of the timing advanced UL such that this symbol is not corrupted (transmissions not dropped) For UL slots paired with DL slots not adjacent to or not containing a configured switching gap the requirements in sub-clause 6.3.3 apply. The time masks apply for a receive timing difference between cells of up to MRTD for colocation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The location of a switching period (subject to UE capability) and its timing relation to trailing/leading transient periods of the timing advanced UL within the configured switching gap is not specified for scheduling cases expected by the UE and therefore unknown to the NW. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.3.3.1, 6.3A.6.6.7 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | . TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Changes relative to the draft CR in R4-2506197: time masks now only specified for UL transmissions. |  |  |  |  |  |  |  |  |

< start of changes >

#### 6.3A.3.3	Transmit ON/OFF time mask for inter-band CA

##### 6.3A.3.3.1	General

For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the transmit ON/OFF time mask requirements in subclause 6.3.3 apply except for downlink carrier aggregation via switching for which the ON/OFF time mask requirements in subclause 6.3A.3.3.7 apply.

< text omitted >

#### 6.3A.3.3.7	Transmit ON/OFF time mask for downlink inter-band CA via switching

For a downlink inter-band CA configuration provided by LBCA-SwitchingPattern, the ON/OFF time masks for NR UL transmissions in FR1 at the start of an UL slot succeeding an SDL to FDD switch and at the end of an UL slot preceding an FDD to SDL switch are specified in Figure 6.3A.3.3.7-1 for a timing advance NTA = 0 µs. These time masks apply for PUSCH, or PUCCH, or SRS, or PRACH transmissions, or applicable combinations thereof, when the set of symbols of these transmissions are not in any switching gap configured by either LBCA-SwitchingGap-Duration-PCelltoSCell for the FDD to SDL switch or LBCA-SwitchingGap-Duration-SCelltoCell for the FDD to SDL switch.

The switching period of a duration X µs indicated by [UE capability] is located between the start/end of the switching gap and the end/start of the transient period.

For scheduled or triggered PUSCH, or PUCCH, or SRS, or PRACH transmission, the ON/OFF time mask at the start of an UL slot succeeding an SDL to FDD switch is specified in Figure 6.3A.3.3.7-1 assuming a timing advance NTA ≥ 0 µs, the set of symbols of these transmissions can be in the preceding switching gap that covers both the timing advance and the switching period.

[The time masks apply for a receive time difference between the Pcell and Scell up to MRTD = 3 µs.]

Figure 6.3A.3.3.7-1: ON/OFF time masks for NR UL transmissions in FR1 at (a) the start of an UL slot succeeding an SDL to FDD switch and (b) at the end of an UL slot preceding an FDD to SDL switch

Figure 6.3A.3.3.7-2: ON/OFF time mask for scheduled or triggered NR UL transmissions in FR1 at the start of an UL slot succeeding an SDL to FDD switch

For UL slots paired with DL slots not adjacent to or not containing a configured switching gap the requirements in sub-clause 6.3.3 apply.

< end of changes >