3GPP TSG-RAN WG4 Meeting #116	                                                                R4-2510319

Bengaluru, India, 25th   – 29th August, 2025

Agenda item:      4.2.2

Source:               MediaTek

Title: 	               Discussion on ModifiedMPR-Behavior updates

Document for:	 Approval

# Introduction

In the last RAN4 Malta Meeting, RAN4 agreed the WF [1] on ModifiedMPR-Behavior to address the multiple approach to apply the latest MPR/A-MPR requirements according to each operating bands, such as what is different point between  bit ‘0’ and absent from BS and UE perspective, whether to define different bit between different device type and different supporting specification version. In this contribution, we share our suggesting for clear application methodology of ModifiedMPR-Behavior and how to handle the specification.

Agreed WF [1] in RAN4 #115 Meeting

In next meeting, at least the following issues will be further discussed.

- Check the difference between bit ‘0’ and absent from BS and UE perspective.

- Check if the spec version number can be removed from next release.

- Check if the requirements are changed several times, how to handle the legacy bits.

  - See n41

| n41 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)41AA UE EN-DC |
| --- | --- | --- | --- |
|  | 1 | EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_41A_n41A EN-DC |
|  | 2 | EN-DC contiguous and non-contiguous intraband MPR and A-MPR as defined in 38.101-3 v16.4.0. If this bit is not set the UE uses Rel-15 MPR or A-MPR for EN-DC contiguous and non-contiguous intraband MPR and A-MPR | -This bit may be set to 1 by a UE supporting DC_(n)41AA or DC_41A_n41A EN-DC |

- Check if any good approach on how to handle the bits for different form factors, if dedicated bits should be used to differentiate the form factors, FWA and smart phone, for future new cases.

|  | 3 | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |
| --- | --- | --- | --- |

# Discussion

## General description for ModifiedMPR behaviour

For the specification handling aspect for ModifiedMPR-behaviour, RAN4 need to clear understanding how can apply the MPR/A-MPR requirements according to the specification updates considering with different MPR/A-MPR parameters assumptions, different regulation limits and different formfactors.

Form the agreed WF at last RAN4 meeting, we can define some general note to get common understanding on the ModifiedMPR-behaviour and try to minimize the legacy RAN4 RF specification in previous release. Hence, we are fine to update only the latest specification in TS38.101-1 to address the ModifiedMPR-behaviour.

Proposal 1: For the existing modifiedMPR-Behaviour bits, RAN4 only focus to update the definition and corresponding Notes in Table L.1-1 in TS38.101-1 from Rel-19.

Also, there may need to clarify the ModifiedMPR-behaviour bit is set to ‘0’ or absent in BS and UE perspective. When the UE support ModifiedMPR-behaviour with optional with capability signaling, then the bit shall be set ‘0’ or ‘1’.  If UE’s capability signaling is not reported to gNB, then the ModifiedMPR-behaviour bit ‘0’ should be set to 1.

Proposal 2: If UE’s capability signaling is not reported to gNB, then the ModifiedMPR-behaviour bit ‘0’ should be considered as set to 1.

To get a common understanding on the ModifiedMPR-behaviour, RAN4 can add Note 2 in the general part.

Proposal 3: RAN4 can add Note 2 in L.1 clause to clear understanding on the ModifiedMPR-behaviour.

### L.1	Indication of modified MPR behavior

This annex contains the definitions of the bits in the field modifiedMPR-Behaviour indicated per supported NR band in the IE RF-Parameters [7] by a UE supporting an MPR or A-MPR modified in a given version of this specification. A modified MPR or A-MPR behaviour can apply to a supported NR band in stand-alone operation (including CA and NN-DC operation) or in non-standalone operation with the said NR band as part of an EN-DC or NE-DC band combination.

NOTE 1:	In the present release, the modifiedMPR-Behaviour is indicated [7] by an 8-bit bitmap per supported NR band.

NOTE 2: Specification version references in Table L.1-1 Definition column refer to the version in which the functionality was modified. UE shall follow the latest version of specification of the release UE complies with.

## n41 Modified MPR behaviour

In TS38.101-1 v.19.2.0, there are 4 different bits are allocated to apply ModifiedMPR-behaviour with different MPR requirements as follow

Table 1: n41 modifiedMPR-Behaviour

| n41 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a HH UE supporting DC_(n)41AA UE EN-DC |
| --- | --- | --- | --- |
|  | 1 | EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a HH UE supporting DC_41A_n41A EN-DC |
|  | 2 | EN-DC contiguous and non-contiguous intraband MPR and A-MPR as defined in 38.101-3 v16.4.0. If this bit is not set the UE uses Rel-15 MPR or A-MPR for EN-DC contiguous and non-contiguous intraband MPR and A-MPR | -This bit shall be set to 1 by a HH UE supporting DC_(n)41AA or DC_41A_n41A EN-DC |
|  | 3 | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit shall be set to 1 by a FWA UE supporting power class 1.5.. |

For the n41 DC UE with bit 2, RAN4 defined the updated MPR requirements for both intra contiguous DC_ (n)41AA and intra-band non-contiguous CA DC_41A_n41A in Rel-16.

So, if the n41 EN-DC UE shall be set to 1 by a UE supporting DC_(n)41AA or DC_41A_n41A UE when the UE support from rel-16.

For the n41 DC UE with bit 3, the bit shall be also set to 1 for PC1.5 FWA UE and can remove the sentence that started with if as above in Rel-19 specification. This shall be supported as mandatory from Rel-17 supporting UE.

Proposal 4: For n41 intra-band contiguous DC and non-contiguous DC UE, the modifiedMPR-behaviour would be updated in Table 1 in Rel-19.

Proposal 5: From Rel-19, RAN4 can differentiate the modifiedMPR-behaviour bit for the different device type, i.e., FWA/HHUE.

## n96 Modified MPR behaviour

In TS38.101-1 v.19.2.0, there are 2 different bits are allocated to apply Modified MPR behaviour with different MPR requirements to reflect the updated as follow

Table 2: n96 modifiedMPR-Behaviour

| n96 | 0 (leftmost bit) | Support of all band n96 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 | This bit shall be set to 1 by a HH UE to support maximum CBW is up to 80MHz |
| --- | --- | --- | --- |
|  | 1 | Support of all band n96 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 | This bit shall be set to 1 for HH UE to support maximum CBW is up to 100MHz |

In Rel-17, the maximum CBW in n96 is up to 80MHz, the A-MPR requirements for n96 were specified to comply regulatory requirements in different countries with NS_53, NS_54, NS_59 and NS_60.

In Rel-18, the maximum CBW in n96 is extended up to 100MHz, also the A-MPR requirements for n96 were specified to comply regulatory requirements in different countries with NS_53, NS_54, NS_59, NS_60, NS_61, NS_66, NS_67 and NS_71.

So, the ModifiedMPR-behaviour with bit 0 for the UE’s maximum supporting CBW is up to 80MHz shall be set to 1. And also the ModifiedMPR-behaviour with bit 1 for the UE’s maximum supporting CBW is up to 100MHz shall be set to 1 in Rel-19.

Proposal 6: To apply the different A-MPR requirements in n96 between bit 0 and bit 1, the modifiedMPR-behaviour would be updated in Table 2 in Rel-19.

## n102 Modified MPR behaviour

In TS38.101-1 v.19.2.0, there are 2 different bits are allocated to apply Modified MPR behaviour with different MPR requirements to reflect the updated as follow

Table 3: n102 modifiedMPR-Behaviour

| n102 | 0 (leftmost bit) | Support of all band n102 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 | This bit shall be set to 1 by a HH UE to support power class 5 with NS_58 |
| --- | --- | --- | --- |
|  | 1 | Support of all band n102 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 | This bit shall be set to 1 by a HH UE to support power class 3 with NS_58 and power class 5 with other NS_xx |

In Rel-17, RAN4 only focus on the Power class 5 in unlicensed band in n102 operating band with NS_58 for Europe regions.

In Rel-18, RAN4 can expand power class 3 device in n102 in EU region and specify the A-MPR requirements to comply the different regulation in EU countries in n102 for power class 5 device with NS_63, NS_64, NS_65, NS_68 and NS_69.

So, the ModifiedMPR-behaviour with bit 0 for power class 5 in Europe region with NS_58 shall be set to 1. And also the ModifiedMPR-behaviour with bit 1 for the power class 3 in Europe region with NS58 and other countries n102 operation shall be set to 1 in Rel-19.

Proposal 7: To apply the different A-MPR requirements in n102 between bit 0 and bit 1, the modifiedMPR-behaviour would be updated in Table 3 in Rel-19.

## n104 Modified MPR behaviour

In TS38.101-1 v.19.2.0, the same ModifiedMPR-behaviour is considered to apply Modified MPR behaviour with different MPR requirements to reflect the updated as follow

Table 4: n104 modifiedMPR-Behaviour

| n104 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2.2-4 in TS38.101-1 v16.5.0 | This bit shall be set to 1 by a FWA UE to supporting power class 1.5. |
| --- | --- | --- | --- |
|  | 1 | PC1.5 MPR as define in Table 6.2D.2-3 | This bit shall be set to 1 by FWA UE to supporting power class 1.5. |
|  | 2 | PC1.5 MPR as define in Table 6.2D.2-2 | This bit shall be set to 1 by HH UE to supporting power class 1.5. |

In Rel-16, RAN4 just consider legacy MPR simulation assumption for 2 transmitter with small antenna isolation between Tx-Rx to derive MPR requirements for PC1.5 single carrier UE.

In Rel-17, RAN4 updated some MPR simulation parameters and consider large antenna isolation to derive MPR requirements for PC1.5 single carrier for both HH UE and FWA UE.

Hence, all of ModifiedMPR-behaviour bits will shall be set to 1 in Rel-19. Furthermore, RAN4 would be define different ModifiedMPR-behaviour bit according to different MPR requirements to consistency with other NR band as above modifiedMPR-behaviour.

Proposal 8: To apply the different MPR requirements in n104 according to different device type, RAN4 would be define bit 0, 1 and 2 for n104 for modifiedMPR-behaviour in Table 4 in Rel-19.

# Conclusion

In this contribution, we provides our understanding of ModifiedMPR-Behavior and how to update in Rel-19 acocrding to different MPR/A-MPR simulation parameters depend on power class and different device type and others. Based on the analysis and understanding, we provide some general approaches and the proposals to address this topic in Rel-19 specification apsect.

Proposal 1: For the existing modifiedMPR-Behaviour bits, RAN4 only focus to update the definition and corresponding Notes in Table L.1-1 in TS38.101-1 from Rel-19.

Proposal 2: If UE’s capability signaling is not reported to gNB, then the ModifiedMPR-behaviour bit ‘0’ should be considered as set to 1.

Proposal 3: RAN4 can add Note 2 in L.1 clause to clear understanding on the ModifiedMPR-behaviour.

NOTE 2: Specification version references in Table L.1-1 Definition column refer to the version in which the functionality was modified. UE shall follow the latest version of specification of the release UE complies with.

Proposal 4: For n41 intra-band contiguous DC and non-contiguous DC UE, the modifiedMPR-behaviour would be updated in Table 1 in Rel-19.

Proposal 5: From Rel-19, RAN4 can differentiate the modifiedMPR-behaviour bit for the different device type, i.e., FWA/HHUE.

Proposal 6: To apply the different A-MPR requirements in n96 between bit 0 and bit 1, the modifiedMPR-behaviour would be updated in Table 2 in Rel-19.

Proposal 7: To apply the different A-MPR requirements in n102 between bit 0 and bit 1, the modifiedMPR-behaviour would be updated in Table 3 in Rel-19.

Proposal 8: To apply the different MPR requirements in n104 according to different device type, RAN4 would be define bit 0, 1 and 2 for n104 for modifiedMPR-behaviour in Table 4 in Rel-19.

# References

[1] R4-2507957, “WF on modifiedMPR-Behavior”, Xiaomi, RAN4#115

[2] R4-2505680,	“CR 38.101-1 R17 Correction for ModifiedMPR table [Modified_MPR]”, Nokia, RAN4#115

[3] R4-2505681,	“CR 38.101-1 R18 Correction for ModifiedMPR table [Modified_MPR]”, Nokia, RAN4#115

[4] R4-2505682,	“CR 38.101-1 R19 Correction for ModifiedMPR table [Modified_MPR]”, Nokia, RAN4#115

[5] R4-2505757,  “Discussion on ModifiedMPR-Behavior and spec”,	Xiaomi, RAN4#115

[6] R4-2507723,	“(PHS_System_NRonly) On the requirements of modifiedMPR-Behavior”, Huawei, RAN4 #115