3GPP TSG RAN WG4 Meeting #116	R4-2510323

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.3.4

Source:	vivo

Title:	Further discussion on RRM requirements for LB CA via switching

Document for:	Discussion

# Introduction

In the RAN4#115 meeting, RAN4 discussed RRM requirements for low band CA via switching. Following agreements were made and captured in the WF [1].

Issue 1-1-1: SSB assumptions for R19 LB CA via switching - The baseline assumption is: - SDL SCell is transmitting SSB - RAN4 to use SSB based requirement as baseline for Rel-19 low band CA via switching - requirement of PCell will still use the existing principle as baseline but the impact of carrier switching in LB CA will be considered Issue 1-1-2: MRTD assumptions for R19 LB CA via switching - The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us. - Whether or not above RTD can be used as side condition for SCell activation will be discussed in another issue. Issue 1-2-1: Necessity of interruption requirement in RRM between FDD CC and SDL CC - For SDL to FDD switching and FDD to SDL switching, RAN4 to not define interruption or scheduling restriction requirement on SDL carrier and FDD carrier. - Note: the above agreement can be revisited if not aligned with RAN1 further agreement. Issue 1-3-1: Necessity of activated SDL SCell L3 measurement and corresponding interruption requirement - RAN4 to define the requirement for activated SDL SCell L3 measurement. Issue 1-3-2: Requirement design of activated SDL SCell L3 measurement - For activated SDL SCell measurement, UE performs the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern. FFS: - If no SSB occasion of the serving cell is contained within the LB-CA switching pattern, the requirements do not apply (from Nokia proposal) - FFS on details of requirement design. Issue 1-3-3: Necessity of deactivated SDL SCell measurement and corresponding interruption requirement - RAN4 to define the requirement for deactivated SDL SCell L3 measurement. Issue 1-3-4: Requirement design of deactivated SDL SCell measurement and corresponding interruption - The legacy deactivated SCell measurement delay requirements are applied for UE to perform deactivated SDL SCell measurement. - For deactivated SDL SCell measurement, interruptions on active serving cell(s) are allowed with up to X% probability of missed ACK/NACK. - FFS: X, and X can be same or different for different SDL SCC measurement cycle (i.e., configured measCycleSCell). Issue 1-3-5: Scope of SDL SCell activation/deactivation - PUCCH SCell activation, Rel-19 fast SCell activation through EMR, and activation of multiple SCells is not in the scope of the WI. Issue 1-3-6: side condition for SDL SCell activation/deactivation requirement - Follow the legacy case and side conditions for SSB based inter-band FR1 SCell activation in existing spec. Issue 1-3-7: SDL SCell activation/deactivation delay requirement - The R18 SSB based FR1 inter-band SCell activation delay requirement can be reused to SDL SCell activation in LB CA via switching if UE supports corresponding feature. - R16 Direct SCell activation with HO is not considered for R19 LB CA. - R17 PUCCH SCell activation is not considered for R19 LB CA. - R18 SSB-less SCell activation for NES is not considered for R19 LB CA. - R17 Fast SCell activation is not considered for R19 LB CA. - Reuse the existing SCell deactivation delay requirement for SDL SCell deactivation. - Known/unknown conditions are reused from existing SCell activation requirements - FFS: - RAN4 to discuss the UE behavior if the CQI or L1-RSRP reporting is colliding with SSB or SMTC of being-activated SDL SCell during the SDL SCell activation. (Apple) - The starting point and ending point of CSI measurement for the sake of CSI reporting should be determined, for both SSB based and SSB-less SCell activation.(ZTE) - Amend the previous agreement with the following - The UE shall continuously monitor PCell disregarding the configured switching pattern. (Nokia) - If RAN4 to define requirement for BFD/CBD for SDL SCell in LB CA via switching, the BFD/CBD on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern. Issue 1-3-11: TCI state switch delay requirement for SDL SCell - The L1-RSRP measurement and T/F tracking for TCI switching on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern. Issue 1-3-12: PL-RS switch delay requirement for SDL SCell - Not define requirement for PL-RS switch delay for SDL SCell in LB CA via switching. Issue 1-3-13: L1-RSRP/SINR measurement requirements for SDL SCell - For L1-RSRP/SINR measurement requirement for SDL SCell, UE can only perform such measurement on the overlapped occasions of L1 RS and SDL SCell reception duration indicated by switching pattern. Issue 1-4-1: Impact on timing requirement for PCell - the existing Tx timing requirement for PCell with the same applicability condition (SSB is available in last 160ms) can apply for R19 LB CA via switching.

In the previous RAN4 meetings, we provided our views in [2, 3, 4]. In this contribution, we further provide our views on RRM requirements impact due to low band carrier aggregation with switching.

# Discussion

## 2.1 General

Switching pattern applicability

In the previous RAN4 meeting, there were agreements on applicability of switching pattern, which were captured in the WF [5].

Issue 1-1-3: switching pattern applicability for RRM requirement in R19 LB CA via switching Agreement: - For the SCell which has been activated(i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is not applied. - RAN4 defines SCell activation requirement assuming UE utilizes the RS occasions to perform SDL SCell activation - the above RS occasions are FFS - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied. - If the deactivated SDL SCell measurement requirement is needed, FFS whether UE follows legacy deactivated SCell measurement requirements to perform deactivated SDL SCell measurement or follow a new measurement requirement for deactivated SDL SCell measurement. Note: the above FFS parts will be discussed and decided in RAN4.

In the RAN1#121 meeting, agreements on switching pattern design in RAN1 was LS-ed to RAN2 and RAN4 in the LS [6].

Agreement - For Rel-19 low NR band carrier aggregation via switching, support only semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between the following two cases - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - FDD carrier 1 is PCell and SDL carrier 2 is SCell - SCS 15KHz on both carriers Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, it is not expected that the same slot (1ms) contains both symbol(s) configured as FDD carrier and symbol(s) configured as SDL carrier. Agreement For Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern and corresponding switching gap are based on the downlink timing (DL reception time at UE side) of Pcell. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS). Agreement - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2 - The pattern starts from the beginning of SFN 0 of PCell - Restriction of maximum X switch(es) within Y slot(s), - FFS: detailed value and relation of X and Y. - FFS: other restriction Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, switching gap is located at the “switch from” carrier, i.e., FDD carrier. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms Agreement For Rel-19 low NR band carrier aggregation via switching, support configuration of only one switching pattern to a UE based on UE-specific RRC configuration.

Based on RAN1 design, the semi-static switching pattern is periodic with periodicity of 40ms. During the 40ms, there may be more than 1 switches according to restriction of maximum X switch(es) within Y slot(s). However, it seems there is no further agreement on this. From RAN4 RRM requirements perspective, we may assume there is one switch during one periodicity of 40ms if necessary.

It was agreed that switching pattern is applied to activated SCell and is not applied to deactivated SCell and activation procedure. In addition to this, we think operations with regard to dormant SCell should also be considered. Dormant SCell is an important and useful feature. Similar to deactivated SCell, if UE still switches to dormant SCell following switching pattern, there will be unnecessary waste of resource for the UE. Thus, switching pattern is not applied to dormant SCell, i.e., when the SCell has switched to a dormant BWP.

Proposal 1: Switching pattern is not applied for dormant SCell.

How exactly is the switching pattern applied or not applied needs to be decided. The switching pattern is configured by UE specific RRC. It is not expected that applicability of gap pattern is via RRC reconfiguration. So, the switching pattern is configured via RRC and application of the switching pattern is based on reporting or events.

When SCell is deactivated or not configured, the UE is continuing staying on PCell without switching though switching pattern is configured. If SCell activation is triggered via MAC-CE or RRC, UE should report valid CSI to indicate complete of SCell activation. After valid CSI report, UE should apply the switching pattern and performs switching accordingly.

As depicted in Fig 1, the UE should be on PCell according to switching pattern. In this case, UE can perform switching according to the configured switching pattern after valid CSI reporting.

Fig 1. SCell activation: Switching pattern and switching case 1

As depicted in Fig 2-1 and Fig 2-2, the UE should be on SCell according to switching pattern. In this case, there could be different switching mechanism.

In Fig 2-1, UE switches to SCell immediately after reporting valid CSI. The pros are that UE can access the SCell spectrum as early as possible. The cons are that UE may stay on the SCell for a very short time depending on the timing of CSI report. In Fig 2-2, UE switches to SCell at the next switching period according switching pattern after reporting valid CSI. It has less overhead due to switching. RAN4 to decide which mechanism is to be used.

Fig 2-1. SCell activation: Switching pattern and switching case 2

Fig 2-2. SCell activation: Switching pattern and switching case 2

In the last RAN4 meeting, there were proposals on the application of switching pattern.

Issue 1-3-9: Activation of Switching Pattern - Proposal 1: - At the end of SCell activation (i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), spec allows UE a minimum time separation of X ms between transmitting CQI report and applying the semi-statically configured switching pattern. - Value of X is 3ms - Proposal 2: - RAN4 needs to clarify when to apply the semi-static switching pattern after compeleting SCell activation, e.g., apply immediately the semi-static switching pattern or apply next cycle semi-static switching pattern.

In general, we don’t see strong motivation to have preparation time for switching pattern application. There at least could be time between CSI RS and CSI reporting which may be used for preparation in advance if it is needed. Furthermore, based on UE implementations, e.g., preparation is done in advance since SCell activation is triggered by MAC-CE or RRC, such time may not be needed at all.

Proposal 2: The switching pattern is applied after UE reports valid CSI for SCell activation.

Proposal 3: If UE should be on PCell according to switching pattern after valid CSI report for SCell activation, UE performs switching according to the configured switching pattern.

Proposal 4: If UE should be on SCell according to switching pattern after valid CSI report for SCell activation, RAN4 to decide how UE performs switching according to the following two options.

Option 1: UE switching to SCell immediately after valid CSI report.

Option 2: UE switching to SCell at the next switching period after valid CSI report.

For SCell deactivation, simple mechanism can be used as depicted in Fig 3. After completion of SCell deactivation, the switching pattern is not applied anymore. The UE should switch to PCell. Depending on UE being on PCell on SCell currently, UE performs switching accordingly. If UE is on PCell currently, UE keeps staying on PCell. If UE is on SCell currently, UE needs to switch to PCell immediately.

Fig 3. SCell deactivation: Switching pattern and switching cases

Proposal 5: The switching pattern is NOT applied after UE completed the SCell deactivation.

Proposal 6: If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to SCell immediately.

If RAN4 agrees that switching pattern is not applicable to dormant SCell either, similar principle as SCell activation/deactivation can be used to determine when the switching pattern is applicable/not applicable and how UE should performs switching.

Proposal 7: Similar principle is used for dormant SCell if switching pattern is not applicable to dormant SCell.

## 2.2 SDL SCell requirements

There are still some open issues needed to be solved for SDL SCell operation.

SDL SCell activation/deactivation

Issue 1-3-7: SDL SCell activation/deactivation delay requirement - FFS: - RAN4 to discuss the UE behavior if the CQI or L1-RSRP reporting is colliding with SSB or SMTC of being-activated SDL SCell during the SDL SCell activation. (Apple) - The starting point and ending point of CSI measurement for the sake of CSI reporting should be determined, for both SSB based and SSB-less SCell activation.(ZTE) - Amend the previous agreement with the following - The UE shall continuously monitor PCell disregarding the configured switching pattern. (Nokia)

Regarding the collision between SSB reception and CQI reporting during SCell activation procedure, it can be separated into two cases. If UE has not finished BB and RF preparation, UE needs to report invalid CQI. If collision happens during this stage, UE can drop CQI reporting. If UE is to report valid CQI to indicate SCell activation completion, CQI reporting should be prioritized. Furthermore, UE may not report invalid CQI at all during the SCell activation procedure.

Proposal 8: UE is to drop invalid CQI reporting and prioritize valid CQI reporting when SSB reception is collided with CQI reporting, or UE doesn’t report invalid CQI during SCell activation.

Issue 1-3-8: interruption or scheduling restriction during SDL SCell activation/deactivation - Proposal 1 (OPPO, CATT(scheduling restriction), Nokia): - RAN4 to define the scheduling restriction or interruption caused by the measurements during SDL SCell activation procedure. - Proposal 2 (Apple): - the interruption pattern due to CC switching in SDL SCell activation shall follow SSB periodicity or SMTC periodicity used for SCell activation. - the interruption length for each CC switching in SDL SCell activation can be defined as {(switching duration for FDD-to-SDL) + SSB/SMTC duration + (switching time for FDD-to-SDL)} or reuse same interruption requirement from legacy FR1 intra-band SCell activation. - Proposal 3 (LGE): - RAN4 to specify SCell activation window which ensures SCell measurements of the deactivated SCell. - Proposal 4 (HW): - During SDL SCell activation, if the to-be-used RS resource is SSB: - Option 1: UE stays on SDL carrier until the SCell activation procedure is completed. The whole SCell activation delay would be specified as interruption or scheduling restriction. It shall be noted that UE shall also wait for one CSI-RS resource for CQI. And a valid CQI will be transmitted on PCC. - Option 2: UE switches to SDL carrier according to SSB occasions. The interruption or scheduling restriction would happen on SSB symbols and 1 symbol before each consecutive SSB symbols to be measured and 1 symbol after each consecutive SSB symbols to be measured. It shall be noted that UE shall also measure one CSI-RS resource for CQI. And a valid CQI will be transmitted on PCC. - FFS if the to-be-used RS is A-TRS or P-TRS. - Proposal 5 (ZTE): - RAN4 needs to consider the switching period/interruption during the SDL SCell activation procedure for the switching between PCell and SDL SCell L3/L1 measurement - Recommended WF - Check if following moderator proposal for compromise(considering P1, P2, P3, P4(option 2), P5) is agreeable: - Proposal 6(Moderator): - RAN4 to define the scheduling restriction caused by the measurements during SDL SCell activation procedure. - The scheduling restriction due to CC switching in SDL SCell activation shall follow RS periodicity or SMTC periodicity used for SCell activation. - the scheduling restriction length for each CC switching in SDL SCell activation can be defined as {(symbol(s) for switching duration of FDD-to-SDL) + SSB/SMTC duration + (symbol(s) for switching time of FDD-to-SDL)}.

During SCell activation, UE needs to receive SSB for AGC and T/F tracking and receive CSI-RS for CSI reporting. Both will cause interruptions to the FDD PCell. It is better to define a scheduling restriction window. Proposal 6 is generally fine for reception of SSB on SCell and SSB duration should be used. For L1-RSRP measurement and reporting, CSI-RS is measured by UE. This case should also be considered.

Proposal 9: Proposal 6 for issue 1-3-8 is used as baseline to define scheduling restriction requirements for SCell activation. In addition, scheduling restriction due to CSI-RS reception should also be defined.

L3 measurement on activated/deactivated SDL SCell

Issue 1-3-2: Requirement design of activated SDL SCell L3 measurement - For activated SDL SCell measurement, UE performs the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern. FFS: - If no SSB occasion of the serving cell is contained within the LB-CA switching pattern, the requirements do not apply (from Nokia proposal) - FFS on details of requirement design.

The periodicity of switching pattern is 40ms. With this condition, it would be easy for network to configure that SMTC for SCell L3 measurement can be overlapped with SCell reception duration in one or multiple switching pattern periodicities since the SMTC periodicity is {5, 10, 20, 40, 80, 160}ms. The available measurement interval can be specified as max (SMTC, 40). If there are no overlap occasions, no requirements are defined.

Proposal 10: For activated SCell L3 measurement, the minimum measurement interval is max (SMTC, 40).

Proposal 11: No requirements are defined if there are no overlapped occasions between SMTC and reception duration of SCell.

L1 measurement for activated SDL SCell

For the L1 measurements in FR1 sub 1GHz, it should not be necessary to BFD/CBD requirements.

Proposal 12: L1 measurement requirements for BFD/CBD are not defined for SDL SCell via switching.

It was agreed that for L1 measurement requirement for SDL SCell, UE can only perform such measurement on the overlapped occasions of L1 RS and SDL SCell reception duration indicated by switching pattern. Similar to activated SCell L3 measurements, the minimum measurement interval should consider switching pattern periodicity.

Proposal 13: For L1 measurement requirements for SDL SCell operation, the evaluation period or measurement delay is based on max of switching pattern periodicity (40 ms) and L1-RS periodicity and DRX cycle if configured.

## 2.3 FDD PCell requirements

For FDD PCell carrier related operation, all the L3 measurement requirements related to availability of SSB on the carrier need to be discussed. For example, intra-frequency measurement delay may need to be extended due to some measurement occasions are not available because the UE dwells on SDL SCell during the supposed measurement occasion. There are also many L1 measurements (e.g., RLM/BFD/L1-RSRP/L1-SINR)/L1 procedures ((e.g., TCI state switch, PL-RS switch delay etc.) that would be impact due to the switching pattern.

Similar discussions on SDL SCell operation, the requirements for FDD PCell also need to be updated to address the unavailability of SSB for L3 measurements and unavailability of L1-RSs for L1 measurements due to switching between FDD PCell and SDL SCell.

It should be noted that updated requirements are only applicable when switching pattern is applied.

Proposal 14: For L1 measurement requirements for FDD PCell operation, the evaluation period or measurement delay is based on max of switching pattern periodicity (40 ms) and L1-RS periodicity and DRX cycle if configured.

Proposal 15: For L3 measurement requirements for FDD PCell operation, the measurement period is based on max of switching pattern periodicity (40 ms) and SSB periodicity and DRX cycle if configured.

Proposal 16: L1 measurement requirements for RLM, L1-RSRP measurement and TCI state switch delay should be updated for FDD PCell via switching.

Proposal 17: L3 measurement requirements for intra-frequency measurement without gap and inter-frequency measurement with gap should be updated for FDD PCell via switching.

Proposal 18: The updated requirements for FDD PCell operation is applicable when switching pattern is applied (enabled).

# Summary

In this contribution, we further provided views on RRM requirements impact due to low band carrier aggregation with switching. Following proposals are present.

Proposal 1: Switching pattern is not applied for dormant SCell.

Proposal 2: The switching pattern is applied after UE reports valid CSI for SCell activation.

Proposal 3: If UE should be on PCell according to switching pattern after valid CSI report for SCell activation, UE performs switching according to the configured switching pattern.

Proposal 4: If UE should be on SCell according to switching pattern after valid CSI report for SCell activation, RAN4 to decide how UE performs switching according to the following two options.

Option 1: UE switching to SCell immediately after valid CSI report.

Option 2: UE switching to SCell at the next switching period after valid CSI report.

Proposal 5: The switching pattern is NOT applied after UE completed the SCell deactivation.

Proposal 6: If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to SCell immediately.

Proposal 7: Similar principle is used for dormant SCell if switching pattern is not applicable to dormant SCell.

Proposal 8: UE is to drop invalid CQI reporting and prioritize valid CQI reporting when SSB reception is collided with CQI reporting, or UE doesn’t report invalid CQI during SCell activation.

Proposal 9: Proposal 6 for issue 1-3-8 is used as baseline to define scheduling restriction requirements for SCell activation. In addition, scheduling restriction due to CSI-RS reception should also be defined.

Proposal 10: For activated SCell L3 measurement, the minimum measurement interval is max (SMTC, 40).

Proposal 11: No requirements are defined if there are no overlapped occasions between SMTC and reception duration of SCell.

Proposal 12: L1 measurement requirements for BFD/CBD are not defined for SDL SCell via switching.

Proposal 13: For L1 measurement requirements for SDL SCell operation, the evaluation period or measurement delay is based on max of switching pattern periodicity (40 ms) and L1-RS periodicity and DRX cycle if configured.

Proposal 14: For L1 measurement requirements for FDD PCell operation, the evaluation period or measurement delay is based on max of switching pattern periodicity (40 ms) and L1-RS periodicity and DRX cycle if configured.

Proposal 15: For L3 measurement requirements for FDD PCell operation, the measurement period is based on max of switching pattern periodicity (40 ms) and SSB periodicity and DRX cycle if configured.

Proposal 16: L1 measurement requirements for RLM, L1-RSRP measurement and TCI state switch delay should be updated for FDD PCell via switching.

Proposal 17: L3 measurement requirements for intra-frequency measurement without gap and inter-frequency measurement with gap should be updated for FDD PCell via switching.

Proposal 18: The updated requirements for FDD PCell operation is applicable when switching pattern is applied (enabled).

- References

- R4-2508259	WF on RRM requirements for NR_LBCA_Sw, Apple

- R4-2500828 Discussion on RRM requirements for low band carrier aggregation via switching, vivo

- R4-2504028 Further discussion on RRM requirements for LB CA via switching, vivo

- R4-2506454	Further discussion on RRM requirements for LB CA via switching, vivo

- R4-2504895	WF on RRM requirements for NR_LBCA_Sw, Apple

- R1-2504869	LS on Low NR band carrier aggregation via switching, RAN1