3GPP TSG RAN WG4 Meeting #116	R4-2510327

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.26.4

Source:	vivo

Title:	On RRM performance requirements for XR

Document for:	Discussion

# Introduction

In the past RAN4 meetings, RAN4 discussed RRM core requirements for Rel-19 XR. Gap cancellation was introduced to enable data transmission during configured gap occasions. Corresponding RRM core requirements will be specified to guarantee the functionality of the feature. RRM test cases should also be specified to verification the core requirements.

In this contribution, we provide our views on RRM performance requirements for Rel-19 XR.

# Discussion

In the past RAN4 meeting, following requirements in Table 1 were agreed to be defined for Rel-19 XR as captured in the WF [1].

Table 1. Core requirements for Rel-19 XR.

| Clause | Topic | Volunteer |
| --- | --- | --- |
| 8.xr (new) Or 9.1 | Time offset for DCI based measurement skipping | Nokia |
| 9.2.6 | Intra-freq measurements with gaps | ZTE |
| 9.3.4 | Inter-freq measurements with gaps | Ericsson |
| 9.4.2 | Inter-RAT measurements E-UTRAN FDD measurements | Huawei |
| 9.4.3 | Inter-RAT measurements E-UTRAN TDD measurements | Apple |
| 9.15.5 | Inter-frequency L1-RSRP with MG | vivo |
| NOTE: The full list of clauses with potential impact due to XR skipping is found in the Ad-hoc minutes R4-2502580 |  |  |

In general, test cases should be specified to verify all the introduced new core requirements for XR. The intra-frequency measurement with gaps and inter-frequency measurement with gaps are import for NR measurements. Inter-RAT E-UTRAN measurements are important for mobility. To ensure mobility performance when some of the gaps are cancelled, corresponding tests should be specified.

Inter-frequency L1-RSRP with MG is for UE supporting LTM. Since UE shares gap occasions between neighbor cell L1_RSRP measurements and L3 measurements, the overall mobility performance may be compromised if LTM measurements with gap cancellation is not verified.

Proposal 1: Define test cases to very core requirements for intra-frequency measurement with gaps, inter-frequency measurement with gaps, inter-frequency L1-RSRP measurement with gaps and inter-RAT E-UTRAN measurements when gap cancellation is enabled.

In the RAN1#121 meeting, RAN1 agreed the FG 64-1 for Rel-19 XR.

| 64. NR_XR_Ph3 | 64-1 | Enabling TX/RX during measurement gap scheduling restriction [by DCI format 0_1/1_1] | 1. Reception/transmission during measurement gaps/scheduling restrictions is enabled based on explicit indication by DCI 2. Supported DCI formats for enabling reception/transmission in gap/restriction 3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 behaviour. 4. The minimum time offset from the end of PDCCH carrying the gap/restriction cancellation to the start of cancelled gap/restriction FFS: other component (which may be separate FG) |
| --- | --- | --- | --- |

The supported DCI formats include DCI 0-1/1-1, 0-2/1-2 and 0-3/1-3, which is indicated by separated UE capability.

There are two options for UE behaviour for indication field value of “0” in DCI. RAN4 needs to discuss whether UE behaviour for indication value of “0” is verified in the test also.

Regarding the time offset, 5ms is baseline and 3ms is optional based on RAN4 agreements though UE just reports one of them. In the test, 5ms time offset should be used.

The ratio of gap occasions to be cancelled during the measurement period should also be properly configured. 20% and 40% can be considered as starting point.

Proposal 2: In the tests, at least DCI 0-1 and DCI 1-1 are used for indication of gap cancellation.

Proposal 3: RAN4 to discuss whether UE behavior of option 1 and option 2 for indication value ‘0’ should be tested.

Proposal 4: Time offset for gap cancellation indication is 5ms in the tests.

Proposal 5: Ratio of gap occasions to be cancelled during the measurement period is 20% or 40% in the tests.

# Summary

In this contribution, we further provided views on RRM requirements impact for XR in Rel-19. Following proposals and observations are present.

Proposal 1: Define test cases to very core requirements for intra-frequency measurement with gaps, inter-frequency measurement with gaps, inter-frequency L1-RSRP measurement with gaps and inter-RAT E-UTRAN measurements when gap cancellation is enabled.

Proposal 2: In the tests, at least DCI 0-1 and DCI 1-1 are used for indication of gap cancellation.

Proposal 3: RAN4 to discuss whether UE behavior of option 1 and option 2 for indication value ‘0’ should be tested.

Proposal 4: Time offset for gap cancellation indication is 5ms in the tests.

Proposal 5: Ratio of gap occasions to be cancelled during the measurement period is 20% or 40% in the tests.

- References

- R4-2504911	WF on RRM requirements for NR_XR_Ph3, Nokia