3GPP TSG RAN WG4 Meeting #116	R4-2510332

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.25.4

Source:	vivo

Title:	On performance requirements for mobility enh phase 4

Document for:	Discussion

# Introduction

In the past RAN4 meetings, RAN4 discussed RRM core requirements for Rel-19 LTM. The discussion in RAN4 mainly focus on CSI-RS based L1 measurement, event triggered L1 measurement reporting and conditional LTM.

In this contribution, we provide our views on performance requirements for Rel-19 LTM, including the above three areas.

# Discussion

## 2.1 CSI-RS based L1 measurement

Accuracy requirements

In Rel-18, accuracy requirements for SSB-based L1 measurement for LTM were specified. since both intra-frequency and inter-frequency SSB-based L1 measurements are supported, accuracy requirements were specified for both cases.

For CSI-RS based L1 measurement, the definition of intra/inter frequency measurement is under discussion. At least, intra-frequency measurement without gap for CSI-RS based L1 measurement will be supported. it was agreed not to support gap-based inter-frequency CSI-RS L1 measurement. It is not clear whether inter-frequency measurement without gap will be supported or not.

In the previous RAN4 meeting, following agreement was reached.

Agreement: - RRM requirements (delay and accuracy) apply under the following condition: - At least 48 RBs of the CSI-RS configured for measurement is within the active BWP.

In legacy CSI-RS based measurement for serving cell, the accuracy was specified based on CSI-RS with 48 RBs and density 3. With same side conditions, the legacy accuracy requirements for CSI-RS based L1 measurements for serving cell can be reused.

Proposal 1: For CSI-RS based L1 measurement for LTM, reuse legacy accuracy requirements for CSI-RS based L1 measurements for serving cell under the condition of CSI-RS resources with 48 PRBs and density 3.

Test cases

Compared to SSB-based L1 measurements, the difference is that the measurement is based on CSI-RS and accuracy could be different. The PCell cell switch and PSCell cell switch procedure should be the same regardless of it is based on SSB-based L1 measurements or CSI-RS based L1 measurements.

As discussed above, at least intra-frequency measurement without gap will be supported for CSI-RS based L1 measurement, test cases should be defined accordingly.

Proposal 2: For CSI-RS based L1 measurement for LTM, test cases are introduced to verify measurement delay and accuracy requirements for both FR1 and FR2.

## 2.2 Event triggered L1 measurement reporting

Event triggered L1 measurement reporting apply to both SSB-based L1 measurement and CSI-RS based L1 measurement. There are no new accuracy requirements needed regarding event triggered L1 measurement reporting.

Test cases

It is necessary to verify whether UE can meet event triggered reporting requirements. Since UE measurement behaviour is verified by other tests, there is no need to define many tests to verify all combination of cases/conditions. In our view, four test cases would be enough to cover SSB-based L1 measurement and CSI-RS based L1 measurement in FR1 and in FR2, respectively.

Proposal 3: Define four test cases to verify whether UE can meet event triggered reporting requirements, covering SSB-based L1 measurement and CSI-RS based L1 measurement in FR1 and in FR2, respectively.

## 2.3 Early CSI acquisition

It is under discussion whether to define RRM core requirements for early CSI acquisition for LTM. If core requirements are introduced, test cases should also be introduced accordingly since it is kind of new requirements.

Proposal 4: Define test case(s) to verify early CSI acquisition if core requirements are specified.

## 2.4 Conditional LTM

For the conditional LTM, the discussion on the core part focuses on the definitions of CLTM Cell switch delay requirements and Subsequent CLTM Cell switch delay requirements. Correspondingly, the test cases for these requirements also need to be defined. Taking CLTM Cell switch delay requirements as an example, the main dimensions considered include the following three:

- RACH-based and RACH-less based

- FR1 and FR2

- Intra-frequency and Inter-frequency

When all three dimensions are considered, the resulting number of combinations is relatively large. Therefore, cross-combinations can be defined during the process (e.g., defining only RACH-less cases for intra-frequency and excluding RACH) to reduce the total number of test cases.

Regarding test case configurations, for CLTM Cell switch, the test configurations corresponding to the existing conditional handover test cases can serve as the baseline. For Subsequent CLTM Cell switch, the test configurations corresponding to the existing subsequent CPC test cases can be used as a reference.

Proposal 5: RAN4 to define test cases for CLTM Cell switch delay and Subsequent CLTM Cell switch delay by from the following dimensions. And for test case configurations, existing conditional handover test cases and subsequent CPC test cases can serve as the baseline, respectively.

- RACH-based and RACH-less based

- FR1 and FR2

- Intra-frequency and Inter-frequency

# Summary

In this contribution, we further provided our views on performance requirements for Rel-19 LTM. Following proposals and observations are present.

Proposal 1: For CSI-RS based L1 measurement for LTM, reuse legacy accuracy requirements for CSI-RS based L1 measurements for serving cell under the condition of CSI-RS resources with 48 PRBs and density 3.

Proposal 2: For CSI-RS based L1 measurement for LTM, test cases are introduced to verify measurement delay and accuracy requirements for both FR1 and FR2.

Proposal 3: Define four test cases to verify whether UE can meet event triggered reporting requirements, covering SSB-based L1 measurement and CSI-RS based L1 measurement in FR1 and in FR2, respectively.

Proposal 4: Define test case(s) to verify early CSI acquisition if core requirements are specified.

Proposal 5: RAN4 to define test cases for CLTM Cell switch delay and Subsequent CLTM Cell switch delay by from the following dimensions. And for test case configurations, existing conditional handover test cases and subsequent CPC test cases can serve as the baseline, respectively.

- RACH-based and RACH-less based

- FR1 and FR2

- Intra-frequency and Inter-frequency

- References

- R4-2502601	WF on RRM requirements for NR_Mob_Ph4_Part1, Apple