3GPP TSG RAN WG4 Meeting #116	R4-2510335

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.17.2

Source:	vivo

Title:	Discussion on general aspects for AI/ML for NR air interface

Document for:	Discussion

- Introduction

Regarding the general aspects of AI/ML in Part 1, it mainly includes content on BM and CSI prediction. In this contribution, we provide our views for the issue on monitoring report delay for BM and CSI prediction. The latest way forward was captured in [1].

- Discussion

  - Monitoring reporting delay and accuracy requirements for BM

Monitoring reporting delay and accuracy: Discuss how to define delay and accuracy requirements for UE sided monitoring where the UE reports the metric: - BM-Case1 and possibly BM-Case 2 o Performance monitoring type 1 option 2 - If it proves to be not possible to define, then reverse the agreement - Note: For BM-Case2, it may not be completed in Rel-19. If it is not covered in Rel-19, there would not be a monitoring delay requirement. If it would be in Rel-19, a monitoring delay requirement is needed.

In last meeting, we had a preliminary discussion on the definition of monitoring reporting delay and accuracy. For reference in the following discussion, we copy the latest RAN1 conclusions on performance monitoring for BM as follows.

Agreement - At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring, - for BM-Case 1, the measurement result of nth (n = 1,..,N) latest transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the nth transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring - Predefines a threshold X = 64 for the minimal slot offset, which is configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report. - Note: CSI reference resource corresponding to the CSI report for monitoring and inference is determined based on legacy for all types (P/SP/AP) of CSI report carrying L1-RSRP, considering discussing associated timeline separately - The associated working assumption made in RAN1#120b will not be confirmed. Agreement For beam prediction accuracy report for monitoring, the report quantity RS-PAI is Np (0 ≤ Np≤ N) - Where Np is the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s) of monitoring resources that no later than CSI reference resource corresponding to the CSI report for monitoring - condition: - for the transmission occasion of monitoring resources, it has a linked inference report - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources mapped to one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the CSI Reporting Setting for inference - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance. - Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig - the size of CSI field associated with the RS-PAI is log2(N+1) Agreement For BM-Case 1, one resource set for monitoring is configured in one CSI-ReportConfig for monitoring. Agreement For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring, - only one resource set is configured in the CSI-ReportConfig - the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring - the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource, no later than CSI reference resource corresponding to the CSI report for monitoring - The measurement result of nth (n = 1,..,N) latest transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the nth transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, are no later than the CSI reference resource corresponding to the CSI report for monitoring - Predefines a threshold X =64, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance. - Note: CSI reference resource corresponding to the CSI report for monitoring is determined based on legacy for all types (P/SP/AP) of CSI report carrying L1-RSRP The associated working assumption made in RAN1#120b will not be confirmed.

- Performance monitoring reporting delay

Firstly, for monitoring delay, according to our understanding, this delay defines whether the UE can report the performance metrics within a certain latency upon performance monitoring is required. Compared to the measurement delay for prediction, the delay requirement for performance monitoring should be similar, with the difference being:

For the KPI of performance monitoring, For BM-Case 1, RAN1 defines it as at least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A. Based on this, the content ultimately reported by the UE to the network is the number of correct predictions made by the UE out of N KPI calculations. Therefore, this process monitors the prediction results over N instances, and thus the delay should correspondingly scale to N times. Specifically, the delay for performance monitoring Tmoitoring_period_BM can be defined as:

Tmoitoring_period_BM = Nmonitor* TL1-RSRP_Prediction_Period_CSI-RS,

Where Nmonitor is the configured number of the latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring

For BM-Case 2, in addition to the above considerations, it needs to consider the prediction results are for future time instances. When defining this delay, it may related to the factors on the performance metric (which is calculated per f-th time instance) and the configured length of the time instance gap. The specific delay definition can be deferred until RAN4 determines whether to specify RRM requirements for Case 2 (or which requirements to define) before further discussion.

Proposal 1: For performance monitoring reporting delay for BM-Case 1, the correasponding delay is defined as

Tmoitoring_period_BM = Nmonitor* TL1-RSRP_Prediction_Period_CSI-RS,

Where Nmonitor is the configured number of the latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring。

Proposal 2：For performance monitoring reporting delay for BM-Case 2, the specific delay definition can be deferred until RAN4 determines whether to specify RRM requirements for Case 2 (or which requirements to define) before further discussion.

- Performance monitoring reporting accuracy

For performance monitoring metrics, we can follow the metrics defined by RAN1, as specified below. For performance monitoring, the final content reported by the UE is the report quantity RS-PAI Np (0 ≤ Np ≤ N). Therefore, to verify the accuracy of UE monitoring, it is necessary to confirm whether the UE correctly reports the count of Np out of N monitoring instances. For example:

If the actual count of Np is 7, but the UE reports Np = 6, then the UE is considered to have reported an incorrect RS-PAI. During testing, we can evaluate the rate of correct report quantity RS-PAI observed across repeated tests. This ratio shall not fall below X% (e.g., 90%).

Agreement For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as: - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling) - Where K is the number of predicted beam(s) in the corresponding inference report per time instance - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, 2

Proposal 3: For Performance Monitoring Reporting Accuracy, it camn be defined in the test as: The rate of correct report quantity RS-PAI Np observed during repeated tests, where the report quantity RS-PAI follows RAN1's definition. Specifically, for BM-Case 2, the rate of correct report quantity RS-PAI reporting should be evaluated per time instance (if defined).

  - Monitoring reporting delay and accuracy requirements for CSI prediction

Monitoring reporting delay and accuracy: CSI prediction: - CSI prediction, the details are subject to RAN1 decision - RAN4 will further study the feasibility to specify the corresponding delay and/or accuracy requirements for UE sided monitoring

Currently, RAN1 has finished the R19 WI of CSI prediction. Some RAN1 LCM agreements have been achieved as following.

RAN1#120bis Agreement For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring Agreement For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric. Agreement For the definition of SGCS, Note: How to handle layer mapping mismatch, if any, is up to UE implementation. RAN1#121 Agreement For CSI prediction using UE-side model, for UE assisted performance monitoring, - Support to reuse CSI framework for the configuration for monitoring result report in L1 signaling - Dedicated resource set for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - For monitoring report type, semi-persistent and aperiodic are supported - The following combination for inference report type and monitoring report type are supported Monitoring report type Inference report type; SP report; AP report. AP report; Not support; Support. SP report; Support; Support. - For monitoring resource type for measurement, periodic, semi-persistent and aperiodic are supported. Agreement - For CSI prediction using UE-side model, for UE assisted performance monitoring, - If the inference measurement resource is aperiodic CSI-RS, UE is expected to receive the monitoring report trigger, if any, no later than the first symbol of the earliest occasion of K inference measurement resources that are no later than the CSI reference resource of the associated inference report; - Additional active resource/port counting time for the aperiodic resource of the inference report is from the end of the inference report to the end of the triggered monitoring report - If inference report is aperiodic and the inference measurement resource is periodic or semi-persistent, UE is expected to receive the monitoring report trigger, if any, no later than the first symbol of the earliest occasion of the most recent Kp inference measurement resource transmission occasions that are no later than the CSI reference resource of the associated inference report Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, - one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCS is reported - wideband frequency granularity - only for one prediction instance configured by NW - per layer, and the total number of layers is the same as the reported RI in the associated inference report - Introduce new RRC parameter for reportQuantity, e.g., ‘SGCS-r19’ - Each SGCS value is quantized with 4-bit - 15 codepoints are used to uniformly quantize in range [0.3 1] in linear scale. - One codepoint is used to indicate the value range of (0 0.3] - Only one monitoring resource set is configured in the CSI-ReportConfig - The one configured time instance (i.e. f-th doppler domain unit in one inference report) for SGCS calculation is configured in the CSI-ReportConfig for monitoring, for N4>1

Currently, RAN1 has agreed that UE assisted performance monitoring is based on Type 3 performance monitoring with SGCS reporting.

In Type 3 performance monitoring, one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance.

The SGCS is reported in wideband frequency granularity, only for one prediction instance configured by NW, and per layer, and the total number of layers is the same as the reported RI in the associated inference report. RAN4 to discuss Type 3 performance monitoring for UE assisted performance monitoring of CSI prediction. RAN4 to discuss whether to capture it in the RRM spec.

Proposal 4: RAN4 to discuss Type 3 performance monitoring for UE assisted performance monitoring of CSI prediction. RAN4 to define the delay requirement and accuracy requirement of SGCS A and SGCS B.

- The delay for performance monitoring is only for one instance.

- In RAN1 agreement, SGCS A is calculated based on predicted CSI for one inference reporting, and ground truth CSI, SGCS B is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance.

- Summary

Proposal 1: For performance monitoring reporting delay for BM-Case 1, the correasponding delay is defined as

Tmoitoring_period_BM = Nmonitor* TL1-RSRP_Prediction_Period_CSI-RS,

Where Nmonitor is the configured number of the latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring。

Proposal 2：For performance monitoring reporting delay for BM-Case 2, the specific delay definition can be deferred until RAN4 determines whether to specify RRM requirements for Case 2 (or which requirements to define) before further discussion.

Proposal 3: For Performance Monitoring Reporting Accuracy, it camn be defined in the test as: The rate of correct report quantity RS-PAI Np observed during repeated tests, where the report quantity RS-PAI follows RAN1's definition. Specifically, for BM-Case 2, the rate of correct report quantity RS-PAI reporting should be evaluated per time instance (if defined).

Proposal 4: RAN4 to discuss Type 3 performance monitoring for UE assisted performance monitoring of CSI prediction. RAN4 to define the delay requirement and accuracy requirement of SGCS A and SGCS B.

- The delay for performance monitoring is only for one instance.

- In RAN1 agreement, SGCS A is calculated based on predicted CSI for one inference reporting, and ground truth CSI, SGCS B is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance.

- References

- R4-2508051, WF on NR_AIML_air_part2, Ericsson