3GPP TSG RAN WG4 Meeting#116                                    R4- 2510338

Bengaluru, India, August 25th – 29th, 2025

Agenda item:			7.17.5

Source:	vivo

Title:	Discussion on RRM core requirement and testing framework for Positioning accuracy enhancement

Document for:	Discussion

- Introduction

RAN4 has studied many aspects about AI/ML use case aspects in Rel-18 SI stage, including CSI feedback enhancement, beam management enhancement and positioning accuracy enhancement. The outcomes of the study for positioning are captured in TR 38.843 [1]. In last meeting, the following agreement is achieved.

Issue 3-1: Requirements for case 1 Agreement: - Discuss how to define a measurement delay requirement (reporting delay) - Do not define any positioning accuracy requirement Issue 3-2: Report mapping for sample based timing measurement reporting Agreement: - Use the value of the legacy UL RTOA mapping for 1st sample - FFS for the additional sample reporting Issue 3-3: Requirements for case 3b Agreement: - Reuse legacy path based Rx-Tx accuracy requirements for case 3b - Requirement applies only for the 1st path

In this contribution, we further provide our views on RRM core requirements and test framework for positioning accuracy enhancement.

- Discussion

For AI/ML positioning cases, 5 sub-use cases are identified and summarized in Table 1.

- A detailed list of various positioning cases

| Cases | Priority | Model deployment | Positioning methods | Measured by which entity | Model output | Position calculation |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 1st priority | UE-side | Direct AI/ML positioning | UE (Downlink positioning) | Position | UE-side |
| Case 2a | 2nd priority | UE-side | AI/ML assisted positioning | UE (Downlink positioning) | Intermediate feature | LMF |
| Case 2b | 2nd priority | LMF-side | Direct AI/ML positioning | UE (Downlink positioning) | Position | LMF |
| Case 3a | 1st priority | gNB-side | AI/ML assisted positioning | gNB (uplink positioning) | Intermediate feature | LMF |
| Case 3b | 1st priority | LMF-side | Direct AI/ML positioning | gNB (uplink positioning) | Position | LMF |

- 2.1	Potential RAN4 requirement for case 1

In RAN2#126 meeting, agreements about UE side model are reached as

Agreements - The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities. FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality. - wait for RAN1 for associate ID discussion - At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation. FFS further details on signaling enhancements

For case 1, RAN2 agreed to use the existing LPP procedures related to Location Information Transfer to provide the UE sided model inference results, which is the UE position, to the NW side. It means that there is a potential interactive process between NW and UE. RAN4 could consider define a delay requirement from the start of UE measuring PRS, through the AI processing, to the end of UE report location to LMF. Since further signaling enhancements may needed in RAN2, more progresses are also needed for defining delay requirement for case 1.

Proposal 1: RAN4 to define delay requirement for case 1 at least considering the following procedures: PRS measuring, AI processing. Legacy requirement (if exists) can be reused.

- 2.2	Potential RAN4 requirement for case 3

For the UL positioning method case 3a and 3b, the model is deployed in gNB side and LMF side, respectively. RAN4 does not need to define any new requirement for such 2 cases. Legacy requirement (if exists) can be reused.

Proposal 2: RAN4 not to define new requirements for positioning case 3. Legacy requirement (if exists) can be reused.

- 2.3 Potential RAN4 requirement for performance monitoring

In RAN1 meetings, following agreements about performance monitoring for case 1 and case 3a are reached as

RAN1#116bis Agreement For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility of the following options. To provide information on how to generate information on ground truth label for each option. - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope, but RAN1 can make recommendation about the option(s), and potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform the monitoring metric calculation is up to implementation Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. RAN1#118 For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved. Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4. RAN1#119 Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B RAN1#120bis Agreement For model performance monitoring of AI/ML positioning Case 1, “FFS: content of monitoring outcome” in RAN1#119 agreement is resolved by: - the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method. RAN1#121 Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - There is no consensus in RAN1 to support Option B (including Option B-1 and B-2).

For case 3a, the monitoring procedures are performed at NW side and no need to define delay requirement for RAN4. For case 1, there are many options under discussion that may have RAN4 impacts for delay requirements.

For option A-1, the label-based monitoring metrics calculation is performed at UE side and the ground truth label is generated by LMF and provided to the target UE. In that case, LMF may indicate UE to perform monitoring procedure through signaling and then the target UE may inform LMF the monitoring decision or monitoring metrics calculation results (NW decision) through report. These procedures imply a delay requirement and the details are depending on RAN1/2 progress.

For option A-2 and 3, the label-based monitoring metrics calculation is performed at UE side and some assistance data can be transferred from LMF to UE side. These procedures may also have delay requirement and more progress of RAN1/2 are needed for discussing.

For option A-4, the PRU measurement are sent from PRU to the target UE, it is more like implementations and vendors could use different ways for sending information, e.g., OTT server, sidelink. Since RAN1 has agreed that it can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. RAN4 would not discuss this option for defining any core requirements.

For option B-1, the label-based monitoring metrics calculation is performed at LMF side and the model inference needs to be sent from target UE to LMF. In that case, it is very clear for RAN4 to identify the delay requirement. For delay requirement, the LMF may indicate UE to perform AI operations, and then UE reports the inference results (e.g. AI estimated UE position) to LMF.

For option B-2, the target UE uses the PRU channel measurements as the model input and outputs the estimated PRU position as the model inference result to report to LMF for performing monitoring procedure. It is also very clear that these procedures may also have the delay requirement from RAN4 perspective.

It is seen that Option A is supported and there is no consensus in RAN1 to support Option B (including Option B-1 and B-2). It can be observed that the delay requirements for many options about the monitoring procedure in case 1 needs to be defined, but more progress and down-selection from RAN1/2 are needed for delated further discussion.

Proposal 3: RAN4 to consider define delay requirements for monitoring Option A-1, A-2 and A-3 for positioning case 1.

- Summary

In this contribution, we provided our views on AI/ML based positioning. Based on above analysis, following observations and proposals are present.

Proposal 1: RAN4 to define delay requirement for case 1 at least considering the following procedures: PRS measuring, AI processing. Legacy requirement (if exists) can be reused.

Proposal 2: RAN4 not to define new requirements for positioning case 3. Legacy requirement (if exists) can be reused.

Proposal 3: RAN4 to consider define delay requirements for monitoring Option A-1, A-2 and A-3 for positioning case 1.

- References

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18).