3GPP TSG-RAN WG4 Meeting #116	R4-2510356

# Bengaluru, India, August 25th – 29th, 2025

# Source: 	ZTE Corporation, Sanechips

# Document for:	Approval

# Title: 	Simulation results of NR NTN Ku band co-existence study

# Agenda item:	7.7.3

# 1 Introduction

In RAN#107 meeting, a revised Work Item [1] was approved on introduction of Ku Band for NR NTN for NR NTN with the following RAN4 objective. In this contribution, we want to share some further views about NR NTN Ku band from coexistence perspective.

The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate Tx/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services. - Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements - Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only.

# 2 Discussion

Based on the agreed simulation assumptions, we share the simulation results as follows.

  - 2.1 Scenario 1: 14GHz NTN UL interfering TN UL

Table 2.1-1: Interpolated ACIR values for Scenario 1 to meet the 5% throughput loss criteria

|  |  | Interpolated ACIR[dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | GEO |  |  | LEO600 |  |
| Elevation angle | Throughput Loss | Parabolic | 45*45 | 78*78 | Parabolic | 45*45 |
| 90° | Average | 6.77 | 0 | 0 | 0 | 0 |
|  | 5%-tile | 33.04 | 20.71 | 7.32 | 8.43 | 0 |
| 35° | Average | 7.8 | 0 | 0 | 0 | 0 |
|  | 5%-tile | 34.03 | 22.57 | 20.77 | 30.15 | 18.91 |

  - 2.2 Scenario 4: 14GHz TN DL interfering NTN UL

Table 2.2-1: Interpolated ACIR values for Scenario 4 to meet the 5% throughput loss criteria

|  |  | Interpolated ACIR[dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | GEO |  |  | LEO600 |  |
| Elevation angle | Throughput Loss | Parabolic | 45*45 | 78*78 | Parabolic | 45*45 |
| 90° | Average | 7.17 | 6.96 | 6.37 | 12.93 | 13.74 |
|  | 5%-tile | 8.98 | 8.82 | 7.35 | 12.94 | 13.75 |
| 35° | Average | 33.36 | 33.04 | 34.25 | >13 | >10 |
|  | 5%-tile | NaN | NaN | NaN | >30 | >26 |

  - 2.3 Scenario 5: 11GHz TN DL interfering NTN DL

Table 2.3-1: Interpolated ACIR values for Scenario 5 to meet the 5% throughput loss criteria

|  |  | Interpolated ACIR[dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | GEO |  |  | LEO600 |  |
| Elevation angle | Throughput Loss | Parabolic | 45*45 | 78*78 | Parabolic | 45*45 |
| 90° | Average | 28 | 12.64 | 4.2 | 36.74 | 14.65 |
|  | 5%-tile | 35.8 | 30.04 | 20.57 | 46.37 | >10 |
| 35° | Average | 33.22 | 13.51 | 9.29 | 37.74 | 16.77 |
|  | 5%-tile | NaN | NaN | NaN | NaN | NaN |

  - 2.4 Scenario 6: 11GHz NTN DL interfering TN DL

Table 2.4-1: Interpolated ACIR values for Scenario 6 to meet the 5% throughput loss criteria

|  |  | Interpolated ACIR[dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | GEO |  |  | LEO600 |  |
| Elevation angle | Throughput Loss | Parabolic | 45*45 | 78*78 | Parabolic | 45*45 |
| 90° | Average | 0 | 0 | 0 | 0 | 0 |
|  | 5%-tile | 0 | 0 | 0 | 0 | 0 |
| 35° | Average | 0 | 0 | 0 | 0 | 0 |
|  | 5%-tile | 0 | 0 | 0 | 0 | 0 |

  - 2.5 Observations

Observation 1: For case 4 and scenario 4 and 5, interpolated ACIR values to meet the 5% throughput loss criteria is NaN when elevation angle is 35°due to large interference.

Observation 2: The required ACIR for NTN Ku band coexistence is as follows:

| Scenario |  | 1 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- |
| Frequency |  | 14GHz | 14GHz | 11GHz | 11GHz |
| Scenario case |  | Aggressor NTN UL to Victim TN UL | Aggressor TN DL to Victim NTN UL | Aggressor TN DL to Victim NTN DL | Aggressor NTN DL to Victim TN DL |
| Required ACIR[dB] | GEO | 34.03 | 34.25 | 35.8 | 0 |
|  | LEO600 | 30.15 | 13.75 | 46.37 | 0 |

# 3 Conclusion

In this contribution, we shared coexistence simulation results for NTN Ku band and the observations are made as follows:

Observation 1: For case 4 and scenario 4 and 5, interpolated ACIR values to meet the 5% throughput loss criteria is NaN when elevation angle is 35°due to large interference.

Observation 2: The required ACIR for NTN Ku band coexistence is as follows:

| Scenario |  | 1 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- |
| Frequency |  | 14GHz | 14GHz | 11GHz | 11GHz |
| Scenario case |  | Aggressor NTN UL to Victim TN UL | Aggressor TN DL to Victim NTN UL | Aggressor TN DL to Victim NTN DL | Aggressor NTN DL to Victim TN DL |
| Required ACIR[dB] | GEO | 34.03 | 34.25 | 35.8 | 0 |
|  | LEO600 | 30.15 | 13.75 | 46.37 | 0 |

# 4 Reference

- RP-243314, Revised WID: Introduction of Ku bands for NR NTN, Intelsat et al.