3GPP TSG-RAN WG4 Meeting #116	R4-2510359

Bengaluru, India, August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | - | rev | - | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to TS38.101-5 Add 9.5.1, 9.5.2 and 9.7 requirements for NTN Ku bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, CHTTL |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2025-07-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of NTN Ku bands. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add occupied bandwidth, out of Band Emissions and additional regional requirements indicated by NS for NTN Ku bands based on the agreed work split. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Occupied bandwidth, out of Band Emissions and additional regional requirements indicated by NS for NTN Ku bands are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.1, 9.5.2, 9.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# << Start of change >>

9.5.1	Occupied bandwidth

Occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel. The occupied bandwidth for all transmission bandwidth configurations (Resources Blocks) shall be less than the channel bandwidth specified in Table 9.5.1-1 and Table 9.5.1-2.

The occupied bandwidth is defined as a directional requirement. The requirement is verified in beam locked mode with the test metric of OBW (Link=TX beam peak direction, Meas=Link angle).

Table 9.5.1-1: Occupied channel bandwidth for FR2-NTN

|  | Occupied channel bandwidth / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Channel bandwidth (MHz) | 50 | 100 | 200 | 400 |

Table 9.5.1-2: Occupied channel bandwidth for FR1-NTN (above 10 GHz)

|  | Occupied channel bandwidth / Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz |
| Channel bandwidth (MHz) | 10 | 15 | 20 | 25 | 35 | 50 | 70 | 100 |

9.5.2	Out of Band Emissions

9.5.2.1	General

The Out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and an adjacent channel leakage power ratio. Additional requirements to protect specific bands are also considered.

The requirements in sub-clause 9.5.2.2 only apply when both UL and DL of an NTN VSAT are configured for single CC operation, and they are of the same bandwidth.

All out of band emissions for FR1-NTN (above 10 GHz) and FR2-NTN are specified as TRP.

The spectrum emission mask of the NTN VSAT applies to frequencies starting from the  edge of the assigned NR channel bandwidth.

9.5.2.2	Spectrum emission mask

9.5.2.2.1	General NR spectrum emission mask

The power of any NTN VSAT emission shall not exceed the Basic limits specified in Table 9.5.2.2-1 for the specified channel bandwidth. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid). Where:

-	f is the separation between the Transmission BW channel edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the channel edge frequency and the centre of the measuring filter.

Table 9.5.2.2.1-1: General NR spectrum emission mask for FR1-NTN (above 10 GHz) and FR2-NTN

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 2× BW | 0.5 MHz f_offset < 2× BW + 0.5 MHz | max11, TRPrated – 10log10(BW) –40×log10 f_offset−0.5BW×2+1dBm | 1 MHz |
| NOTE 1: TRPrated is the declared rated output power lower than or equal to TRPmax specified in sub-clause 9.2.1; NOTE 2: Transmission BW is in the unit of MHz; NOTE 3: The 11dBm/1MHz value corresponds to the spurious emission limit specified in spurious emission sub-clause 9.5.3, and is converted from the SE limit requirement defined on 4 kHz to a value defined over 1 MHz; NOTE 4: PSD attenuation as in ITU-R SM.1541-6 [6], Annex 5 OoB domain emission limits for earth stations. |  |  |  |

NOTE:	When the UE is operating in an NGSO deployment, to support coexistence, it is assumed that a guardband at least equivalent to the maximum doppler shift expected for the NGSO constellation between the channel edge of the channel bandwidth operated by the UE and the spectrum block edge has been accounted for as part of system deployment configuration by the operator.

9.5.2.2.2	Additional spectrum emission mask

For bands n511 and n510, and for bands with network signalling values NS_203N, NS_204N, NS_207N and NS_208N, the mean power of emissions shall be attenuated below the mean output power of the transmitter (measured in dBm) in accordance with [FCC 25.202].

The power of any NTN VSAT emission shall not exceed the Basic limits specified in Table 9.5.2.2.2-1 for the specified channel bandwidth. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid). Where: f_offset is the separation between the channel edge frequency and the centre of the measuring filter.

Table 9.5.2.2.2-1: Additional spectrum emission mask

| Frequency offset of measurement filter centre frequency, f_offset | Basic limits (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0.002MHz+0.5xBW f_offset < 1xBW-0.002MHz | TRPrated(dBm) - 25 dB | 4 kHz |
| 0.002MHz+1xBW f_offset < 2.5xBW-0.002MHz | TRPrated(dBm) - 35 dB | 4 kHz |
| 0.002MHz+2.5xBW f_offset < 2nd harmonic of the upper frequency edge of the UL operating band in GHz | -13 dBm | 4 kHz |
| NOTE 1: TRPrated is the declared rated output power lower than or equal to TRPmax specified in sub-clause 9.2.1; NOTE 2: Transmission BW is in the unit of MHz; NOTE 3: Measurement bandwidths as in ITU-R SM.329 [16], s4.1. NOTE 4: Upper frequency as in ITU-R SM.329 [16], s2.5 table 1. |  |  |

9.5.2.3	Adjacent channel leakage ratio

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. ACLR requirement is specified for a scenario in which adjacent carrier is another NR channel.

NR Adjacent Channel Leakage power Ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 9.5.2.3-1 for FR2-NTN and Table 9.5.2.3-2 for FR1-NTN (above 10 GHz).

If the measured adjacent channel power is greater than [–35] dBm then the NRACLR shall be higher than the value specified in Table 9.5.2.3-1. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

Table 9.5.2.3-1: General requirements for NRACLR for FR2-NTN

|  | Channel bandwidth / NRACLR / Measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| NRACLR for band n512, n511, n510 | 14 dB | 14 dB | 14 dB | 14 dB |
| NRACLR for band n509, n508 | TBD | TBD | TBD | TBD |
| NR channel measurement bandwidth (MHz) | 47.58 | 95.16 | 190.20 | 380.28 |
| Adjacent channel centre frequency offset (MHz) | +50 / -50 | +100 / -100 | +200 / -200 | +400 / -400 |

Table 9.5.2.3-2: General requirements for NRACLR for FR1-NTN (above 10 GHz)

|  | Channel bandwidth / NRACLR / Measurement bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 15 | 20 | 25 | 35 | 50 | 70 | 100 |
| NRACLR for band n248, n247 | TBD |  |  |  |  |  |  |  |
| NR channel measurement bandwidth (MHz) | 9.375 | 14.235 | 19.095 | 23.955 | 33.855 | 48.615 | 68.07 | 98.31 |
| Adjacent channel centre frequency offset (MHz) | +10 / -10 | +15 / -15 | +20 / -20 | +25 / -25 | +35 / -35 | +50 / -50 | +70 / -70 | +100 / -100 |

<< Next change >>

9.7	Additional regional requirements indicated by NS

9.7.1	General

Additional regional requirements can be signalled by the network. Each group of additional regional requirements is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR NTN frequency band number of the applicable FR1-NTN operating bands (above 10 GHz) and FR2-NTN operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [8].

Table 9.7.1-1 specifies the additional regional requirements with their associated network signalling values, the applicable satellite orbit scenario(s) and applicable FR1-NTN operating bands (above 10 GHz) and FR2-NTN operating band(s) for each NS value. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 9.7.1-2.

Table 9.7.1-1: Additional regional requirements indicated by Network Signalling label

| Network Signalling label | Requirements (clause) | Applicable Satellite orbit scenario | NR satellite Band | Channel bandwidth (MHz) |
| --- | --- | --- | --- | --- |
| NS_200N |  | GSO and LEO | Table 5.2.2-1 Table 5.2.3-1 | Table 5.3.5-1 or Table 5.3.5-2 |
| NS_201N | Clause 9.2.2.3 Clause 9.5.3.2 Clause 9.5.3.3 Clause 9.6.1.1 Clause 9.6.1.2 Clause 10.8.1 | GSO | n512 | 50, 100, 200, 400 |
| NS_202N | Clause 9.5.3.2 Clause 9.5.3.3 Clause 9.6.1.1 | LEO | n512 | 50, 100, 200, 400 |
| NS_203N | Clause 9.2.2.4 Clause 9.5.2.2.2 Clause 9.6.2.2 | GSO | n509 | 50, 100, 200, 400 |
|  |  |  | n248 | 10, 15, 20, 25, 35, 50, 70, 100 |
| NS_204N | Clause 9.5.2.2.2 | LEO | n509 | 50, 100, 200, 400 |
|  |  |  | n248 | 10, 15, 20, 25, 35, 50, 70, 100 |
| NS_205N | Clause 9.2.2.5 Clause 9.5.3.2 Clause 9.5.3.3 Clause 10.8.2 | GSO | n509 | 50, 100, 200, 400 |
|  |  |  |  |  |  | n248 |  | 10, 15, 20, 25, 35, 50, 70, 100 |
| NS_206N | Clause 9.5.3.2 Clause 9.5.3.3 Clause 10.8.2 | LEO | n509 | 50, 100, 200, 400 |
|  |  |  | n248 | 10, 15, 20, 25, 35, 50, 70, 100 |
| NS_207N | Clause 9.2.2.5 Clause 9.5.2.2.2 Clause 9.6.2.2 | GSO | n508 | 50, 100, 200 |
|  |  |  | n247 | 10, 15, 20, 25, 35, 50, 70, 100 |
| NS_208N | Clause 9.5.2.2.2 | LEO | n508 | 50, 100, 200 |
|  |  |  | n247 | 10, 15, 20, 25, 35, 50, 70, 100 |

Table 9.7.1-2: Mapping of network signalling label

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n512 | NS_200N | NS_201N | NS_202N |  |  |  |  | Reserved |
| n511 | NS_200N |  |  |  |  |  |  | Reserved |
| n510 | NS_200N |  |  |  |  |  |  | Reserved |
| n509 | NS_200N | NS_203N | NS_204N | NS_205N | NS_206N |  |  | Reserved |
| n508 | NS_200N | NS_207N | NS_208N |  |  |  |  | Reserved |
| n248 | NS_200N | NS_203N | NS_204N | NS_205N | NS_206N |  |  | Reserved |
| n247 | NS_200N | NS_207N | NS_208N |  |  |  |  | Reserved |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of 3GPP TS 38.331 [8]. NOTE 2: For band n511 and n510, only NS_200N can be used to map. |  |  |  |  |  |  |  |  |

# << End change >>