3GPP TSG RAN4 Meeting #116																R4-2510369

Bangalore, India 25th – 29th Aug, 2025

Title: 	Discussion on modifiedMPR

Agenda Item:	4.2.2

Source: 	Nokia

Document for:	Discussion

# 1				Introduction

ModifiedMPR tables were discussed in previous RAN4 meeting and WF was approved [1]. In WF following issues were listed which we will discuss in this paper.

- Check the difference between bit ‘0’ and absent from BS and UE perspective.

- Check if the spec version number can be removed from next release.

- Check if the requirements are changed several times, how to handle the legacy bits.

  - See n41

| n41 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)41AA UE EN-DC |
| --- | --- | --- | --- |
|  | 1 | EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_41A_n41A EN-DC |
|  | 2 | EN-DC contiguous and non-contiguous intraband MPR and A-MPR as defined in 38.101-3 v16.4.0. If this bit is not set the UE uses Rel-15 MPR or A-MPR for EN-DC contiguous and non-contiguous intraband MPR and A-MPR | -This bit may be set to 1 by a UE supporting DC_(n)41AA or DC_41A_n41A EN-DC |

- Check if any good approach on how to handle the bits for different form factors, if dedicated bits should be used to differentiate the form factors, FWA and smart phone, for future new cases.

| 3 | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |
| --- | --- | --- |

# 2	Discussion

ModifiedMPR-Behaviour bit are used to indicate also other UE capabilities than MPR/A-MPR which was original intent as there is no other means to do that. In this paper we focust mainly on MPR/A-MPR indications.

In this section we go trough WF [1] issues briefly one by one.

Issue 1: Check the difference between bit ‘0’ and absent from BS and UE perspective.

TS 36.331 says i.e. absence means no support for modified mpr behaviour i.e. same as all bits are zero BUT as 36.331 also says UE is not allowed to send this field with all the bits with zero. Hence from BS point of view a case that UE does not send anything should never happen for UEs that supports release later than when modifiedMPR bit was introduced.

Issue 2: Check if the spec version number can be removed from next release.

It is acknowledged that current specification wording concearning specification version number is little bit ambiguous. What the version number indicates is the version which has the “new” modified MPR/A-MPR definition specified. UEs made after the modification even if complying to earlier release can indicate to BS that they support the new modified MPR/A-MPR with this signalling. There has been proposal to remove the version number atleast from later versions of 307 as UE complying to latest release will support MPR/A-MPR of the latest version anyways.

We see not benefit of removing version number from 307. Changes should be kept minimum for such a mature feature (since LTE). What was proposed in previous RAN4 meeting was to add clarification into 307. Possible informative note is proposed below and use symbols which are already used and defined in 307

NOTE 2: Specification version in the “Definition” column in Table L.1-1 refer to the version N in which the requirements were modified. If UE supports release P which is later than release N then UE supports requirements defined in that release P.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

N	Release in which a feature is introduced into TS 38.101 [2-5, 8] or TS 38.133 [6]

M	Release from which onwards (including release M) a feature is release independent

P	Represent the present release of this specification

Issue 3: Check if the requirements are changed several times, how to handle the legacy bits.

| n41 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)41AA UE EN-DC |
| --- | --- | --- | --- |
|  | 1 | EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_41A_n41A EN-DC |
|  | 2 | EN-DC contiguous and non-contiguous intraband MPR and A-MPR as defined in 38.101-3 v16.4.0. If this bit is not set the UE uses Rel-15 MPR or A-MPR for EN-DC contiguous and non-contiguous intraband MPR and A-MPR | -This bit may be set to 1 by a UE supporting DC_(n)41AA or DC_41A_n41A EN-DC |

UEs designed before v15.5.0 do not know that there is modified MPR behavior defined and send nothing.

UEs designed after v15.5.0 but earlier than 16.4.0

If UE is Rel14 UE or earlier, it may set bits 0 or 1 as “1” but it does not have to

If UE is Rel15 UE or later  but earlier that 16.4.0 it shall set bit 0 or 1 as “1”

UEs designed after v16.4.0

If UE is Rel14 UE or earlier, it may set bit 0 or 1 or 2 as “1” but it does not have to

If UE is Rel15 UE or later but earlier that 16.4.0 it shall sets bit 0 or 1 as “1” or it may set bit 2 as “1” in which case it sets sets bit 0 or 1 as “0”

If UE is Rel16 UE or later, it shall set bit 2 as “1” and bits 0 and 1 as “0”

Issue 4: Check if any good approach on how to handle the bits for different form factors, if dedicated bits should be used to differentiate the form factors, FWA and smart phone, for future new cases.

| 3 | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |
| --- | --- | --- |

Example above is a case where a bit is assigned to a different form factor i.e. FWA device. The handling different form factors in above example is in principle ok if the description text is properly written and clear. But current text is maybe bit soft as it uses word “intended” which is not proper specification language. Instead, it could say “FWA UE shall set this bit.”

# 3	Conclusion

In this contribution we have discussed issues in WF on Modified MPR Behavior from previous meeting.

# 4	References

[1] R4-2507957, WF on Modified MPR Behavior, RAN4#115, Xiaomi