3GPP TSG-RAN4 Meeting # 116	R4-2510378

# Bengaluru, India,  25th August - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_IoT_NTN_req_test_enh-Perf) Draft CR on Link Recovery Procedures for NTN less than 5MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NTN less than 5MHz was introduced in Rel-19, test cases shall also be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the test case verifying link recovery procedure for NTN less than 5MHz |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No link recovery procedure test case for NTN less than 5MHz |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Add A.14.4.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Start of Change 1

#### A.14.4.2.5	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode for a UE operating on a cell with less than 5 MHz BW

##### A.14.4.2.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting support-3MHz-ChannelBW-r18 properly detects SSB-based beam failure in the set q0 configured for a serving cell which is served by satellite access node (SAN) and operatw on a less than 5 MHz bandwidth, and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell which is served by satellite access node (SAN) requirements in clause 8.5C for a UE operating on a cell with less than 5 MHz BW.

Supported test configurations are specified in table A.14.4.2.5.1-1. General test parameters as specified in table A.14.4.2.2.1-2 with config 1 apply except those specified in table A.14.4.2.5.1-2. Cell specific test parameters as specified in table A.14.4.2.2.1-3 apply except those specified in table A.14.4.2.5.1-3.

The test procedure specified in clause A.14.4.2.2.1 applies to this test.

Table A.14.4.2.5.1-1: Supported test configurations for FR1 Pcell with less than 5 MHz BW

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| NOTE: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

Table A.14.4.2.5.1-2: General test parameters for FR1 PCell with less than 5 MHz BW

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| BWchannel |  | Config 1 | MHz | 3: NPRB,c = 15 |  |
| SSB Configuration |  | Config 1 |  | SSB.13 FR1 |  |
| PRACH Configuration |  | Config 1 |  | 1 |  |
| Beam failure detection transmission parameters | Number of Control OFDM symbols |  |  | 3 |  |
|  | Aggregation level |  | CCE | 8 |  |
|  | Mapping from REG to CCE |  |  | Non-Distributed |  |
| T1 |  |  | s | 1 | During this time the the UE shall be fully synchronized to Cell 1 |
| T2 |  |  | s | 5.17 |  |
| T3 |  |  | s | 3.24 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 1.97 |  |
| D1 |  |  | s | 1.94 |  |
| NOTE 1: All configurations are assigned to the UE prior to the start of time period T1. NOTE 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

Table A.14.4.2.5.1-3: Cell specific test parameters for FR1 PCell with less than 5 MHz BW

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR_SSB of set q0 | Config 1 | dB | 6 | -2 | -11 | -11 | -11 |

##### A.14.4.2.5.2	Test Requirements

Test requirements specified in clause A.14.4.2.2.1 apply to this test.

# End of Change 1