3GPP TSG-RAN WG4 Meeting #116	R4-2510380

Bengaluru, India, 25th – 29th August 2025

Source: 	Ericsson

Title:  	Discussion on RRM requirements for Ku bands for NR NTN

Agenda Item:	7.7.7

Document for:	Discussion

# Introduction

In the light of the WI for NR_NTN_Ku_bands, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [1].

This contribution outlines our point of view on on the underlying impacts to RRM requirements mandated by the issues listed in the WF.

# Discussion

Issue 1-2: Assumption for defining RRM requirements in Ku-band NTN

Candidate options: - Option 1: (LGE) - The same assumptions considered for Rel-18 Ka-band RRM requirements should apply to Ku-band RRM requirements for both FR1-NTN and FR2-NTN numerologies; - Single SAN Tx beam per radio cell in DL - For inter satellite HO, only blind HO and no neighbor cell measurement - Intra satellite neighbor cell measurement is needed - Same UE Rx beam is used for both serving and neighboring cells which belong to the same satellite (i.e., no Rx beam sweeping) - Option 2: other options are not precluded.

Since we have agreed that “Unless otherwise stated, the requirements defined for FR2-NTN apply for VSAT working in FR1-NTN”, we can align with the proposed options except for ‘Single SAN Tx beam per radio cell in DL’.

Our understanding is that it is a network implementation matter to configure either a single SAN Tx beam or multiple Tx beams per radio cell in DL. From an RRM perspective, there is no need to restrict to a single SAN Tx beam per radio cell.

Proposal 1: Not to restrict only single SAN Tx beam per radio cell, the other options remain acceptable, as listed below:

- For inter satellite HO, only blind HO and no neighbor cell measurement

- Intra satellite neighbor cell measurement is needed

- Same UE Rx beam is used for both serving and neighbor cells which belong to the same satellite (i.e., no Rx beam sweeping)

Issue 1-3: satellite switch requirements

Agreement: - Introduce RRM requirement for satellite switching with re-sync for Ku band with VSAT UE for FR1 and FR2 numerologies - Define requirement for hard satellite switch, - Tinterrupt = Tsat_beam + Tsearch + Tprocessing + T∆ + Tmargin ms - Further discuss the details on the definition of each component. - FFS whether to define requirements for soft satellite switch - FFS only switching delay requirement or both switching delay requirement and scheduling restriction requirement - Option for the delay requirement: - Tsoft_switch = max(t-service - t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing ms

For hard satellite switch, it is straightforward that a VSAT UE must steer its directional antenna toward the target satellite prior to searching for the target satellite RS. The following formula was agreed in the last meeting:

Tinterrupt = Tsat_beam + Tsearch + Tprocessing + T∆ + Tmargin ms

For soft satellite switch, in the same manner, steering the directional antenna to the target satellite and searching the target satellite RS shall be initiated during ( t-service - t-serviceStart).

Tsoft_switch = max(t-service - t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing  ms

The next consideration concerns the scheduling restriction requirement.

From our perspective, for a UE with mechanically steering antenna, it’s not expected for the UE to be scheduled during (t-service - t-serviceStart), as this would require steering back to the source satellite and steering to the target satellite again after completing the target satellite search in Tsat_beam + Tsearch .

By contrast, for a UE with an electrically steering antenna, scheduling availability is less constrained; however, introducing differentiated scheduling behavior based on antenna type could complicate the requirements.

For simplicity and robustness, we propose that the VSAT UE shall not be scheduled after initiating antenna steering toward the target satellite.

Proposal 2: Define the interruption requirement for soft satellite switch as follows

Tsoft_switch = max(t-service - t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing  ms

Proposal 3: VSAT UE isn’t scheduled by the source satellite after initiating antenna steering toward the target satellite, e.g., from start of Tsat_beam during (t-service - t-serviceStart).

Issue 1-5: inter-satellite cell reselection requirements (section 4.2C).

Candidate options: - Option 1: RAN4 to create inter-satellite cell reselection requirements for VSAT UEs with electronically steered beams in Rel-19 (Nokia) - Option 2: Do not define inter-satellite cell reselection requirements in this WI. (HW, ZTE, E///) - Option 2a: RAN4 to consider inter-satellite measurement related discussion for Ku-band NTN in a future release (e.g., Rel-20). (LGE)

From a technical perspective, it is feasible to define inter-satellite cell reselection requirements for VSAT UEs equipped with electronically steered beams without significant additional effort. That said, we do not observe any immediate need to introduce this use case at the current stage.

Proposal 4: Not to define inter-satellite cell reselection requirements in this WI.

Issue 1-12: RLM (section 8.1C)

Candidate options: - Option 1: LMax is equal to 8 for FR1-NTN numerolgoy, and 64 for FR2-NTN numerology. - Option 2: LMax is equal to 8 for both FR1-NTN numerology and FR2-NTN numerology.

For frequencies above 3GHz, Lmax is equal to 8. Regarding the frequency band scope of Ku-band as presented in WI, Lmax is equal to 8 for FR1-NTN.

Even the assumption that LMax is equal to 8 for FR1-NTN is correct, it doesn’t change the existing Table 8.1C.1-2 in TS 38.133, as duplicated in the below.

Table 8.1C.1-2: Maximum number of RLM-RS resources NRLM Carrier frequency range of PCell; Lmax; Maximum number of RLM-RS resources, NRLM. FR1, ≤ 3 GHzNote; 4; 2. FR1, > 3 GHzNote; 8; 4. FR2-NTNNote 2; 64; 8. NOTE: For unpaired spectrum operation with Case C - 30 kHz SCS, 3 GHz is replaced by 1.88 GHz, as specified in clause 4.1 in TS 38.213 [3]. NOTE 2: NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications unless otherwise stated..

Proposal 5: Confirm the assumption that LMax is equal to 8 for FR1-NTN and it doesn’t change the existing Table 8.1C.1-2 in TS 38.133.

# Conclusion

Proposal 1: Not to restrict only single SAN Tx beam per radio cell, the other options remain acceptable, as listed below:

- For inter satellite HO, only blind HO and no neighbor cell measurement

- Intra satellite neighbor cell measurement is needed

- Same UE Rx beam is used for both serving and neighbor cells which belong to the same satellite (i.e., no Rx beam sweeping)

Proposal 2: Define the interruption requirement for soft satellite switch as follows

Tsoft_switch = max(t-service - t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing  ms

Proposal 3: VSAT UE isn’t scheduled by the source satellite after initiating antenna steering toward the target satellite, e.g., from start of Tsat_beam during (t-service - t-serviceStart).

Proposal 4: Not to define inter-satellite cell reselection requirements in this WI.

Proposal 5: Confirm the assumption that LMax is equal to 8 for FR1-NTN and it doesn’t change the existing Table 8.1C.1-2 in TS 38.133.

# References

- R4-2508262, WF on RRM requirements for NR_NTN_Ku_bands, Apple