3GPP TSG-RAN WG4 Meeting #116	R4-2510386

Bengaluru, India, 25th – 29th August 2025

Source: 	Ericsson

Title:  	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Agenda Item:	7.14.3.1

Document for:	Discussion

# Introduction

In the light of the WI of NR Radio Resource Management (RRM) enhancements (WID in [1]) for Rel-19, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [2].

This contribution outlines our point of view on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor as follows.

# Discussion

## Topic #1: FR2-1 SSB based L3 measurement delay reduction for connected mode

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”:

### Issue 1-1-1: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” Agreement: For the quitting condition of FBS: - Introduce a new UAI indicating “overheating” issue: - RAN4 will not discuss the UE behaviour after reporting UAI - Introduce a timer-based solution: - UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition - Further discuss the candidate numbers for T1. Way forward: For re-evaluation mechanism for scenario 1/2/3/4: - Check whether the following mechanism is agreeable: - UE re-evaluates the activation condition after the timer expires, where the timer starts after UE quits from Multi-Rx mode.

With respect to the agreed quitting condition, the UE may exit FBS upon expiry of the FBS timer (T1). To ensure valid measurement results, T1 should at least cover the typical measurement delay.

Theoretically, T1 has a predefined fixed duration; however, applying a common T1 across different measurement configurations is challenging. That is, a more flexible duration may be preferable.

Proposal 1: RAN4 to take the following considerations into account for the design of T1:

- T1 shall be configurable.

- The maximum candidate value for T1 should be longer than the intra/inter‑frequency L3 measurement delay for all possible parameter configurations, assuming only one FR2 serving cell and more than one inter‑frequency layers to be measured.

- A candidate value for maximum candidate value of T1 is 5 minutes.

A leftover is, if the UE has exited FBS, it shall re-evaluate the FBS activation condition to determine whether FBS should be re-enabled.

- On one hand, the UE should not switch between FBS and non-FBS too frequently (from FBS to non-FBS and from non-FBS to FBS), as this may complicate network handling of measurement reporting.

- On the other hand, re-entering FBS too late may reduce the potential benefits of FBS.

Taking the above concerns into account, we suggest that, after exiting from FBS, the UE shall not immediately re-evaluate the FBS activation condition. Instead, the re-evaluation should resume after a brief delay, but not excessively late. To support this, two parameters are introduced below and illustrated in Figure 1:

- ‘Time-to-activate’ period, from the time when the UE exits from FBS.

- Timer (T2), starts after a defined ‘time-to-activate’ period.

Figure 1 Timeline of FBS re-evaluation

Proposal 2: After deactivating FBS,

- The UE shall not re-evaluate the FBS activation condition during a subsequent ‘time to activate’ period.

- After the ‘time to activate’ period, the UE shall start a timer (T2) and initiate re-evaluation of the condition for enabling FBS not later than the expiry of T2.

Issue 1-1-3: UE capability for Rx BSF optimization

Agreement: For FBS capability design: - UE capability is per-band and optional - TDD only - FR2-1 only - FFS: in the same FG, whether to define separate UE capability for (intra-frequency) and (intra-frequency + inter-frequency)

Referring to the online discussions in the last meeting, the FFS noted in the issue is related to concerns regarding UE capability.

Proposal 3: We are neutral regarding the definition of separate capabilities for:

- Intra‑frequency only, and

- Intra‑frequency + inter‑frequency.

# Conclusion

Proposal 1: RAN4 to take the following considerations into account for the design of T1:

- T1 shall be configurable.

- The maximum candidate value for T1 should be longer than the intra/inter‑frequency L3 measurement delay for all possible parameter configurations, assuming only one FR2 serving cell and more than one inter‑frequency layers to be measured.

- A candidate value for maximum candidate value of T1 is 5 minutes.

Proposal 2: After deactivating FBS,

- The UE shall not re-evaluate the FBS activation condition during a subsequent ‘time to activate’ period.

- After the ‘time to activate’ period, the UE shall start a timer (T2) and initiate re-evaluation of the condition for enabling FBS not later than the expiry of T2.

Proposal 3: We are neutral regarding the definition of separate capabilities for:

- Intra‑frequency only, and

- Intra‑frequency + inter‑frequency.

# References

- RP-240830, New WID: WI resulting from discussion on RRM enhancements, vivo (Moderator)

- R4-2508266, WF for NR_RRM_Ph5_part2, CATT