3GPP TSG-RAN WG4 Meeting #116				                R4-2510390

Bengaluru, India, August 25th – 29th, 2025

Source: 	China Telecom

Title: 	Views on Power domain enhancements for single carrier

Agenda Item:	7.1.2.2.1

Document for:	Discussion

Introduction

In last meeting[1], we reached an initial agreement on MPR reduction for single carrier. For derivation of new inner RB region, the views among companies are convergent and the starting point is acceptable. For signaling and UE capability, we still need a clear description to inform RAN2 to design the RRC signaling.

So in this contribution we focus on approaches to enable MPR reduction and signalling, to provide our views.

Discussion

Approaches to enable MPR reduction

For extension ratios, as OPPO proved in their last paper [2], when only extending X*NRB at the one side of the UE CBW, ratio 1/3 can achieve the same inner RB region compared to ratio 1/2. For scenarios where spectrum resources are limited, 1/3 extension can be an effective solution. So when UE support X*NRB extension, i.e. capability 2, we should add 1/3 as an optional value.

Proposal1: Introduce extension ratio 1/3 to capability 2, and only applied when extending at one side of the UE CBW.

To support different scenarios about UE CBW and BS CBW, asymmetric cases for dual-sided case when non-zero extension is applied for both sides should be adopted. Overall, the optional extension cases are follows: {1/2, 0}, {0, 1/2}, {1/2, 1/2} for capability 1 and {1/3, 0}, {0, 1/3}, {1/2, 0}, {0, 1/2}, {1/2, 1/2}, {1/4, 0}, {0, 1/4}, {1/4, 1/4}, {1/2, 1/4}, {1/4, 1/2} for capability 2.

Proposal2: Asymmetric cases for dual-sided case when non-zero extension is applied for both sides should be adopted; the optional extension cases are follows: {1/2, 0}, {0, 1/2}, {1/2, 1/2} for capability 1 and {1/3, 0}, {0, 1/3}, {1/2, 0}, {0, 1/2}, {1/2, 1/2}, {1/4, 0}, {0, 1/4}, {1/4, 1/4}, {1/2, 1/4}, {1/4, 1/2} for capability 2.

For derivation of new inner RB region in extended CBW, we think the starting point in WF [1] can cover all symmetric and asymmetric cases. But a typo in the equation should be corrected: in definition of “RBStart,High_new”, the “RBStart_Low_new” should be corrected to “RBStart_Low”, i.e. “max(1, floor(LCRB/2))”. So the equation of getting new inner region can be listed as follows:

- RBStart,Low_new = max(0, RBStart,Low – RBExtension_low);

- RBStart,High_new = min(NRB – LCRB, NRB + RBExtension_high – RBStart,Low – LCRB);

- RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBExtension_low + RBExtension_high)/2);

Proposal3: The equation of getting new inner region can be listed as follows:

- RBStart,Low_new = max(0, RBStart,Low – RBExtension_low);

- RBStart,High_new = min(NRB – LCRB, NRB + RBExtension_high – RBStart,Low – LCRB);

- RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBExtension_low + RBExtension_high)/2);

The corresponding changes to TS 38.101-1 is in our dCR [3].

Signaling and UE capability

It’s time for us to send LS to RAN2. Generally, the granularity and content of signaling and UE capability are basically clear, but we still need convergence the details to inform RAN2 for signalling design.

For UE capability, we should clarify the extension ratio means RB extension and perform on NRB.

So description of UE capability should be as follows:

[MPRReduction-RBExtension-r19]: Indicate whether UE support MPR reduction for single UL carrier with UE CBW(NRB) extension. [value1] indicate whether UE supports MPR reduction for single UL carrier with 1/2*UE NRB extension only, the supported combinations are {1/2, 0}, {0, 1/2}, {1/2, 1/2}; [value2] indicate whether UE supports MPR reduction for single UL carrier with 1/2 and 1/3 and 1/4*UE NRB extension, the supported combinations are {1/3, 0}, {0, 1/3}, {1/2, 0}, {0, 1/2}, {1/2, 1/2}, {1/4, 0}, {0, 1/4}, {1/4, 1/4}, {1/2, 1/4}, {1/4, 1/2}. The first value of combinations means extension ratio at lower side of UE NRB, the second value means extension ratio at higher side of UE NRB.

Proposal4: Description of UE capability should be as follows:

[MPRReduction-RBExtension-r19]: Indicate whether UE support MPR reduction for single UL carrier with UE CBW(NRB) extension. [value1] indicate whether UE supports MPR reduction for single UL carrier with 1/2*UE NRB extension only, the supported combinations are {1/2, 0}, {0, 1/2}, {1/2, 1/2}; [value2] indicate whether UE supports MPR reduction for single UL carrier with 1/2 and 1/3 and 1/4*UE NRB extension, the supported combinations are {1/3, 0}, {0, 1/3}, {1/2, 0}, {0, 1/2}, {1/2, 1/2}, {1/4, 0}, {0, 1/4}, {1/4, 1/4}, {1/2, 1/4}, {1/4, 1/2}. The first value of combinations means extension ratio at lower side of UE NRB, the second value means extension ratio at higher side of UE NRB.

For signalling of NW, we need two IEs and each IE at least 2bits to convey 0, 1/2, 1/3 and 1/4. The NW will configurate a proper ratio after considering multiple factors and not reflected in the specification.

Proposal5: For signalling of NW, we need two IEs and each IE at least 2bits.

The description of NW signalling should be:

[RB-extension-lowerSide/higherSide]: Indicate the extended ratio at lower/higher side of UE NRB for MPR reduction. If UE support [MPRReduction-RBExtension-r19-value1], this field can indicate 0 or 1/2; If UE support [MPRReduction-RBExtension-r19-value2], this field can indicate 0 or 1/4 or 1/3 or 1/2.

Proposal6: The description of NW signalling should be:

[RB-extension-lowerSide/higherSide]: Indicate the extended ratio at lower/higher side of UE NRB for MPR reduction. If UE support [MPRReduction-RBExtension-r19-value1], this field can indicate 0 or 1/2; If UE support [MPRReduction-RBExtension-r19-value2], this field can indicate 0 or 1/4 or 1/3 or 1/2.

Conclusion

Proposal1: Introduce extension ratio 1/3 to capability 2, and only applied when extending at one side of the UE CBW.

Proposal2: Asymmetric cases for dual-sided case when non-zero extension is applied for both sides should be adopted; the optional extension cases are follows: {1/2, 0}, {0, 1/2}, {1/2, 1/2} for capability 1 and {1/3, 0}, {0, 1/3}, {1/2, 0}, {0, 1/2}, {1/2, 1/2}, {1/4, 0}, {0, 1/4}, {1/4, 1/4}, {1/2, 1/4}, {1/4, 1/2} for capability 2.

Proposal3: The equation of getting new inner region can be listed as follows:

- RBStart,Low_new = max(0, RBStart,Low – RBExtension_low);

- RBStart,High_new = min(NRB – LCRB, NRB + RBExtension_high – RBStart,Low – LCRB);

- RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBExtension_low + RBExtension_high)/2);

Proposal4: Description of UE capability should be as follows:

[MPRReduction-RBExtension-r19]: Indicate whether UE support MPR reduction for single UL carrier with UE CBW(NRB) extension. [value1] indicate whether UE supports MPR reduction for single UL carrier with 1/2*UE NRB extension only, the supported combinations are {1/2, 0}, {0, 1/2}, {1/2, 1/2}; [value2] indicate whether UE supports MPR reduction for single UL carrier with 1/2 and 1/3 and 1/4*UE NRB extension, the supported combinations are {1/3, 0}, {0, 1/3}, {1/2, 0}, {0, 1/2}, {1/2, 1/2}, {1/4, 0}, {0, 1/4}, {1/4, 1/4}, {1/2, 1/4}, {1/4, 1/2}. The first value of combinations means extension ratio at lower side of UE NRB, the second value means extension ratio at higher side of UE NRB.

Proposal5: For signalling of NW, we need two IEs and each IE at least 2bits.

Proposal6: The description of NW signalling should be:

[RB-extension-lowerSide/higherSide]: Indicate the extended ratio at lower/higher side of UE NRB for MPR reduction. If UE support [MPRReduction-RBExtension-r19-value1], this field can indicate 0 or 1/2; If UE support [MPRReduction-RBExtension-r19-value2], this field can indicate 0 or 1/4 or 1/3 or 1/2.

References

- R4-2507930 WF on NR_ENDC_RF_Ph4_part1, Moderator (Huawei, HiSilicon), RAN4#115

- R4-2507319 Discussion on MPR reduction for single carrier, OPPO, RAN4#115

- R4-2510391 draftCR on approaches to enable MPR reduction for single CC in Rel-19, China Telecom, RAN4#116