3GPP TSG-RAN WG4 Meeting #116	R4-2510392

Bangalore, India, 25th – 29th August 2025

Source:	Ericsson

Title:	Discussion on Rel-19 MMSE-IRC BS demodulation requirements

Agenda item:	7.13.4

Document for:	Discussion

# Introduction

In previous RAN4#115 meeting, companies agreed on number of DM-RS CDM group without data for Rel-19 NR MMSE-IRC BS requirement [1]. However, the open issue on applicability rules and MMSE-IRC receiver assumption still exists and is discussed in this contribution.

# Discussion

## 2.1	Applicability Rules

Open Issue:

Applicability for different BS classes and test scenarios

- FFS whether to consider additional applicability rule on how the test is performed:

- Option 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped

- Option 2: Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

Agreements:

Online agreement:

- Follow the LTE approach – Only specifies that HomNet requirements are not applicable for Local Area BS.

In the last meeting, companies wanted to follow the approach for applicability rule as done in LTE TS 36.141, where only homogenous network requirements are specified, which are not applicable for Local Area BS. But since heterogenous network requirements are not mentioned, this implies that for Wide and Medium Area BS, both homogenous and heterogenous requirements are applicable. Therefore, Wide and Medium Range BS would need to pass the test for both requirements, which would eventually increase the test time. Hence an additional applicability rule is needed.

Observation 1: Without additional applicability rule, Wide and Medium Area BS needs to pass the test for both homogenous (INR set 1) and heterogenous (INR set 2) network requirements, thereby increasing the test time.

Observation 2: It would be feasible to test Wide and Medium Area BS with more stringent INR set only.

Proposal 1: We prefer to consider additional applicability rules to reduce the test effort.

Open Issue:

MMSE-IRC receiver assumption - estimation of interference covariance matrix

- RAN4 agree that MMSE-IRC receiver assumption is not going to be captured in the RAN4 spec

- There is no consensus whether to modify the previously agreed MMSE-IRC receiver assumption.

2.2	MMSE-IRC Receiver Assumption

As discussed in the last meeting, the concern arises from the typical Rel-15 setup, where 2 CDM groups are assumed, along with a 3dB power boosting on DM-RS is applied to both serving and interfering cells. As mentioned by companies, this results in a power mismatch between DM-RS REs and data REs, potentially causing a misalignment in the interference covariance matrix estimation, which is typically based on DM-RS REs. Therefore, interference covariance matrix estimation could also include non-DMRS RE.

Observation 3: Both DMRS and non-DMRS REs can be used to implement interference covariance matrix estimation, where the noise power is averaged over the PRB.

Further, in real deployment each BS manages its own transmission independently and thus has no information if the interfering cell has enabled or disabled DM-RS power boosting.

Observation 4: The serving BS has no information on DM-RS power boosting from the interfering cell.

In the typical test setup, to obtain the MMSE-IRC receiver weight matrix, the covariance matrix including the sources of inter-cell interference needs to be estimated. Currently, the covariance matrix is estimated at DM-RS REs like in LTE. But for NR, since we use both DMRS + non-DMRS REs, we can consider modifying the receiver assumption.

Observation 5: It may be acceptable to modify the MMSE-IRC receiver’s assumption based on the following:

1) Since the specification does not capture receiver assumptions, companies can still deliver simulation results based on either the baseline receiver or the new proposed receiver assumption. This flexibility allows the receiver assumption to remain within the BS implementation scope.

2) While we previously agreed on the baseline receiver assumption in R4-2413570, this does not preclude updates if justified.

3) The simulation results are already aligned and show that it is sufficient to test the IRC algorithm, which can be done under an updated receiver assumption if needed.

Proposal 2: We don’t have a strong opinion; however, it may be ok to modify the current BS MMSE-IRC receiver assumptions.

# Conclusion

Based on the discussion, we made the following observations:

Observation 1: Without additional applicability rule, Wide and Medium Area BS needs to pass the test for both homogenous (INR set 1) and heterogenous (INR set 2) network requirements, thereby increasing the test time.

Observation 2: It would be feasible to test Wide and Medium Area BS with more stringent INR set only.

Observation 3: Both DMRS and non-DMRS REs can be used to implement interference covariance matrix estimation, where the noise power is averaged over the PRB.

Observation 4: The serving BS has no information on DM-RS power boosting from the interfering cell.

Observation 5: It may be acceptable to modify the MMSE-IRC receiver’s assumption based on the following:

1) Since the specification does not capture receiver assumptions, companies can still deliver simulation results based on either the baseline receiver or the new proposed receiver assumption. This flexibility allows the receiver assumption to remain within the BS implementation scope.

2) While we previously agreed on the baseline receiver assumption in R4-2413570, this does not preclude updates if justified.

3) The simulation results are already aligned and show that it is sufficient to test the IRC algorithm, which can be done under an updated receiver assumption if needed.

Based on the discussion, we made the following proposals:

Proposal 1: Proposal 1: We prefer to consider additional applicability rules to reduce the test effort.

Proposal 2: We don’t have a strong opinion; however, it may be ok to modify the current BS MMSE-IRC receiver assumptions.

# References

[1] R4-2508618, “Way Forward for [115][322] NR_demod_Ph5_Part1_General_BS”, China Telecom, RAN4#115, Malta.