3GPP TSG-RAN WG4 Meeting #116	R4-2510394

Bangalore, India, 25th – 29th August 2025

Source:	Ericsson

Title:	Discussion on Rel-19 NR MIMO Phase 5 demodulation requirements

Agenda item:	7.20.6

Document for:	Discussion

# Introduction

Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

In RAN#105, a revised WID [1] on NR MIMO Phase 5 was approved for Release 19. The WID covers five main objectives, where one of the objectives is to specify 3-antenna-port codebook-based transmissions. The enhancements for uplink performance are copied below from the WID.

In last meeting, we kicked-off the demodulation discussions. In this paper, we present our general view on BS demodulation requirements 3-port PUSCH supporting up to 3 layers, along with UE demodulation requirements.

# BS Demodulation requirements

NR currently supports PUSCH configurations with 1, 2, 4, and 8 ports. However, most commercial mobile UEs today are equipped with only 1 or 2 Tx antennas. Looking ahead, as hardware and design technologies continue to evolve, the introduction of UEs with 3TX antennas is a plausible next step. To support this anticipated development and improve uplink performance, it is beneficial to extend the NR specifications to include support for 3-port PUSCH with up to 3 transmission layers.

2.1 Scenario and channel model

Based on the description in WI [1], it seems still in similar scope as legacy TN scenario. In that case, same network deployment would be applied.

Observation 1: No new deployment scenario is introduced for 3Tx MIMO transmission.

In general, achieving higher layer transmission requires favorable channel conditions, typically characterized by high SNR. Such conditions are more likely in line-of-sight (LOS) scenarios with relatively low UE mobility. However, from a performance evaluation perspective, it is more insightful to consider worst-case scenarios—such as non-line-of-sight (NLOS) channels and higher Doppler shifts—if such conditions are feasible to test. Since no new scenarios are being introduced, the same channel models used in traditional BS demodulation—such as TDLA30-10, TDLB100-400, and TDLC300-100—can be reused for initial 3Tx simulations. In existing BS demodulation requirements, different channel models are used to evaluate performance across various coverage conditions. A similar approach can be adopted here as well. To reduce complexity, companies may optionally limit the number of test cases, for instance, by specifying requirements for only one or two channel models

Proposal 1: Re-use the legacy channel models, e.g., TDLA30-10/TDLB100-400/TDLC300-100, for initial 3Tx MIMO demodulation simulations.

2.2 Requirement Scope

The TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource selected by the SRI when multiple SRS resources are configured, or if a single SRS resource is configured TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource.

In the current test parameters for normal PUSCH demodulation requirements, TPMI is fixed to 0, meaning only a single-layer or fixed precoding is tested, and no selection among multiple precoding matrices is done. However, this WI focus on non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions. This expands the test scope by requiring multiple TPMI indexes. As discussed in clause 6.1.1.1 of TS 38.214,

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for single-layer transmission.

13100,13010,13001

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for two-layer transmission.

13100100, 13100001, 13 001001

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for three-layer transmission.

13100010001

As per the RAN1 agreements [2] in RAN1#116 meeting,

Observation 2:  For non-coherent uplink precoding by a 3TX UE, new precoders are agreed in RAN1 and need to be considered by RAN4 to define BS demodulation requirements.

Since new precoding matrix for single, two and three-layer transmission needs to be considered, the corresponding TPMI index can be used to check for the PUSCH performance requirements. For BS Demodulation requirement, typically TPMI index 0 is used to test maximum layers.

Proposal 2: Consider new precoding matrix for non-coherent uplink precoding by 3Tx UE to define 3Tx MIMO BS demodulation requirements. Correspondingly, the TPMI index 0 should be used for three-layer transmission.

2.3 Requirement Impact

Enabling 3Tx MIMO primarily affects PUSCH performance as specified in [1]. In the legacy requirements outlined in [3], 2Tx and 4Tx configurations are addressed only within the normal PUSCH demodulation requirements. A similar approach can be adopted for introducing 3Tx demodulation requirements.

Proposal 3: Only new normal PUSCH demodulation requirements are considered for 3Tx MIMO.

Some analysis for initial simulations is as follows:

Duplex Mode: Following the legacy requirements, both FDD and TDD operation should be considered, and the same requirements are applicable for both duplex modes.

Channel Bandwidth and Subcarrier Spacing (SCS): For the initial simulations, the minimum channel bandwidth for each SCS may be considered, as per the applicability rule. If significant performance differences are observed, it is advisable to also include typical or even the maximum channel bandwidth to ensure adequate test coverage. The following configurations are suggested:

- 15 kHz SCS with 5 MHz channel bandwidth

- 30 kHz SCS with 10 MHz channel bandwidth

Modulations: To maintain consistency with 2 and 4Tx requirements, QPSK, 16QAM, and 64QAM can be used to ensure comparable test coverage. Legacy MCS indices can be applied for initial simulations and adjusted later based on simulation outcomes. The following candidate configurations are proposed:

  - TDLA30-10 (low delay spread and low Doppler shift): High-order modulation (up to 64QAM) and higher coding rate (e.g., R = 567/1024) are typical. Therefore, MCS 20 is suggested.

  - TDLB100-400 (medium delay spread and high Doppler shift): A more robust setting using MCS 2 with QPSK modulation is appropriate.

  - TDLC300-100 (high delay spread and medium Doppler shift): MCS 16, using 16QAM modulation, is recommended.

Additionally, higher MCS indices per modulation level may also be considered in initial simulations, especially given the high-throughput expectations for the 3Tx scenarios.

Number of layers: As per existing 4Tx requirements, where only 4-layers are considered for the requirements, similarly 3-layers transmission can be considered for 3Tx requirements. However, less-layers are supported but it can be up to BS implementation to schedule it.

Number of Receiver Antennas: For 1Tx and 2Tx configurations, 2, 4, and 8 Rx antennas are typically considered in the requirements. However, since this work item focuses specifically on evaluating 3 uplink layers, the following antenna configurations are proposed for initial simulations: 3Tx 4Rx and 3Tx 8Rx. Since we do not have any other test requirement with 3 and 6 Rx, we can test 3-layer transmission with 4 and 8 Rx antenna. This would also mean that there will be no test for a 2Rx BS with a 3Tx UE.

Waveform: only CP-OFDM is suggested for initial evaluation.

DM-RS configuration: 1+1 DMRS configuration can be considered for normal PUSCH requirement.

DM-RS port: Following Rel-15 DM-RS configuration type 1, the DM-RS ports can be configured as {0,1,2}.

PUSCH Mapping Type: Although both mapping type A and B are considered for 1Tx, 2Tx and 4Tx requirement, we prefer to consider mapping type A to reduce the test effort, if there is no significant performance difference between mapping type A and B.

Test metric: 70% normalized throughput.

Proposal 4: It can be feasible to adopt the parameter set in Table 2.3-1 for initial simulations.

- If significant performance differences are observed between channel bandwidths for each SCS, typical bandwidth may be considered in the requirements to ensure comprehensive test coverage.

- Companies are also encouraged to assess the feasibility of using higher MCS values for each modulation level.

Table 2.3-1 Initial simulation parameters for FDD and TDD 3Tx MIMO PUSCH (3 layers) demodulation

| nTx | nRx | Mapping type | precoding | DM-RS | DM-RS ports | Channel model | MCS | SCS [kHz] | CBW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 4 / 8 | A | Enable | 1+1 | {0,1,2} | TDLB100-400 | 2 | 15 | 5 |
|  |  |  |  |  |  | TDLC300-100 | 16 |  |  |
|  |  |  |  |  |  | TDLA30-10 | 20 |  |  |
|  |  |  |  |  |  | TDLB100-400 | 2 | 30 | 10 |
|  |  |  |  |  |  | TDLC300-100 | 16 |  |  |
|  |  |  |  |  |  | TDLA30-10 | 20 |  |  |

2.4 Specification Impact

Since 3Tx MIMO is a new feature with non-coherent UL codebook, the test parameter and requirements can be captured along with 1, 2 and 4Tx requirements in the same table of TS 38.104. Further separate notes can be added to the table, if needed for codebook selection. However, this can be further discussed among companies.

Observation 3: It may be feasible to add 3TX requirements in the same table as existing 1,2 and 4Tx requirements in TS 38.104 “Sec. 8.2: Performance requirements for PUSCH”.

# UE Demodulation requirements

3.1	CSI reporting requirements

3.1.1	Refined Type I Single-Panel Codebook (typeI-SinglePanel-r19)

RAN4#115 agreed to define PMI reporting requirements with the refined Type I Single-Panel codebook, but RAN4 has not concluded the detailed simulation parameters. Table 3.1-1 is the summary of candidate CSI configuration parameter options for eTypeI reporting requirements.

Table 3.1-1 Proposed PMI reporting requirements with Rel-19 refined Type-I single panel codebook.

| Test case | CSI-RS ports (N1, N2) | Port mapping method (portMappingMethod) | Codebook mode (typeI-codebookMode-r19) | CMR configuration (K and nrofPorts per CSI-RS resource) | Number of slots for CSI-RS transmission |
| --- | --- | --- | --- | --- | --- |
| 64 ports | 64 Option 1: (8, 4) Option 2: (16, 2) | FFS | Mode A | K=2 32 CSI-RS ports | One slot |
| FFS: 128 ports | 128 (16, 4) | FFS | FFS | K=4 32 CSI-RS ports | FFS |

64 CSI-RS ports

For 64 CSI-RS ports scenario, one of the open issues is the antenna configuration. Figure 3.1-1 shows our simulation results for 64 CSI-RS ports with two antenna configurations 8x4 and 16x2. Although we don’t see significant performance differences between the two configurations, we prefer to use 8x4.

Figure 3.1-1. PDSCH throughputs for 64 CSI-RS ports.

Observation 4: No significant performance difference between antenna configuration options 8x4 and 16x2.

Proposal 5: For eTypeI PMI reporting requirements with 64 ports, configure (N1, N2, O1, O2) = (8, 4, 4, 4).

Another open issue is the port mapping method (portMappingMethod), where RAN1 specifies ‘method1’ and ‘method2’. If UE supports 'typeI-SinglePanel-r19’, the UE needs support both the mapping method, and it is configured by the network. Since this is how to map the CSI-RS resource to CSI-RS port, we don’t expect significant performance difference. We therefore propose to set ‘method1’ for 64 ports scenario.

Proposal 6: For eTypeI PMI reporting requirements with 64 ports, set the port mapping method (portMappingMethod) to ‘method1’.

128 CSI-RS ports

In RAN4#115, companies proposed to define eTypeI PMI reporting requirements with 128 ports, but RAN4 has not decided whether the define or not. Although it is the optional feature for UE supporting 'typeI-SinglePanel-r19’, we still think it is beneficial to define the requirements for the largest CSI-RS ports configuration.

Proposal 7: Define eTypeI PMI reporting requirements with 128 CSI-RS ports.

RAN1 specifies two codebook mode ‘Mode A’ and ‘Mode B’. Since we are going to use FRC with rank 2, we don’t see significant differences between Mode A and B for lower rank. Therefore, we propose configuring ‘Mode A’ for codebook mode. On the port mapping method, if we configure ‘method1’ for 64 ports, we propose configuring ‘method2’ for 128 ports scenario for test coverage. Finally for the number of slots for CSI-RS transmission, since RAN4 PMI reporting test does not transmit PDSCH in the slots where CSI-RS for CSI acquisition is transmitted, we prefer to schedule all the CSI-RS resources in one slot to avoid lowering the maximum throughput.

Proposal 8: For eTypeI PMI reporting requirements with 128 ports, configure ‘Mode A’ for Codebook mode (typeI-codebookMode-r19).

Proposal 9: For eTypeI PMI reporting requirements with 128 ports, configure ‘method2’ for the port mapping method (portMappingMethod).

Proposal 10: For eTypeI PMI reporting requirements with 128 ports, schedule all the CSI-RS resources for CSI acquisition in one slot.

Other parameters

RAN4 has not concluded the CSI reporting delay for eTypeI reporting requirements.

- Option 1: RAN4 set the CQI/RI/PMI delay according to the number of CSI-RS ports and CSI processing capabilities. - Option 2: Re-discuss the CSI-RS scheduling and CSI reporting pattern to match the capability 2 of timeline to support up to 128 CSI-RS ports

RAN1#121 updated the UE feature list [6] and, specified two UE capabilities for processing time as follows:

- Capability 1: Reuse legacy Z/Z’ values, OCPU = ceil(P/32)

- Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources, OCPU = ceil(P/32)

This means the reporting delay is the same as 32 ports for Capability 1, but the reporting delay becomes 2 times longer for 64 ports, and 4 times longer for 128 ports for Capability 2. Considering the minimum performance requirements, we think RAN4 need to assume the reporting delay based on Capability 2.

Observation 5:	There are two CSI reporting delays according to UE capability.

RAN4 has used TDLA30 with 5Hz Doppler spread for PMI reporting requirements. Theoretically the channel coherent time for 5Hz is 200 ms (= 1/fd = 1/5). For 32 CSI-RS ports PMI reporting test, the scheduling delays are 9 ms and 7ms for FDD 15kHz and TDD 30kHz, respectively. Even if the scheduling delays quadruple, the channel coherency can be kept.

Figure 3.1-1 also compares two CSI reporting delay options: one for 9 slots assuming Capability 1, and 14 slots assuming Capability 2. For Capability 2, when the UE reports PMI in slot #n, we assume that UE estimates PMI in slot (n-7) and the reported PMI is applied to PDSCH transmission in slot (n+7). In our simulation results we don’t see significant performance loss due to the longer CSI delay assuming Capability 2.

We have also investigated the performance impact for 128 CSI-RS ports, as shown in Figure 3.1-2. For Capability 2, we assume when the UE reports PMI in slot #n, UE estimates PMI in slot (n-13) and the reported PMI is applied to PDSCH in slot (n+13). Unlike 64 CSI-RS ports, we see the significant performance loss due to the CSI delay assuming Capability 2, about 2dB at SNR corresponding to 90% of the peak rate. However, if we consider the test metric based on throughput ratio between follow PMI and random PMI, the throughput ratio is still around 1.3 at SNR corresponding to 90% of the peak rate with follow PMI. Please note that the throughput ratio is around 1.6 for CSI delay with Capability 1. We think it is still possible to specify the 128 CSI-RS ports PMI reporting requirements with CSI reporting Capability 2.

Figure 3.1-2. PDSCH throughputs for 128 CSI-RS ports.

Observation 6: For eTypeI PMI reporting requirements with 64 ports, the performance loss due to the CSI reporting delay Capability 2 is not significant compared to Capability 1.

Observation 7: For eTypeI PMI reporting requirements with 128 ports, the performance loss due to the CSI reporting delay Capability 2 is about 2dB compared to Capability 1, however it is still observed the gain compared with random PMI.

Proposal 11: For eTypeI PMI reporting requirements, set CSI feedback delay based on Capability 2.

Proposal 12: RAN4 uses TDLA30-5 for eTypeI PMI reporting requirements.

3.1.2	Refined eType II Codebook (eTypeII-r19)

- Option 1: RAN4 introduce PMI reporting requirements for eTypeII-r19 - Option 2: RAN4 do not introduce PMI requirements for eTypeII-r19

RAN4 has not concluded whether to define the PMI reporting requirements Rel-19 eTypeII. Since RAN4 has defined PMI reporting requirements with Rel-15 TypeII and Rel-16 eTypeII, we also prefer to define the PMI reporting requirements for Rel-19 enhancements. Considering minimizing the number of test cases, we propose to define the PMI reporting requirements with 64 CSI-RS ports because it is the mandatory feature if UE supports eTypeII-r19, according to [6].

Proposal 13: Define eTypeII-r19 PMI reporting requirements with 64 CSI-RS ports, where (N1, N2) = (8, 4).

Proposal 14: For eTypeII-r19 PMI reporting requirements with 64 ports, set the test metric as the throughput ratio between the following PMI and random eTypeI-r19 PMI Mode A with 64 ports.

Proposal 15: For eTypeII-r19 PMI reporting requirements with 64 ports, consider using the test parameters of the existing Rel-16 eType-II PMI reporting requirements with 16 CSI-RS ports for the initial evaluation, e.g., TDLA30-5, MCS20 with rank 2, Codebook Configuration.

Like eTypeI-19 PMI reporting, RAN1 specified two UE capabilities for CSI reporting delay; capability 1 keeps the same CSI reporting delay as Rel-15/16 (e)TypeII reporting delay and capability 2 allows longer delay according to the number of CSI-RS ports. As we discussed in eTypeI-r19 reporting requirements, considering the minimum requirements, we propose RAN4 assume capability 2 for the simulation assumption.

Proposal 16: For eTypeII-r19 PMI reporting requirements, set CSI feedback delay based on Capability 2.

We also propose to define the PMI reporting requirements for eTypeII-r19 for 2Rx/4Rx UEs with FR1 FDD SCS=15kHz and FR1 TDD SCS=30kHz as same as the eTypeI-r19 PMI reporting requirements.

Proposal 17: Define eTypeII-r19 PMI reporting requirements for 2Rx/4Rx UEs with FR1 FDD SCS=15kHz and FR1 TDD SCS=30kHz.

3.1.3	Refined eType II Codebook for predicted PMI (typeII-Doppler-r19)

- Option 1: RAN4 introduce PMI reporting requirements for typeII-Doppler-r19 - Option 2: RAN4 do not introduce PMI requirements for typeII-Doppler-r19

RAN4 has not concluded whether to define the PMI reporting requirements ‘typeII-Doppler-r19’. In our understanding, the new codebook ‘typeII-Doppler-r19’ is a simple extension of 'typeII-Doppler-r18' to support more than 32 ports. If RAN4 define PMI reporting requirements with eTypeII-r19 with 64 ports, we may not need to define typeII-Doppler-r19 additionally.

Proposal 18: RAN4 does not define eTypeII-r19 PMI reporting requirements.

3.1.4	CSI reporting requirements for CJT calibration

- Option 1: RAN4 do not introduce CSI reporting requirements for UE reporting enhancements for CJT - Option 2: RAN4 introduce new requirements for CJT calibration offset reporting for the time offset (TO), the carrier frequency offset (CFO) and the phase offset (PO) scenarios. Further study exact implementation of the new requirements. - RAN4 do not introduce new CQI/RI and dedicated PMI reporting requirements but consider PMI calculation as part of the CJT calibration reporting requirements. - Introduce a new subsection in TS 138 101-4 to cover introduction of CJT calibration offset reporting requirements.

In RAN4#115, RRM session has agreed to define CJT calibration reporting delay and accuracy requirements, where RRM session are going to consider delay offset and frequency offset reporting, but they agreed to deprioritize phase offset in Rel-19. We think it is sufficient to verify the UE performance on CJT calibration reporting, and therefore RAN4 does not need to define new CSI reporting requirements for CJT calibration.

Proposal 19: RAN4 does not define CSI reporting requirements for CJT calibration.

3.2	PDSCH demodulation requirements

3.2.1	PDSCH demodulation requirements for CJT

- Option 1: RAN4 do not introduce demodulation requirements for reporting enhancements for CJT deployments - Option 2: RAN4 further discussion on whether to introduce new PDSCH demodulation performance requirements for PDSCH transmission Scheme C/Scheme D/Scheme E.

RAN1 introduced three new CJT schemes in TS 38.214 5.1.5 to configure the UE for two TCI relations as follows:

- CJT Scheme C: UE should assume Doppler shift is different between two TCIs (TRPs).

- CJT Scheme D: UE should assume Propagation delay is different between two TCIs (TRPs).

- CJT Scheme E: UE should assume Doppler shift and propagation delay are different between two TCIs (TRPs).

According to the RAN1 discussion, the purpose of these three schemes is to inform UE that Doppler shift and/or propagation delay between two TRPs may be different. Unlike the CJT calibration reporting, TS 38.214 does not specify any UE behavior when the UE is configured Scheme C/D/E. We should also emphasize that RAN4 did not define new PDSCH demodulation requirements when RAN1 introduced CJT schemes A and B in Rel-18.

Proposal 20: RAN4 does not define PDSCH demodulation requirements for ctjSchemeC, ctjSchemeD, and ctjSchemeE.

3.2.2	PDSCH demodulation requirements for SRS port grouping

RAN4 also discussed whether to define new PDSCH demodulation requirements for SRS port grouping feature.

- Option 1: RAN4 introduce PDSCH demodulation requirements for SRS port grouping - Option 1A: RAN4 to discuss reuse of existing demod requirements for SRS port grouping scheme. - Option 1B: RAN4 to discuss the new performance requirements for R19 8Rx UE supporting SRS port grouping, FFS the test setup. - Option 2: RAN4 do not introduce UE demodulation/CSI reporting requirements for SRS port grouping

According to TS 38.214 V19.0.0 5.2.2.5.1, for the SRS port grouping feature, UE can assume that SRS port group 0 corresponds to codeword 0, and that SRS port group 1 corresponds to codeword 1, when the gNB configures SRSPortGrouping-r19. This means the antenna connector(s) transmitting SRS port group 0 are used to receive the Rx signals for the receive antenna port group (RAPG) 0, and the antenna connector(s) transmitting SRS group 1 are used to receive the Rx signals for RAPG 1. As it is described in WID [1], this feature targets the low-complexity 6Rx and 8Rx UEs, which corresponds to ‘Simplified SU-MIMO 8Rx Receiver’ RAN4 specified in Rel-18.

In our understanding, this feature assumes reciprocity-based downlink transmission. However, RAN4 has not specified the reciprocity-based DL performance requirements. Moreover, when we look RAN5 for UE DL demodulation test conformance specification, the UL signal cable is directly connected to NR system simulator (SS) Rx port without fader or noise source (See appendix A.3.1.7 in TS38.508). This means the SS cannot build the beamforming weight for each RAPG according to SRS port group. If the SS cannot use the reciprocity-based channel information, this is same test setup as Rel-18 Simplified SU-MIMO 8Rx receiver requirements. We are wondering about the testability of the PDSCH demodulation requirements based on SRS port grouping.

Observation 8: SRS port grouping feature assumes the reciprocity-based downlink transmission.

Observation 9: RAN4 UE demodulation and CSI reporting performance requirements have not been considered specifying the requirements using the reciprocity-based channel information.

Observation 10: RAN5 UE demodulation and CSI reporting performance test setup so far uses direct connection between UE Tx antenna ports and SS Rx antenna ports, without fader and noise source.

# Conclusion

4.1	BS Demodulation Requirements

Based on the above discussion following observations are captured:

Observation 1: No new deployment scenario is introduced for 3Tx MIMO transmission.

Observation 2:  For non-coherent uplink precoding by a 3TX UE, new precoders are agreed in RAN1 and need to be considered by RAN4 to define BS demodulation requirements.

Observation 3: It may be feasible to add 3TX requirements in the same table as existing 1,2 and 4Tx requirements in TS 38.104 “Sec. 8.2: Performance requirements for PUSCH”.

Based on the above discussion following proposals are captured:

Proposal 1: Re-use the legacy channel models, e.g., TDLA30-10/TDLB100-400/TDLC300-100, for initial 3Tx MIMO demodulation simulations.

Proposal 2: Consider new precoding matrix for non-coherent uplink precoding by 3Tx UE to define 3Tx MIMO BS demodulation requirements. Correspondingly, the TPMI index 0 should be used for three-layer transmission.

Proposal 3: Only new normal PUSCH demodulation requirements are considered for 3Tx MIMO.

Proposal 4: It can be feasible to adopt the parameter set in Table 2.3-1 for initial simulations.

- If significant performance differences are observed between channel bandwidths for each SCS, typical bandwidth may be considered in the requirements to ensure comprehensive test coverage.

- Companies are also encouraged to assess the feasibility of using higher MCS values for each modulation level.

- Table 2.3-1 Initial simulation parameters for FDD and TDD 3Tx MIMO PUSCH (3 layers) demodulation

| nTx | nRx | Mapping type | precoding | DM-RS | DM-RS ports | Channel model | MCS | SCS [kHz] | CBW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 4 / 8 | A | Enable | 1+1 | {0,1,2} | TDLB100-400 | 2 | 15 | 5 |
|  |  |  |  |  |  | TDLC300-100 | 16 |  |  |
|  |  |  |  |  |  | TDLA30-10 | 20 |  |  |
|  |  |  |  |  |  | TDLB100-400 | 2 | 30 | 10 |
|  |  |  |  |  |  | TDLC300-100 | 16 |  |  |
|  |  |  |  |  |  | TDLA30-10 | 20 |  |  |

4.2	UE Demodulation Requirements

Based on the above discussion following observations are captured:

Observation 4: No significant performance difference between antenna configuration options 8x4 and 16x2.

Observation 5: There are two CSI reporting delays according to UE capability.

Observation 6: For eTypeI PMI reporting requirements with 64 ports, the performance loss due to the CSI reporting delay Capability 2 is not significant compared to Capability 1.

Observation 7: For eTypeI PMI reporting requirements with 128 ports, the performance loss due to the CSI reporting delay Capability 2 is about 2dB compared to Capability 1, however it is still observed the gain compared with random PMI.

Observation 8: SRS port grouping feature assumes the reciprocity-based downlink transmission.

Observation 9: RAN4 UE demodulation and CSI reporting performance requirements have not been considered specifying the requirements using the reciprocity-based channel information.

Observation 10: RAN5 UE demodulation and CSI reporting performance test setup so far uses direct connection between UE Tx antenna ports and SS Rx antenna ports, without fader and noise source.

Based on the above discussion following proposals are captured:

Proposal 5: For eTypeI PMI reporting requirements with 64 ports, configure (N1, N2, O1, O2) = (8, 4, 4, 4).

Proposal 6: For eTypeI PMI reporting requirements with 64 ports, set the port mapping method (portMappingMethod) to ‘method1’.

Proposal 7: Define eTypeI PMI reporting requirements with 128 CSI-RS ports.

Proposal 8: For eTypeI PMI reporting requirements with 128 ports, configure ‘Mode A’ for Codebook mode (typeI-codebookMode-r19).

Proposal 9: For eTypeI PMI reporting requirements with 128 ports, configure ‘method2’ for the port mapping method (portMappingMethod).

Proposal 10: For eTypeI PMI reporting requirements with 128 ports, schedule all the CSI-RS resources for CSI acquisition in one slot.

Proposal 11: For eTypeI PMI reporting requirements, set CSI feedback delay based on Capability 2.

Proposal 12: RAN4 uses TDLA30-5 for eTypeI PMI reporting requirements.

Proposal 13: Define eTypeII-r19 PMI reporting requirements with 64 CSI-RS ports, where (N1, N2) = (8, 4).

Proposal 14: For eTypeII-r19 PMI reporting requirements with 64 ports, set the test metric as the throughput ratio between the following PMI and random eTypeI-r19 PMI Mode A with 64 ports.

Proposal 15: For eTypeII-r19 PMI reporting requirements with 64 ports, consider using the test parameters of the existing Rel-16 eType-II PMI reporting requirements with 16 CSI-RS ports for the initial evaluation, e.g., TDLA30-5, MCS20 with rank 2, Codebook Configuration.

Proposal 16: For eTypeII-r19 PMI reporting requirements, set CSI feedback delay based on Capability 2.

Proposal 17: Define eTypeII-r19 PMI reporting requirements for 2Rx/4Rx UEs with FR1 FDD SCS=15kHz and FR1 TDD SCS=30kHz.

Proposal 18: RAN4 does not define eTypeII-r19 PMI reporting requirements.

Proposal 19: RAN4 does not define CSI reporting requirements for CJT calibration.

Proposal 20: RAN4 does not define PDSCH demodulation requirements for ctjSchemeC, ctjSchemeD, and ctjSchemeE.

# References

- RP-242394, “WID revision: NR MIMO Phase 5”, Samsung, RAN#105, September 2024, Melbourne, Australia.

- R1-2501746, “FL Summary Support for 3TX CB-based Uplink; First Round”, April 2025.

- 3GPP TS 38.104, “NR; Base Station (BS) radio transmission and reception”.

- R1-2502977, “Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis”, RAN1.

- R4-2508627, “Way Forward for [115][329] NR_MIMO_demod”, Samsung.

- R1-2504673, “Updated RAN1 UE features list for Rel-19 NR after RAN1 #121”, RAN1.