3GPP TSG-RAN WG4 Meeting #116	R4-2510397

Bangalore, India, 25th – 29th August 2025

Source:	Ericsson

Title:	Discussion on Rel-19 NR NTN Phase 3 SAN demodulation requirements

Agenda item:	7.27.7

Document for:	Discussion

# Introduction

Agreements:

Issue 2-1-1: SAN demodulation requirements for RedCap/eRedCap

- No SAN demodulation requirements for the support of (e)RedCap UEs.

Issue 2-2-1: PUSCH requirements for UL coverage enhancements

- Define SAN PUSCH demodulation requirements for DFT-s-OFDM with orthogonal cover codes.

Issue 2-2-2: OCC parameters

- Apply inter-slot OCC length 2.

- Apply PUSCH type A repetitions

In RAN#104, a revised WID [1] on NR NTN enhancements was endorsed for Release 19. In RAN4#115 meeting, companies agreed on defining SAN PUSCH requirements for UL coverage enhancements with orthogonal cover codes (OCC). This was due to additional SAN baseband processing that must support OCC de-spreading to correctly separate and decode the multiplexed UE transmissions. The agreements in the previous meeting for SAN demodulation requirements are copied below:

Further, as per the current WF [2], the open issues are copied below. In this meeting, we discuss the detailed simulation assumptions.

Way forward:

Issue 2-2-2: OCC parameters

- FFS whether to define PUSCH requirements with inter-slot OCC length 4.

Issue 2-2-3: Detailed PUSCH parameters

- Assume to set the following parameters:

  - FDD SCS=15KHz with 5MHz CBW and FDD SCS=30KHz with 10MHz CBW

  - Both 1Rx and 2Rx, with test applicability rule for different Rx number

  - Same MCS for two UEs for PUSCH with OCC

  - Same channel model for two UEs for PUSCH with OCC

  - Frequency offset between two UEs

    - Baseline assumption: same frequency offset for two UEs in one OCC group. FFS what frequency offset value is assumed.

    - FFS whether to consider different frequency offsets values for two UEs additionally.

- FFS for other parameters.

# SAN Demodulation Requirements

This WI focuses on capacity enhancement on uplink through multiplexing technique with OCC, ensuring improved system capacity for large number of UEs under NTN coverage, especially for LEO where rapid access and release of satellite resources are needed.

2.1 Simulation Assumptions

As per the WF [2], companies agreed to apply PUSCH repetitions type A requirements defined in TS 38.108, while incorporating OCC for DFT-s-OFDM PUSCH. In this section, we discuss the parameters for initial simulations.

- OCC length

Based on RAN1 agreement [4], inter-slot OCC with OCC length 2 and 4 are supported. Typically, in RAN4 SAN demodulation, we consider minimum OCC-length to define the requirements and test the impact of OCC on baseband processing. From our simulations [3] for initial evaluation, we observed a reasonable target SNR with OCC length-2. Therefore, OCC-length 2 seems enough to define the requirements.

Proposal 1: Consider only OCC-length 2 to define Rel-19 SAN PUSCH demodulation requirements.

- Channel Bandwidth and Sub-carrier spacing

As per the test applicability rules, only the widest supported channel bandwidth (CBW) is tested for each sub-carrier spacing (SCS). Therefore, following current requirements for PUSCH repetition type A, we prefer to consider minimum CBW per SCS.

Proposal 2: Consider minimum channel bandwidth per sub-carrier spacing, i.e.15KHz, 5MHz and 30KHz, 10MHz.

## RAN1#120bis agreements

Agreement

RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH.

- FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling

Agreement

Send LS to RAN4 on requirements for the phase continuity and power consistency:

RAN1 has agreed the following:

- RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH.

  - FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling

RAN1 ask RAN4 whether the same phase continuity requirements as for DMRS bundling can be applied for OCC length 2 and/or OCC length 4 under the same conditions as for DMRS bundling, or if new requirements are needed.

Duplex Mode

As per the RAN1 agreement [4], UE is required to maintain phase and continuity in one OCC group. Although this has not been concluded in RAN4 RF discussion, but in FDD mode it is more likely for a UE to maintain phase and power consistency than TDD mode, as observed in Rel-17 NR coverage enhancement requirements. Also, currently for all NR-NTN bands there are only requirements for the FDD mode.

Observation 1: RAN4 SAN Demodulation should wait for the conclusion of phase continuity and power consistency discussions in RAN4 RF.

Observation 2: For PUSCH OCC, it is better to model the phase continuity tolerance of UE and the carrier frequency offset for the minimum performance requirement to check receiver algorithm if it is feasible for TE implementation.

Proposal 3: For the simulations and minimum requirement, we only consider the FDD mode.

- Antenna Configuration

Following existing FR1 NR-NTN requirements, we prefer to consider both 1Tx 1Rx and 1Tx 2Rx antenna configuration.

Proposal 4: Consider 1Tx with 1/2 Rx antenna configuration for defining the SAN PUSCH demodulation requirements with OCC.

- Channel Model

As per the WF [2], it is reasonable to consider the same channel model for two or four UEs multiplexed with OCC for simulations and to simplify the test.

Proposal 5: Consider same channel model for multiple UEs for SAN PUSCH requirements with OCC.

Regarding the propagation condition, typically NTN-TDLA100-200 channel models are used for non-LoS (line of sight) and NTN-TDLC5-200 for LoS scenarios for Rel-17 FR1-NTN requirements scenarios. Since we apply PUSCH type A repetitions as per the WF, we can consider the same channel model being used for those requirements as a starting point, i.e. NTN-TDLA100-200. However, from our initial simulations in Fig.1, we observe OCC performance is sensitive to the Doppler shift. As the Doppler increases, channel correlation decreases, which degrades the effectiveness of symbol-level combining at the receiver. Therefore, to mitigate the impact of high Doppler, we could consider also the LoS channel model, where performance loss can be smaller.

Figure 1: Comparison for NTN-TDLA and NTN-TDLC channels with different Doppler shifts

Observation 3: As per our initial simulations, it was seen that OCC performance is sensitive to the Doppler shift with non-LoS (line of sight) channel model, but not much in LoS channel model.

Observation 4: In the low SNR region, there is not much impact with Doppler shift. However, LoS channel model provides a better performance than non-LoS channel.

Proposal 6: Consider both ‘NTN-TDLA100-200 Low’ and ‘NTN-TDLC5-200 Low’ channel models for initial simulations for SAN PUSCH requirements with OCC with further down selection.

- MCS

Since there has been no discussion on channel model yet, the decision for MCS could be based on simulation with LoS/non-LoS channel model with different antenna configurations. However, typically MCS 4 (in Table 1) and MCS 5 (in Table 3) have been used for Rel-17 FR1-NTN requirements, where MCS 5 is defined with PUSCH repetitions type A (for coverage limited NTN). Therefore, both MCS 4 and 5 would be suitable for initial evaluation.

Proposal 7: Consider MCS 4 and 5 as a starting point. However, the decision of MCS depends on the agreement of the channel model.

## RAN1#120bis agreements

Agreement

For OCC with CG-PUSCH, the RV sequence applied across OCC groups is RRC configured among the RV sequences defined for legacy CG PUSCH, i.e., [0,2,3,1], [0,3,0,3] or [0,0,0,0].

- Note: no new RRC parameter is needed for the above

- FFS: if OCC group dropping is later agreed, how to count RVs may need to be discussed for that case

## RAN1#120 agreements

Agreement

For RV cycling for OCC with DG-PUSCH, support Option 1

- Option 1: RV cycling is used across OCC groups.

  - Note: when option 1 is applied, RV cycling is applied when the number of repetitions is greater than the OCC length

No optimization in Rel-19 for pairing UEs with OCC2 and UEs with OCC4.

Redundancy version

According to the RAN1 agreements, RV cycling is used across OCC groups when the number of repetitions is greater than the OCC length. When number of repetitions are same as the OCC length, one OCC group is available with constant RV.

Proposal 8: For initial simulations, we prefer to consider only one OCC group, with RV value configured as 0.

- Number of repetitions

Currently, for Rel-17 FR1-NTN PUSCH requirements with repetitions type A, we have used aggregation factor as 2 in FDD duplex mode for both 15 and 30 KHz SCS. For initial simulations, we prefer to consider the same number of repetitions in one OCC group.

Proposal 9: For starting point, consider PUSCH aggregation factor as n2 for simulations with SAN PUSCH OCC requirements.

- Frequency offset

RAN1#119 agreements

Agreement

RAN1 to confirm the working assumption of RAN1#118bis with revisions as follows:

Note 2: gNB can ensure the performance of Option 1 by UE grouping with similar CFO (e.g. maximum differential CFO of 50 or 100 Hz or 200 Hz). Without CFO grouping (e.g. maximum differential CFO of 400 Hz), the performance of option 1 is degraded by at least 1 dB in several cases. For CFO grouping, several companies in RAN1 state that CFO grouping is feasible based on network implementation without any new specification impact.

- RAN1 assumes no specification impact for CFO grouping

- RAN1 does not pursue closed-loop frequency adjustment commands.

- RAN1 assumes that RAN4 does not define new UE requirements for CFO.

As per the understanding of OCC group, ideally the UEs multiplexed with OCC should not have CFO in one OCC group. Further, the UE should maintain phase and power consistency within the OCC group to enable inter-slot OCC. However, there has been no agreements in RAN4 RF discussion and RAN1 regarding this yet.

As per RAN4 assumptions, we typically consider 0.1ppm in CFO in the RF requirements, while ideally as per the definition of OCC group there should not be any CFO between the multiplexed UEs. While we are open to discussion of frequency offset assumption, we recommend for the initial simulations and evaluation to consider both ideal and worse-case scenario by assuming 0Hz and 0.1ppm CFO, respectively in one OCC group.

Proposal 10: For initial simulations, take both ideal and worse-case scenario for CFO, i.e. 0 Hz and 0.1ppm which is typically used for RAN4 requirement.

- DMRS Configuration type

DMRS Configuration Type defines different patterns for placing DMRS within a Physical Resource Block. Type 2 is typically used in high-mobility scenarios, MU-MIMO configurations, etc. where DMRS symbols are mapped to a denser set of subcarriers and support more orthogonal DMRS ports, compared to type 1. This enables improved channel estimation, especially in scenarios with higher frequency selectivity. For SAN PUSCH demodulation requirements, we have typically used configuration type 1 and the same can be considered for SAN PUSCH requirements with OCC.

Proposal 11: Only consider DMRS configuration type 1 for SAN PUSCH requirements with OCC.

- OCC Sequence index

RAN1#121 agreements

OCC length indication:

Agreement

For the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2, support:

- Option 3: Candidates values are configured by RRC higher-layer parameter as part of the TDRA table with a new column for configuring the OCC length, and the applied value is indicated by scheduling DCI for DG PUSCH or by activation DCI for CG PUSCH Type 2.

  - No additional rows for the TDRA table

OCC sequence indication:

Agreement

For the OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2, the sequence is indicated dynamically in DCI Format 0_1 and Format 0_2. Support implicit DCI indication with re-use of antenna port field.

Association between antenna ports and OCC sequence is defined by re-using the legacy tables with two new columns added for OCC sequence. Note: the tables could directly reference the OCC sequence index (Table 6.3.2.5A-1 and Table 6.3.2.5A-2 in TS38.211)

As per the RAN1 agreements, OCC length is known to the UE configured with higher-layer RRC parameters and OCC sequence is indicated via DCI scheduling.

This can be further discussed among the companies, but OCC sequence index can be associated with antenna port indicated in the DCI as the following:

For OCC-2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | OCC sequence index |
| --- | --- | --- | --- |
| 0 | 2 | 0 | 0 |
| 1 | 2 | 1 | 1 |
| 2 | 2 | 2 | 0 |
| 3 | 2 | 3 | 1 |

For our initial simulations, we suggest multiplexing two UEs with OCC sequence index {0,1} corresponding to OCC sequences [1 1] and [1 -1] on DMRS ports {0,1}.

Proposal 12: Consider OCC sequence index as {0,1} as the starting point for initial simulations.

Furthermore, to verify performance, it would be practical to evaluate the UE multiplexed with the OCC sequence [1, -1], ensuring that the OCC sequence is applied correctly in the test.

Proposal 13: Evaluate UE performance multiplexed with OCC sequence [1, -1] to verify correct baseband implementation of the OCC sequence in testing.

- DMRS port

As mentioned, UEs can be multiplexed with different OCC sequences on different DMRS ports. As a starting point, we prefer to consider DMRS ports {0,1}.

Proposal 14: Consider DMRS ports {0,1} as the starting point for initial simulations.

- DMRS Sequence generation

Regarding DMRS sequence generation, nSCID (scrambling ID) is used to define variations in the DMRS sequence generation and allows different sequences on the same physical resources, which is useful in handling multi-layer transmissions in MIMO systems. For NID0 (cell-specific ID), different values ensure orthogonality of DMRS sequences.

Following the Rel-17 FR1-NTN requirements for single layer transmission, nSCID=0 and  NID0=0 is suitable choice for DMRS sequence generation.

Proposal 15: Choose nSCID=0 and  NID0=0 for DMRS sequence generation for SAN PUSCH requirements for OCC.

- Additional DMRS

For DMRS position, typical used configuration if ‘1+1, i.e. pos1’ in NTN scenario. Therefore, we can consider the same configuration for requirements with OCC.

Proposal 16: Use Additional DMRS position as ‘pos1’ for SAN PUSCH requirements for OCC.

RAN1#121 agreements

Agreement

A UE is not expected to be configured with frequency hopping for PUSCH repetitions with inter-slot OCC.

Frequency hopping

According to RAN1 agreements, frequency hopping should not be considered for PUSCH repetitions with OCC, since phase continuity and power consistency may not be guaranteed after frequency hopping. This is in-line with existing SAN PUSCH repetitions requirements.

Proposal 17: Do not consider frequency hopping for SAN PUSCH requirements with OCC.

- Test Metric

In current FR1 NR-NTN, we typically SNR@70% throughput as the test metric for most of the performance requirements. However, only for PUSCH repetitions type A, we have used target BLER as the test metric, since we evaluate cell-edge coverage. In this WI, we emphasize more on the capacity. Therefore, we prefer to use target SNR, as the test metric.

Proposal 18: Consider SNR@70% normalized throughput as the test metric to define the requirements.

# UE Demodulation Requirements

In this section, we discussed the UE demodulation requirements for adding the support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1 NTN bands.

## 3.1	Test scope

Based on the result of discussion in the last RAN4 #115 meeting, it’s been agreed to only consider the demodulation impact for the feature of the support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands.

As a result of the last meeting’s discussion, the following agreement/way forward has been agreed:

|  | Rel-17 RedCap | Rel-18 eRedCap |
| --- | --- | --- |
| 1Rx UE | Discuss whether to add new PDSCH requirement with NTN channel | Discuss whether to add new PDSCH requirement with NTN channel |
| 2Rx UE | Reuse Rel-17 NTN PDSCH demodulation requirements. - Companies can check whether all the Rel-17 NTN features can be applicable for RedCap. | Discuss whether to add new PDSCH requirements with BW reduction (25RB) with NTN channel Or to reuse Rel-17 NTN PDSCH demodulation requirements without BW reduction |

For Rel-17 RedCap, as part of the feature objectives, RAN4 defined demodulation requirements applicable to 1Rx UEs. However, in Rel-17 NTN, the specified PDSCH demodulation requirements are based solely on 2Rx reception. Therefore, for Rel-19, it is preferred to introduce new PDSCH demodulation requirements to ensure that NTN-specific requirements are also defined for 1Rx RedCap UEs, in order to guarantee their performance.

Proposal 19: Define new PDSCH requirements with NTN channel model for Rel-17 RedCap 1Rx UE.

For Rel-17 RedCap 2Rx UEs, it was agreed to reuse the Rel-17 NTN PDSCH demodulation requirements. During our review of Rel-17 NTN features in the TS38.101-4, as listed below, the two features — “increasing the number of HARQ processes (max-HARQ-ProcessNumber-r17)” and “disabling HARQ feedback for downlink transmission (harq-FeedbackDisabled-r17)” — were identified as unique aspects corresponding to NTN transmission.

Table 8.2.1.1.2-1: Requirements applicability for optional UE features

| UE feature/capability [TBD] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| NR NTN access (nonTerrestrialNetwork-r17) | FR1 FDD | PDSCH | Clause 8.2.1.2.2.1 (Test 1-1, Test 1-2, Test 1-3, Test 1-4) |  |
| NR NTN scenario support (ntn-ScenarioSupport-r17) | FR1 FDD | PDSCH | Clause 8.2.1.2.2.1 (Test 1-1, Test 1-2, Test 1-3, Test 1-4) | The requirements apply only when ntn-ScenarioSupport-r17 is “ngso” or is not configured. |
| Increasing the number of HARQ processes (max-HARQ-ProcessNumber-r17) | FR1 FDD | PDSCH | Clause 8.2.1.2.2.1 (Test 1-3) |  |
| Disabled HARQ feedback for downlink transmission (harq-FeedbackDisabled-r17) | FR1 FDD | PDSCH | Clause 8.2.1.2.2.1 (Test 1-4) |  |
| Note: For UE supporting NR NTN access (nonTerrestrialNetwork-r17), the requirements in TS38.101-4 also applies to UE according to applicability rules in TS38.101-4 Clause 5.1, 6.1, 7.1 and 8.1 |  |  |  |  |

These two HARQ enhancement features should be both applied to Rel-17 RedCap 2Rx UEs as well.

When defining new requirements for Rel-17 RedCap 1Rx UEs, if introduced, we prefer to consider both two NTN features of HARQ enhancement. Proposed test cases can be found in the next section.

For Rel-18 eRedCap, RAN4 only defined additional PDSCH requirements since the difference between RedCap and eRedCap is that the peak rate and/or PDSCH BW of eRedCap is limited, and for PDCCH/PBCH, the eRedCap UE should be able to receive with 20MHz CBW, RAN4 can reuse the requirements specified in Rel-17 RedCap.

When adding the support of NTN(Rel-19 enhancement), we think it’s necessary to define some dedicated test cases to guarantee the performance of reduced baseband bandwidth. In our view, we prefer to define some new cases for 1Rx and 2Rx UEs.

In Rel-18 eRedCap, RAN4 defined two sets of PDSCH demodulation requirements according to capability 1) with reduced baseband bandwidth (FG 48-1) and 2) without reduced baseband bandwidth (FG 48-2). Since FG 48-2 is the capability that the UE receives PDSCH with 10MHz, but the peak rate is limited to 10 Mbps, we think this is close to the Rel-17 RedCap. Also, FG 48-2 is an additional capability to FG 48-1. Therefore, we propose to focus on the basic eRedCap feature FG 48-1, ‘with reduced baseband bandwidth’ (25RB), for NTN scenario.

Proposal 20: For eRedCap UEs, to add new PDSCH requirements with BB BW reduction with NTN channel for 1Rx and 2Rx UEs.

Regarding the PDSCH scheduling for HD-FDD, we prefer to define new reference channels for it with the same performance requirements as for FDD (if introduced).

Proposal 21: Define new reference channel for HD-FDD (e)RedCap with the same performance requirements as for FDD.

To summarize the test scope, we think for Rel-19 NTN enhancement, RedCap and eRedCap UEs need to first pass the corresponding Rel-17, Rel-18 PDSCH/PDCCH/PBCH requirements according to the UE capability, then some additional requirements configured with NTN channel model should be passed as well. Thus, as for the test scope of UE demodulation requirements, we summarized our proposals as below:

|  | Rel-17 RedCap | Rel-18 eRedCap |
| --- | --- | --- |
| 1Rx UE | Add new PDSCH requirement with NTN channel - SCS=15kHz, CBW=10MHz, 1Rx, NTN channel model | Add new PDSCH requirement with NTN channel - SCS=15kHz: CBW=10MHz with reduced baseband bandwidth, 25PRB |
| 2Rx UE | Reuse Rel-17 NTN PDSCH demodulation requirements. - Apply all the Rel-17 NTN features | Add new PDSCH requirement with NTN channel - SCS=15kHz: CBW=10MHz with reduced baseband bandwidth, 25PRB |

In the following section we discussed the detailed test case design.

## 3.2	PDSCH requirements

Since the target of Rel-19 NTN enhancement is to add support of NTN to RedCap and eRedCap UEs, it is very important that those UE to pass the legacy RedCap and eRedCap PDSCH requirements. On top of that, the PDSCH performance under the NTN channel model should also be verified.

Based on the analysis above, for RedCap 1Rx UEs, we propose the following new PDSCH test cases:

| Test num. | Reference channel (Refer to TS 38.101-4) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | HARQ Feedback | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | R.PDSCH.1-1 FDD R.PDSCH.1-1.1 HD-FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | Disable HARQ | 1x1, ULA Low | 70 |  |
| 2 | R.PDSCH.1-1.1 FDD R.PDSCH. 1-1.1 HD-FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 32 HARQ processes | 1x1, ULA Low | 70 |  |
| 3 | R.PDSCH.1-2.1 FDD R.PDSCH. 1-1.2 HD-FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 16 HARQ processes | 1x1, ULA Low | 70 |  |

For eRedCap 1Rx UEs, we propose the following PDSCH test cases (with reduced baseband bandwidth):

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | HARQ Feedback | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | R.PDSCH.1-1.5 FDD R.PDSCH.1-2.2 HD-FDD | 10 / 15 (25PRB) | QPSK, 0.30 | NTN-TDLA100-200 | Disable HARQ | 1x1, ULA Low | 70 |  |
| 2 | R.PDSCH.1-1.5 FDD R.PDSCH.1-2.2 HD-FDD | 10 / 15 (25PRB) | QPSK, 0.30 | NTN-TDLA100-200 | 32 HARQ processes | 1x1 ULA Low | 70 |  |
| 3 | R.PDSCH.1-25.1 FDD R.PDSCH.1-2.3 HD-FDD | 10 / 15 (25PRB) | 16QAM, 0.48 | NTN-TDLC5-200 | 16 HARQ processes | 1x1, ULA Low | 70 |  |

For eRedCap 2Rx UEs, we propose the following PDSCH test cases (with reduced baseband bandwidth):

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | HARQ Feedback | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | R.PDSCH.1-1.5 FDD R.PDSCH.1-2.2 HD-FDD | 10 / 15 (25PRB) | QPSK, 0.30 | NTN-TDLA100-200 | Disable HARQ | 1x2, ULA Low | 70 |  |
| 2 | R.PDSCH.1-1.5 FDD R.PDSCH.1-2.2 HD-FDD | 10 / 15 (25PRB) | QPSK, 0.30 | NTN-TDLA100-200 | 32 HARQ processes | 1x2, ULA Low | 70 |  |
| 3 | R.PDSCH.1-25.1 FDD R.PDSCH.1-2.3 HD-FDD | 10 / 15 (25PRB) | 16QAM, 0.48 | NTN-TDLC5-200 | 16 HARQ processes | 1x2, ULA Low | 70 |  |

Proposal 22: Define new PDSCH demodulation requirements with the following test setup:

Test cases of PDSCH for RedCap 1Rx

- MCS4, NTN-TDLA100-200, 1x1, Disabled HARQ

- MCS13, NTN-TDLC5-200, 1x1, 16 HARQ processes

- MCS4, NTN-TDLA100-200, 1x1, 32 HARQ processes

Test cases of PDSCH for eRedCap 1Rx and 2Rx

- MCS4, NTN-TDLA100-200, 1x1, with reduced BB BW (25RB), Disabled HARQ

- MCS13, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 16 HARQ processes

- MCS4, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 32 HARQ processes

In the meantime, we found that it is worth to have discussions on the feasibility of implementing the NGSO channel model for the potential Rel-19 NTN enhancement requirements. Since Rel-19 WI NR_IoT_NTN_req_test_enh has completed the DL Doppler shift and propagation delay modeling with NGSO satellite and RAN4 is discussing the performance impact to the existing Rel-17 NTN PDSCH demodulation requirements if the model is applied. Considering the realistic scenario, RAN4 should discuss whether the NGSO channel model is applied for (e)RedCap UE in NTN later. We are open to consider it if it finally concludes with no additional margin to be added.

Proposal 23: Open to consider applying the NGSO model for Rel-19 NTN enhancement requirements to (e)RedCap UE PDSCH demodulation requirements.

# Conclusion

  - SAN Demodulation Requirements

Based on our discussion, the following observations have been captured:

Observation 1: RAN4 SAN Demodulation should wait for the conclusion of phase continuity and power consistency discussions in RAN4 RF.

Observation 2: For PUSCH OCC, it is better to model the phase continuity tolerance of UE and the carrier frequency offset for the minimum performance requirement to check receiver algorithm if it is feasible for TE implementation.

Observation 3: As per our initial simulations, it was seen that OCC performance is sensitive to the Doppler shift with non-LoS (line of sight) channel model, but not much in LoS channel model.

Observation 4: In the low SNR region, there is not much impact with Doppler shift. However, LoS channel model provides a better performance than non-LoS channel.

Based on our discussion, the following proposals have been captured:

Proposal 1: Consider only OCC-length 2 to define Rel-19 SAN PUSCH demodulation requirements.

Proposal 2: Consider minimum channel bandwidth per sub-carrier spacing, i.e.15KHz, 5MHz and 30KHz, 10MHz.

Proposal 3: For the simulations and minimum requirement, we only consider the FDD mode.

Proposal 4: Consider 1Tx with 1/2 Rx antenna configuration for defining the SAN PUSCH demodulation requirements with OCC.

Proposal 5: Consider same channel model for multiple UEs for SAN PUSCH requirements with OCC.

Proposal 6: Consider both ‘NTN-TDLA100-200 Low’ and ‘NTN-TDLC5-200 Low’ channel models for initial simulations for SAN PUSCH requirements with OCC with further down selection.

Proposal 7: Consider MCS 4 and 5 as a starting point. However, the decision of MCS depends on the agreement of the channel model.

Proposal 8: For initial simulations, we prefer to consider only one OCC group, with RV value configured as 0.

Proposal 9: For starting point, consider PUSCH aggregation factor as n2 for simulations with SAN PUSCH OCC requirements.

Proposal 10: For initial simulations, take both ideal and worse-case scenario for CFO, i.e. 0 Hz and 0.1ppm which is typically used for RAN4 requirement.

Proposal 11: Only consider DMRS configuration type 1 for SAN PUSCH requirements with OCC.

Proposal 12: Consider OCC sequence index as {0,1} as the starting point for initial simulations.

Proposal 13: Evaluate UE performance multiplexed with OCC sequence [1, -1] to verify correct baseband implementation of the OCC sequence in testing.

Proposal 14: Consider DMRS ports {0,1} as the starting point for initial simulations.

Proposal 15: Choose nSCID=0 and  NID0=0 for DMRS sequence generation for SAN PUSCH requirements for OCC.

Proposal 16: Use Additional DMRS position as ‘pos1’ for SAN PUSCH requirements for OCC.

Proposal 17: Do not consider frequency hopping for SAN PUSCH requirements with OCC.

Proposal 18: Consider SNR@70% normalized throughput as the test metric to define the requirements.

4.2   UE Demodulation Requirements

Based on our discussion, the following proposals have been captured:

Proposal 19: Define new PDSCH requirements with NTN channel model for Rel-17 RedCap 1Rx UE.

Proposal 20: For eRedCap UEs, to add new PDSCH requirements with BB BW reduction with NTN channel for 1Rx and 2Rx UEs.

Proposal 21: Define new reference channel for HD-FDD (e)RedCap with the same performance requirements as for FDD.

Proposal 22: Define new PDSCH demodulation requirements with the following test setup:

Test cases of PDSCH for RedCap 1Rx

•	MCS4, NTN-TDLA100-200, 1x1, Disabled HARQ

•	MCS13, NTN-TDLC5-200, 1x1, 16 HARQ processes

•	MCS4, NTN-TDLA100-200, 1x1, 32 HARQ processes

Test cases of PDSCH for eRedCap 1Rx and 2Rx

•	MCS4, NTN-TDLA100-200, 1x1, with reduced BB BW (25RB), Disabled HARQ

•	MCS13, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 16 HARQ processes

•	MCS4, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 32 HARQ processes

Proposal 23: Open to consider applying the NGSO model for Rel-19 NTN enhancement requirements to (e)RedCap UE PDSCH demodulation requirements.

# References

RP-243300, Moderator (Thales), New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#106, Madrid, Spain, December 9-12, 2024.

R4-2508755, “Way Forward for [115][333] NR_NTN_Ph3_demod”, Ericsson, RAN4#115, Malta.

R4-2510398, “Simulation results for Rel-19 NR-NTN Phase 3 SAN demodulation requirements”, Ericsson, RAN4#116, Bangalore, India.

R1-2503686, “Feature lead summary #1 of AI 9.11.3 on NR-NTN uplink capacity and throughput”, Malta