# 3GPP TSG-RAN WG4 Meeting #116                                 R4-2510400

# Bengaluru, India, 25th-29th Aug., 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on REFSENS, ASCS, ACS of LP-WUR

# Agenda Item:   7.24.3.2

# Document for:  Approval

- Introduction

In last meeting, RAN4 has achieved agreements on some of the issues for LP-WUS, such as system parameters, SNR and ASCS requirement. The detailed spec language and TR language needs to finalized in this meeting. Besides, there are still some issues left to be decided, such as LP-WUR type, REFSENS, ACS requirements and receiver characteristics[1]. In this contribution, we provided our views and simulation results on REFSENS, ACS for LP-WUR based on the latest output on signal design from RAN1[2].

- Discussion

## 2.1 LP-WUR Type

In RAN4, there are two candidate methods to define LP-WUR types. One is to align with RAN1 UE feature list. The other one is based on NF+IM package.

In RAN1 latest feature list as shown in Fig.1 [3], UE feature of NR_LPWUS is separated by LP-WUS operation based on OOK signal and OFDM overlaid sequence. It can be found that OFDM-based receiver needs to achieve more functions than OOK-based receiver, such as sequence detection and SSB-based RRM measurement.

Fig. 1  Partial content of LP-WUS UE feature list (IDLE/INACTIVE mode) [3]

Observation 1: OFDM-based receiver needs to achieve more functions than OOK-based receiver, such as sequence detection and SSB-based RRM measurement.

Also, we checked some endorsed draft CR in RAN2, for example draft CR on RRC specification [4]. According to the highlighted green part in Fig. 2, it can be found that in RAN2’s draft CR, ‘OOK based LP-WUR’ corresponds with the components of LP-WUS operation based on OOK signal, and ‘OFDM based LP-WUR’ corresponds with the components of LP-WUS operation based on OFDM overlaid sequence.

Fig. 2  Partial content of RAN2 RRC Running CR for LP-WUS/WUR (Endorsed) [4]

Observation 2: In RAN2’s draft CR, ‘OOK based LP-WUR’ corresponds with the components of LP-WUS operation based on OOK signal, and ‘OFDM based LP-WUR’ corresponds with the components of LP-WUS operation based on OFDM overlaid sequence.

Therefore, in order to align with RAN1 and RAN2 and reduce complex mapping issues, we prefer to define LP-WUR types as:

Type 1 LP-WUR: OOK-based receiver, map to LP-WUS operation based on OOK signal in RAN1 feature list;

Type 2 LP-WUR: OFDM-based receiver, map to LP-WUS operation based on OFDM overlaid sequence in RAN1 feature list.

Proposal 1: Propose to define LP-WUR types as:

Type 1 LP-WUR: OOK-based receiver, map to LP-WUS operation based on OOK signal in RAN1 UE feature list;

Type 2 LP-WUR: OFDM-based receiver, map to LP-WUS operation based on OFDM overlaid sequence in RAN1 UE feature list.

## 2.2 REFSENS

The REFSENS for MR can be expressed as

REFSENS(dBm) = kT0 + 10log10(NRB*12*SCS) + NF + SNR + IM - Diversity Gain

The REFSENS of normal 2Rx MR is calculated based on: SNR=-1dB, NF+IM=11.5dB, 2Rx Diversity Gain = 3dB.

It In last meeting, RAN4 has agreed to define -4.5dB SNR for both OOK-based receiver and OFDM-based receiver.

Then the REFSENS for LP-WUR can be expressed as

REFSENS(dBm) = kT0 + 10log10(NRB_WUS*12*SCS) + NF + SNR + IM.

Based on the conclusions in previous meetings, the REFSENS of two types of LP-WURs are calculated based on:

- Type 1 LP-WUR: SNR=-4.5dB, NF+IM=18.0dB, 1Rx Diversity Gain = 0dB;

- Type 2 LP-WUR: SNR=-4.5dB, NF+IM=13.5dB, 1Rx Diversity Gain = 0dB.

According to the REFSENS formula, we can derive the REFSENS gap between LR and normal 2Rx MR under the same operating band, transmission bandwidth configuration and SCS, i.e., for Type 1 LP-WUR, REFSENS is 6dB higher than MR and for Type 2 LP-WUR, REFSENS is 1.5dB higher than MR.

However, there is not an MR operating in the same condition with LP-WUR in current specification. Besides, although the formula can be used to evaluate the REFSENS, we still need to consider the insertion loss of front-end filter, the interference between FDD UL band and DL band, etc., which may cause the REFSENS difference among different operating bands as that specified in TS38.101-1 for MR. Therefore, for LR, we prefer to define REFSENS for LP-WUR per band.

Proposal 2: Propose to define REFSENS for LP-WUR per band.

We understand that it is difficult to define REFSENS for LR in all the operating bands in one meeting, we can choose some example bands for now, and leave other REFSENS values to maintenance discussion.

Proposal 3: Choose some example bands for now to define REFSENS for LP-WUR, for example as shown in Table 1.

Table 1  Reference sensitivity of LP-WUR PREFSENS

| Operating Band | SCS (kHz) | Type 1 LP-WUR (dBm) | Type 2 LP-WUR (dBm) |
| --- | --- | --- | --- |
| n3 | 15 | -94.6 | -99.1 |
|  | 30 | -91.0 | -95.5 |
| n8 | 15 | -94.6 | -99.1 |
|  | 30 | -91.0 | -95.5 |

## 2.3 ACS

In last meeting, RAN4 has agreed that 0 guard RB is assumed for deriving both Type 1 and Type 2 ACS requirements. The agreements on ACS for LP-WUR are listed as follows [1].

Issue 2-4-1: Baseline ACS requirements (same interference) and required guard RB for LP-WUR Agreement: - 0 guard RB is assumed for deriving both type-1 and type-2 ACS requirement. Issue 2-4-2: case 1 ACS test parameter WF: - Finalize detailed test parameters next meeting. endorse the definition and clause structure, requirement table (with requirement TBD) this meeting. Issue 2-4-3: case 2 ACS test parameter WF: - Finalize detailed test parameters next meeting. endorse the definition and clause structure, requirement table (with requirement TBD) this meeting.

It has been agreed that same interference level as MR for LP-WUR ACS requirements should be used considering that LP and MR will suffer the same radio environment. Namely, the interference level of LP-WUR can be derived based on the REFSENS gap between LP-WUR and MR. The interference level in ACS test of MR is REFSENSMR + 45.5dB. Therefore, according to the REFESNS gap between MR and LR in Section 2.2, the interference level in ACS test of Type 1 LR should be REFSENSType1_LR + 39.5dB, the interference level in ACS test of Type 2 LR should be REFSENSType2_LR + 44dB.

Then, with the fixed interference level, we need to check how much relaxation is need for wanted signal, namely LP-WUS. We evaluate the MDR of Type 1 LP-WUR and Type 2 LP-WUR versus different relaxation levels. The simulation results showed that 1dB additional relaxation compared to MR is needed for OOK-based receiver and OFDM-based receiver, namely the LP-WUS power should be REFSENSType1/Type2_LR + 15dB.

Proposal 3: Consider the ACS test parameters in Table 2-1, 2-2 and Table 3-1, 3-2.

Table 2-1  Test parameters for ACS of Type 1 LP-WUR, Case 1

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| LP-WUS Power | dBm | REFSENSType1_LR + 15dB |
| Pinterferer | dBm | REFSENSType1_LR + 39.5dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| Guard RB | RB | 1 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier without adjacent sub-carrier interference. |  |  |

Table 2-2  Test parameters for ACS of Type 1 LP-WUR, Case 2

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| LP-WUS Power | dBm | -55 |
| Pinterferer | dBm | -30.5 |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| Guard RB | RB | 1 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier without adjacent sub-carrier interference. |  |  |

Table 3-1  Test parameters for ACS of Type 2 LP-WUR, Case 1

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| LP-WUS Power | dBm | REFSENSType2_LR + 15dB |
| Pinterferer | dBm | REFSENSType2_LR + 44dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| Guard RB | RB | 0 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier without adjacent sub-carrier interference. |  |  |

Table 3-2  Test parameters for ACS of Type 2 LP-WUR, Case 2

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| LP-WUS Power | dBm | -54 |
| Pinterferer | dBm | -25 |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| Guard RB | RB | 0 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier without adjacent sub-carrier interference. |  |  |

- Conclusion

Observations:

Observation 1: OFDM-based receiver needs to achieve more functions than OOK-based receiver, such as sequence detection and SSB-based RRM measurement.

Observation 2: In RAN2’s draft CR, ‘OOK based LP-WUR’ corresponds with the components of LP-WUS operation based on OOK signal, and ‘OFDM based LP-WUR’ corresponds with the components of LP-WUS operation based on OFDM overlaid sequence.

Proposals:

Proposal 1: Propose to define LP-WUR types as:

Type 1 LP-WUR: OOK-based receiver, map to LP-WUS operation based on OOK signal in RAN1 UE feature list;

Type 2 LP-WUR: OFDM-based receiver, map to LP-WUS operation based on OFDM overlaid sequence in RAN1 UE feature list.

Proposal 2: Propose to define REFSENS for LP-WUR per band.

Proposal 3: Choose some example bands for now to define REFSENS for LP-WUR, for example as shown in Table 1.

Table 1  Reference sensitivity of LP-WUR PREFSENS

| Operating Band | SCS (kHz) | Type 1 LP-WUR (dBm) | Type 2 LP-WUR (dBm) |
| --- | --- | --- | --- |
| n3 | 15 | -94.6 | -99.1 |
|  | 30 | -91.0 | -95.5 |
| n8 | 15 | -94.6 | -99.1 |
|  | 30 | -91.0 | -95.5 |

Proposal 3: Consider the ACS test parameters in Table 2-1, 2-2 and Table 3-1, 3-2.

Table 2-1  Test parameters for ACS of Type 1 LP-WUR, Case 1

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| LP-WUS Power | dBm | REFSENSType1_LR + 15dB |
| Pinterferer | dBm | REFSENSType1_LR + 39.5dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| Guard RB | RB | 1 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier without adjacent sub-carrier interference. |  |  |

Table 2-2  Test parameters for ACS of Type 1 LP-WUR, Case 2

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| LP-WUS Power | dBm | -55 |
| Pinterferer | dBm | -30.5 |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| Guard RB | RB | 1 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier without adjacent sub-carrier interference. |  |  |

Table 3-1  Test parameters for ACS of Type 2 LP-WUR, Case 1

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| LP-WUS Power | dBm | REFSENSType2_LR + 15dB |
| Pinterferer | dBm | REFSENSType2_LR + 44dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| Guard RB | RB | 0 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier without adjacent sub-carrier interference. |  |  |

Table 3-2  Test parameters for ACS of Type 2 LP-WUR, Case 2

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| LP-WUS Power | dBm | -54 |
| Pinterferer | dBm | -25 |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| Guard RB | RB | 0 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier without adjacent sub-carrier interference. |  |  |

- Reference

- R4-2507936 WF for [115][136] NR_LPWUS_UERF, Vivo, RAN4#115, St. Julian’s, Malta, May 2025.

- R1-2504943 Reply LS on LP-WUS UE RF, Vivo, RAN1#121, St. Julian’s, Malta, May 2025.

- R4-2509005 LS on updated Rel-19 RAN1 UE features lists for NR after RAN1#121, RAN4#116, Bengaluru, India, Aug. 2025.

- R2-2503612 RRC Running CR for LP-WUS/WUR, Vivo, RAN2#130, St. Julian’s, Malta, May 2025.

- Annex: Simulation assumptions

The simulation assumptions as listed in Table A, especially the channel structure part.

Table A	Link level simulation assumptions for FR1 LP-WUS

| Attributes | Assumptions |
| --- | --- |
| Case name (waveform) | OOK-4 waveform |
| Center frequency | 900MHz, 2.6GHz |
| Channel structure | 5 information bits with 16bit RM coding No CRC is configured |
| Chip rate | M=4 |
| Overlaid OFDM sequence | 4 candidates overlaid sequences for M=4 |
| WUS duration | OOK：RM coding length*2/M = 8 OFDM：ceil(5/log2(4) / (M/2)) = 2 |
| Coding | 1/2 rate Manchester coding |
| Repetition | None |
| Time error | 0/0.9us |
| Residual Frequency error | 0/10/20 ppm |
| SCS | 30kHz |
| UE Channel BW | 20MHz (51 RB)-case 1 10MHz (24 RB)-case 2 5MHz (11 RB)-case 3 |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MHz, 805kHz for 20MHz - For interference cell2 5MHz: fixed 505kHz |
| Guard RB | - For ASCS: 0RB for OFDM-based LP-WUR, 1RB for OOK-based LP-WUR - For ACS: 0 RB |
| Filter | - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded - The filter bandwidth is adapted with actual WUS RBs, for 5MHz case |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = 15~33dB NOTE: Interference level depends on MR |
| Wanted signal level | For ACS, REFSENSLR + 14~20 dB for LP-WUS |
| Sampling rate | 7.68MHz |
| ADC bit width | 8 bits ADC for ASCS/ACS |
| Non-linearities | Not modeled |
| Power boosting | None |
| Channel Model | AWGN |
| Performance metric | - 1% MDR |