# 3GPP TSG-RAN WG4 Meeting #116                                 R4-2510401

# Bengaluru, India, 25th-29th Aug., 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on receiver characteristics for LP-WUR

# Agenda Item:   7.24.3.3

# Document for:  Approval

- Introduction

In this contribution, we provided our views on receiver characteristics for LP-WUR.

- Discussion

## 2.1 Blocking characteristics

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies specified for IBB and OBB. LP-WUR and MR may share the same antenna and baseband processor, which means that LP-WUR and MR will suffer the same level of interference. RAN4 had agreed that same interference level as MR should be considered for IBB and OBB in last meeting[1].

Issue 3-1-1: IBB, OBB requirements Agreements: - Same interference level as MR. further discuss and decide next meeting on relaxed wanted signal.

For the test parameters of LP-WUR, first, similar to ACS test, bandwidth of the interferer should be 5MHz and WUS should be located at edge of the NR carrier. We understand that envelope detection of OOK-based receiver and time-domain sequence detection of OFDM-based receiver are not able to match the performance of traditional frequency-domain demodulation of MR. With same interference level as MR, to guarantee 1% MDR, higher LP-WUS power than MR wanted signal power is required.

Therefore, in terms of the LP-WUS power in IBB and OBB parameters, we agree to add further relaxation for LP-WUR based on legacy REFSENS+6dB. We propose to consider the test parameters for IBB of LP-WUR in Table 1-1, 1-2, 1-3, and the test parameters for OBB of LP-WUR in Table 2-1, 2-2, 2-3 and the test parameters for OBB of LP-WUR in Table 3, Table 4.

Proposal 1: Consider the test parameters for IBB of LP-WUR in Table 1-1, 1-2, 1-3, and the test parameters for OBB of LP-WUR in Table 2-1, 2-2, 2-3. For these test parameters, the missed detection rate shall be ≤1%.

Table 1-1  In-band blocking parameters for Type 1 LP-WUR

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| LP-WUS Power | dBm | REFSENSType1_LR + 15dB |
| BWinterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5 |
| FIoffset case 2 | MHz | 12.5 |
| NOTE 1: LP-WUS is configured at the closest edge from the interferer of the NR carrier without adjacent sub-carrier interference. |  |  |

Table 1-2  In-band blocking parameters for Type 2 LP-WUR

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| LP-WUS Power | dBm | REFSENSType2_LR + 9dB |
| BWinterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5 |
| FIoffset case 2 | MHz | 12.5 |
| NOTE 1: LP-WUS is configured at the closest edge from the interferer of the NR carrier without adjacent sub-carrier interference. |  |  |

Table 1-3  In-band blocking for LP-WUR

| Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- |
| Pinterferer | dBm | -56 | -44 |
| Finterferer (offset)1 | MHz | -BWChannel/2 – FIoffset, case 1 and BWChannel/2 + FIoffset, case 1 | ≤ -BWChannel/2 – FIoffset, case 2 and ≥ BWChannel/2 + FIoffset, case 2 |

Table 2-1  Out-of-band blocking parameters for Type 1 LP-WUR

| WUS transmission bandwidth configuration (RB) | Power in LP-WUS transmission bandwidth configuration [dBm] |
| --- | --- |
| 11RB | REFSENSType1_LR + 15dB |
| NOTE 1: LP-WUS is configured at the closest edge from the interferer of the NR carrier without adjacent sub-carrier interference. |  |

Table 2-2  Out-of-band blocking parameters for Type 2 LP-WUR

| WUS transmission bandwidth configuration (RB) | Power in LP-WUS transmission bandwidth configuration [dBm] |
| --- | --- |
| 11RB | REFSENSType2_LR + 9dB |
| NOTE 1: LP-WUS is configured at the closest edge from the interferer of the NR carrier without adjacent sub-carrier interference. |  |

Table 2-3  Out of-band blocking for LP-WUR

| Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- |
| Pinterferer | dBm | -44 | -30 | -15 |
| Finterferer (CW) | MHz | -60 < f - FDL_low < -15 or 15 < f - FDL_high < 60 | -85 < f - FDL_low ≤ -60 or 60 ≤ f -FDL_high < 85 | 1 ≤ f ≤ FDL_low -85 or FDL_high + 85 ≤ f ≤ 12750 |

## 2.2 Intermodulation Requirements

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal.

The wide band intermodulation requirement is defined using a CW carrier and modulated NR signal as interferer 1 and interferer 2 respectively in TS38.101-1. The missed detection rate shall be ≤1% under these assumptions. Similar to the discussion in Section 2.2, bandwidth of the interferer should be 5MHz and WUS should be located at edge of the NR carrier. Same as IBB and OBB, we prefer to use the same interference level as MR and add further relaxation for LP-WUR based on legacy REFSENS+6dB.

Proposal 2: Consider the test parameters for intermodulation of LP-WUR in Table 3-1 and 3-2. For these test parameters, the missed detection rate shall be ≤1%.

Table 3-1  Wide band intermodulation parameters for Type 1 LP-WUR

| Rx parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| Pw in LP-WUS Transmission Bandwidth Configuration, per CC | dBm | REFSENSType1_LR + 15dB |
| PInterferer 1 (CW) | dBm | -46 |
| PInterferer 2 (Modulated) | dBm | -46 |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset) | MHz | -BWChannel/2 – 7.5 / +BWChannel/2 + 7.5 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 1: LP-WUS is configured at the center of the NR carrier without adjacent sub-carrier interference. |  |  |

Table 3-2  Wide band intermodulation parameters for Type 2 LP-WUR

| Rx parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| Pw in LP-WUS Transmission Bandwidth Configuration, per CC | dBm | REFSENSType2_LR + 9dB |
| PInterferer 1 (CW) | dBm | -46 |
| PInterferer 2 (Modulated) | dBm | -46 |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset) | MHz | -BWChannel/2 – 7.5 / +BWChannel/2 + 7.5 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 1: LP-WUS is configured at the center of the NR carrier without adjacent sub-carrier interference. |  |  |

## 2.3 Spurious Response

Spurious response is also used to evaluate the receiver capability when there is presence of an unwanted CW interfering signal on the frequency where out-of-band blocking limit is not met. Spurious response requirement is needed for LP-WUR. Same as IBB and OBB, , we prefer to use the same interference level as MR and add further relaxation for LP-WUR based on legacy REFSENS+6dB.

Proposal 2: Consider the test parameters for spurious response of LP-WUR in Table 4-1, 4-2 and 4-3. For these test parameters, the missed detection rate shall be ≤1%.

Table 4-1  Spurious response parameters for Type 1 LP-WUR

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS transmission bandwidth configuration | dBm | REFSENSType1_LR + 15dB |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier without adjacent sub-carrier interference. |  |  |

Table 4-2  Spurious response parameters for Type 2 LP-WUR

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS transmission bandwidth configuration | dBm | REFSENSType2_LR + 9dB |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier without adjacent sub-carrier interference. |  |  |

Table 4-3  Spurious response for LP-WUR

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

- Conclusion

Proposal1: Consider the test parameters for IBB of LP-WUR in Table 1-1, 1-2, 1-3, and the test parameters for OBB of LP-WUR in Table 2-1, 2-2, 2-3. For these test parameters, the missed detection rate shall be ≤1%.

Proposal 2: Consider the test parameters for intermodulation of LP-WUR in Table 3-1 and 3-2. For these test parameters, the missed detection rate shall be ≤1%.

Proposal 3: Consider the test parameters for spurious response of LP-WUR in Table 4-1, 4-2 and 4-3. For these test parameters, the missed detection rate shall be ≤1%.

- Reference

- R4-2507936 WF for [115][136] NR_LPWUS_UERF, Vivo, RAN4#115, St. Julian’s, Malta, May 2025.