# 3GPP TSG-RAN WG4 Meeting #116                                 R4-2510403

# Bengaluru, India, 25th-29th Aug., 2025

# Source:        ZTE Corporation, Sanechips

# Title:           TP to TR 38.774 on other Rx requirements (Clause 7.1.9)

# Agenda Item:   7.24.2

# Document for:  Approval

- Introduction

In this contribution, we provided a text proposal for TR 38.774 on Clause 7.1.9 on other Rx requirements for LP-WUR based on the WF and contributions in last meeting [1][2][3][4].

- Reference

- R4-2507936 WF for [115][136] NR_LPWUS_UERF, Vivo, RAN4#115, St. Julian’s, Malta, May 2025.

- R4-2505519 Further consideration on UE RF other Rx requirements for Rel-19 LP-WUS, Huawei,  RAN4#115, St. Julian’s, Malta, May 2025.

- R4-2506116 Other RX requirements for LP-WUR, Nokia, RAN4#115, St. Julian’s, Malta, May 2025.

- R4-2506316 Discussion on receiver characteristics for LP-WUR, ZTE, RAN4#115, St. Julian’s, Malta, May 2025.

- Text Proposal

<Start of Text Proposal>

### 7.1.9	Other Rx requirements

Other Rx requirements for LP-WUR include spurious response, intermodulation requirements, and spurious emission.

Spurious response is a measure of the ability of the receiver to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency for which a response is obtained, i.e. for which the out-of-band blocking limit is not met. The source of generating spurious response includes mirror frequency interference, intermediate frequency interference, high-order mixing products of mixers, and other Nyquist zone aliasing of ADCs that happen to fall within the receiving band. These kinds of interference cannot be processed by the receiver, thus cause a decrease in sensitivity. The only way to address this is by selecting appropriate intermediate frequency points, ADC sampling frequencies, and mixers with high-order suppression to reduce the number of spurious response points.

Spurious response reflects situations where the UE receiver architecture generates response points that cannot be adequately processed by the receiver. Blocking requirements cannot pass the test on these scattered points, therefore, spurious response can serve as a remedial measure for blocking tests. Therefore, when encountering spurious response, similar relaxation on LP-WUS power as OBB should be considered. The MDR for LP-WUS shall be ≤ 1% with the reference measurement channels as specified in Annex A.3M in TS38.101-1 with the parameters for LP-WUS as specified in Table 7.1.9-1 and Table 7.1.9-2 for Type 1 LP-WUR and Type 2 LP-WUR, respectively, and with the parameters for interferer as specified in Table 7.1.9-3.

Table 7.1.9-1  Spurious response parameters for Type 1 LP-WUR

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS transmission bandwidth configuration | dBm | REFSENSType1_LR + 15dB |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier without adjacent sub-carrier interference. |  |  |

Table 7.1.9-2  Spurious response parameters for Type 2 LP-WUR

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS transmission bandwidth configuration | dBm | REFSENSType2_LR + 9dB |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier without adjacent sub-carrier interference. |  |  |

Table 7.1.9-3  Spurious response for LP-WUR

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal. Considering that this requirement is designed to verify the linearity of the Rx chain, similar to the analysis of blocking requirements, the implementation of LR, specifically the linearity of the LR, may not be as good as that of MR. Therefore, the power of LP-WUS should be further relaxed based on legacy REFSENS+6dB for MR. The MDR for LP-WUS shall be ≤ 1% with the reference measurement channels as specified in Annex A.3M in TS38.101-1 with the parameters specified in Table 7.1.9-4 and Table 7.1.9-5 for Type 1 LP-WUR and Type 2 LP-WUR, respectively.

Table 7.1.9-4  Wide band intermodulation parameters for Type 1 LP-WUR

| Rx parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| Pw in LP-WUS Transmission Bandwidth Configuration, per CC | dBm | REFSENSType1_LR + 15dB |
| PInterferer 1 (CW) | dBm | -46 |
| PInterferer 2 (Modulated) | dBm | -46 |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset) | MHz | -BWChannel/2 – 7.5 / +BWChannel/2 + 7.5 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 1: LP-WUS is configured at the center of the NR carrier without adjacent sub-carrier interference. |  |  |

Table 7.1.9-5  Wide band intermodulation parameters for Type 2 LP-WUR

| Rx parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11 |
| Pw in LP-WUS Transmission Bandwidth Configuration, per CC | dBm | REFSENSType2_LR + 9dB |
| PInterferer 1 (CW) | dBm | -46 |
| PInterferer 2 (Modulated) | dBm | -46 |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset) | MHz | -BWChannel/2 – 7.5 / +BWChannel/2 + 7.5 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 1: LP-WUS is configured at the center of the NR carrier without adjacent sub-carrier interference. |  |  |

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector. For receiver, the emissions at the antenna connector usually come from the reverse LO leakage. As illustrated in Figure 7.1.9-1 marked with the green line, the LO leakage ends at the antenna or antenna connector. The main LO frequency will dominate in-band spurious emissions, and the harmonic will dominate out-of-band spurious emissions. In a design with potential low isolation from LO to RF input, an LNA is required to attenuate the in-band spurious, and a band pass filter is required to ensure that out-of-band spurious is suppressed. Since the emissions could be considered as kind of regulatory requirements, the same levels for MR should also be defined for LR. Namely, the spurious emissions as specified in clause 7.9 in TS38.101-1 still applies.

Figure 7.1.9-1  Illustration of a DC receiver

<End of Text Proposal>