3GPP TSG-RAN WG4 Meeting #116	R42510411

Bengaluru, India, Aug. 2025

Source: 	Qualcomm

Title: 	General aspects regarding the 4940–4990 MHz band

Agenda Item:	6.14.1

Document for:	Approval

# Introduction

A new Rel-20 WI [1] was created to ‘allow for public safety operation in line with the framework identified by the FCC within the frequency range, 4940-4990 MHz, within 5G NR’, for which ‘a new NR TDD operating band needs to be defined’. In this contribution, we survey the regulatory landscape and propose a foundation for future work.

# Discussion

## Background

Since 2002, the 4940–4990 MHz band (‘US4.9’) has been reserved in the US for public safety, primarily broadband applications that are related to the protection of life, health or property. To enhance the utilization of the US4.9 band, FCC is facilitating the integration of the latest commercially available technologies, including 5G to the band [2]. Accordingly, the scope of the work in [1] is to define a new NR TDD operating band in the 4940-4990 MHz to be deployed in the US. It is noteworthy to mention that the proposed band US4.9 is a subset of the already defined C-band n79 4400-5000 MHz.

Observation 1: The proposed NR TDD band in the 4940-4900MHz is a subset of the already defined C-band n79.

## Regulations for US4.9

### Existing FCC requirements

The US4.9 band already has public safety related deployments in the US, and the FCC has established regulations for other networks to operate in that band. The current FCC technical requirements for the 4.9GHz band are therefore key to any work in RAN4. In this regard, relevant technical requirements from the FCC for the 4.9GHz are listed as follows:

- FCC § 90.1213 - Band plan

- FCC § 90.1215 - Power limits

- FCC § 90.210 - Emission masks

Proposal 1: RAN4 to consider existing FCC technical requirements for the 4940-4990 MHz throughout the work item.

### Band plan

The FCC defines in § 90.1213 [3] the band plan and possible channel bandwidth arrangements for the 4.9GHz band. The FCC defines 18 channels, where ‘Channel numbers 1 through 5 and 14 through 18 are 1 MHz bandwidth channels, and channel numbers 6 through 13 are 5 MHz bandwidth channels. Note that the FCC allows aggregation of channels up to 50 MHz to support wider bandwidth operation. The WID targets enabling 10, 15, 20, 25, 30, 40 and 50 MHz channels, which is consistent with FCC rules for US4.9.

Observation 1: The FCC provides a band plan in § 90.1213 which allows up to 50 MHz channels. The target channel BWs in the WID (10, 15, 20, 25, 30, 40 and 50 MHz) are consistent with the FCC band plan.

### Power limits

The FCC defines in § 90.1215 [4] transmission power limits in three ways: conducted, PSD and max. EIRP.

The conducted power (closely correlated with TRP) limits apply to ‘base, mobile, and temporary fixed operations’ and are reproduced in Table 1. The conducted power limit is effectively an average PSD limit, since allowed transmission power scales with bandwidth.

Table 1 FCC maximum conducted power limits for 4.9GHz stations

| CHBW (MHz) | Low power max conducted power (dBm) | High power max Conducted power (dBm) |
| --- | --- | --- |
| 1 | 7 | 20 |
| 5 | 14 | 27 |
| 10 | 17 | 30 |
| 15 | 18.8 | 31.8 |
| 20 | 20 | 33 |
| 30 | 21.8 | 34.8 |
| 40 | 23 | 36 |
| 50 | 24 | 37 |

PSD limits are separately applicable, with peak PSD limited to 21 dBm/MHz and 8 dBm/MHz, for high- and low-power devices respectively (as defined in table1). The peak PSD limit allows for a certain degree of non-flatness of the Tx spectrum when transmitting at close to the conducted power limit.

An EIRP limit is applicable due to the requirement that states, ‘If transmitting antennas of directional gain greater than 9 dBi are used, both the maximum conducted output power and the peak power spectral density should be reduced by the amount in decibels that the directional gain of the antenna exceeds 9 dBi’. Effectively it means that an antenna of greater than 9 dBi gain cannot be paired with an applicable device transmitting at the conducted limits in table 1.

These limits are analyzed for impact on UEs and gNBs in a later section.

### Emissions mask

The FCC defines in § 90.210 [5] the transmitter emission masks L (for low power transmitters, 20 dBm or less) and M (for high power transmitters, greater that 20 dBm) operating in the 4940-4990 MHz. The masks provide the emissions attenuation levels as a function of frequency offset from the centre frequency. A visualization of both masks for 10 MHz and 20 MHz channel bandwidths is given in Figure 1 below.

Figure 1 Example of 10 MHz and 20 MHz CHBW FCC emission masks for the 4.9GHz

Observation 2: For legacy UE power classes excluding PC5, and for gNBs, FCC mask ‘M’ would apply for the proposed 4.9 GHz band.

This SEM is analyzed for impact on UEs and gNBs in a later section.

## Impact to 3GPP RF requirements

### General

The WID sets out this objective: ‘Specify the UE RF requirements for the new bands. Reuse NR band n79 UE hardware assumption if possible’. From a UE perspective, since US4.9 is a subset of n79, reuse of hardware is possible, and a valid assumption. The transmit side requirements may impose new challenges (owing to different SEM) but that can be handled separately, for example by allowing sufficient PA back-off uniquely for US4.9. For the gNB, RAN4 might need to discuss how to incorporate the emission mask, highlighted above, into existing or new OBUE requirements.

Proposal 2: RAN4 to confirm assumption of reuse of n79 hardware for US4.9

### Output power

Regarding maximum output power, a TDD PC2 UE is allowed to transmit up to 28.5 dBm with Rel-18 boosting (26.0 +/2.0 + 0.5 dB). The narrowest channel BW in the WID is 10 MHz, for which the FCC allows a max. conducted power of 30.0 dBm (see section 2.2.3). This suggests that PC2 can be enabled for all target channel bandwidths for US4.9. Similarly, PC1.5 can be enabled for a min. chan. BW of 15 MHz with no boosting, and 20 MHz while making room for boosting should it become applicable for PC1.5 also. PC1 (31 dBm +/-2 dB) can be enabled for 20 MHz and wider channels (33 dBm max.). Note that UEs are not expected to have n79 antenna with gains approaching the regulatory 9 dBi limit, so no throttling of conducted power seems necessary.

Proposal 3: RAN4 to enable PC3, PC2, PC1.5 and PC1 UEs for US4.9

Proposal 4: RAN4 to discuss mechanisms to limit UE transmit power to comply with FCC conducted power limit (which is effectively an average conducted PSD limit). For example, PC 1.5 must be limited to 28 dBm +/- 2 dB for 10 MHz channels.

The 9 dBi regulatory gain limit while not significant for UEs is relevant for gNBs, since the peak EIRP would be limited to (for example) 46 dBm for 50 MHz channels. It is important to note that FCC also considers the case of high power transmissions as follows:

However, high power point-to-point and point-to-multipoint operations (both fixed and temporary-fixed rapid deployment) may employ transmitting antennas with directional gain up to 26 dBi without any corresponding reduction in the maximum conducted output power or spectral density. Corresponding reduction in the maximum conducted output power and peak power spectral density should be the amount in decibels that the directional gain of the antenna exceeds 26 dBi.

Given that 3GPP RAN4 specifies either conducted power limits (BS type 1-C or 1-H) or TRP limits (BS type 1-O), RAN4 might need to further discuss how the abovementioned EIRP limits could be captured in the spec. Accordingly, it can be considered that NR base stations falls under the above category of transmitters, and thus, the maximum antenna gain that could be considered is 26 dBi instead of the 9dBi mentioned earlier.

Proposal 5: RAN4 to discuss how to capture gNB peak EIRP limit as derived from the FCC conducted power limits while considering the different options for the maximum gain assumed by the FCC.

### Unwanted emissions

In section 2.2.4, FCC mask ‘M’, for high power devices, was identified as the likely applicable mask for UEs and gNBs in US4.9. The particulars of the FCC mask are specified thusly: ‘The zero dB reference is measured relative to the highest average power of the fundamental emission measured across the designated channel bandwidth using a resolution bandwidth of at least one percent of the occupied bandwidth of the fundamental emission and a video bandwidth of 30 kHz.’

Observation 3: the FCC mask reference (0 dB) is defined by the max. conducted power of UE or BS and the channel bandwidth.

Observation 4: The FCC wording leaves open the choice of res. BW provided it is greater than 1% of occupied BW.

Note that the minimum 1% BW stipulation for res. BW in the FCC mask is consistent with the 3GPP SEM for channel BWs < = 45 MHz. i.e. the FCC masks can be evaluated with the same resolution BWs as the 3GPP SEM. Consequently, for these channel bandwidths, the relative stringency of the FCC 4.9 and legacy 3GPP masks can be checked with ease, see figure 2.4.3-1 and -2.

Figure 2.4.3-1: SEM for 10 MHz channels

Figure 2.4.3-1: SEM for 10 MHz channels

Figure 2.4.3-2: SEM for 45 MHz channels

Figure 2.4.3-2: SEM for 45 MHz channels

For the 10-45 MHz channels proposed for US 4.9, the applicable FCC SEM masks are more stringent that the legacy 3GPP masks.

For 50 MHz channels, in the ‘sliver’ region where Foob < 1MHz, the FCC mask cannot be directly compared to the legacy SEM because the FCC mandated minimum res. BW (500 kHz) is much higher than the res. BW of the legacy 3GPP SEM (30 kHz). It is however possible to translate the 3GPP SEM to a wider res. BW (only scaling in the reverse direction is problematic). This comparison is in figure 2.4.3-3. The observations about the relative stringency of the SEMs for 50 MHz channels mirror those for 10-45 MHz channels:

Observation 5: The US4.9 FCC SEM (regulatory requirement) is more stringent than the legacy 3GPP SEM

Figure 2.4.3-3: SEM for 50 MHz channels

Figure 2.4.3-3: SEM for 50 MHz channels

Since compliance with the legacy 3GPP SEM poses no additional burden for a UE designed to comply with the US4.9 FCC SEM requirement, the most expedient standards route is to use the legacy NS framework. The FCC SEM would be defined as an additional requirement in addition to the legacy SEM general requirement.

Proposal 6: For UEs, RAN4 to incorporate FCC US4.9 SEM (FCC part 90.210) as an additional requirement under the NS framework, i.e. in addition to the legacy 3GPP SEM requirement.

For the BS side; TS 38.104 lists in Section 6.6.4 the OBUE masks for Cat A and Cat B as well as different BS classes (i.e., wide-area, medium range, and local area). Like the exercise done above, it is important to understand how the regulatory mask M is compared to the existing RAN4 requirements (e.g, Cat A OBUE for NR bands above 1 GHz).

Proposal 7: RAN4 to discuss how to incorporate the US4.9-specific OBUE regulation for gNBs.

### 2.4.4 	CA operation

In the US, n79 is not available for mobile use in its entirety. US4.9 is motivated by a desire to exploit spectrum overlapping with n79 for non-public safety operations. Consequently, there should be no occasion for the UE to support inter-band CA between n79 and US4.9.

Proposal 8: There is no need for UEs to support inter-band CA between n79 and US4.9.

The max. available spectrum in US4.9 is 50 MHz, which can be serviced by a single channel. There is hence no need to define intra-band CA in US4.9

Proposal 9: Intra-band CA in US4.9 is not enabled for Rel-20.

# 3. Conclusion

In this paper, we have shared our preliminary views on the new WI to introduce new 4.9GHZ NR TDD band for US operation. In summary, our observations and proposals can be summarized as follows:

Proposal 1: RAN4 to consider existing FCC technical requirements for the 4940-4990 MHz throughout the work item.

Observation 1: The FCC provides a band plan in § 90.1213 which allows up to 50 MHz channels. The target channel BWs in the WID (10, 15, 20, 25, 30, 40 and 50 MHz) are consistent with the FCC band plan.

Observation 2: For legacy UE power classes excluding PC5, and for gNBs, FCC mask ‘M’ would apply for the proposed 4.9 GHz band.

Proposal 2: RAN4 to confirm assumption of reuse of n79 hardware for US4.9

Proposal 3: RAN4 to enable PC3, PC2, PC1.5 and PC1 UEs for US4.9

Proposal 4: RAN4 to discuss mechanisms to limit UE transmit power to comply with FCC conducted power limit (which is effectively an average conducted PSD limit). For example, PC 1.5 must be limited to 28 dBm +/- 2 dB for 10 MHz channels.

Proposal 5: RAN4 to discuss how to capture gNB peak EIRP limit as derived from the FCC conducted power limits while considering the different options for the maximum gain assumed by the FCC.

Observation 3: the FCC mask reference (0 dB) is defined by the max. conducted power of UE or BS and the channel bandwidth.

Observation 4: The FCC wording leaves open the choice of res. BW provided it is greater than 1% of occupied BW.

Observation 5: The US4.9 FCC SEM (regulatory requirement) is more stringent than the legacy 3GPP SEM

Proposal 6: For UEs, RAN4 to incorporate FCC US4.9 SEM (FCC part 90.210) as an additional requirement under the NS framework, i.e. in addition to the legacy 3GPP SEM requirement.

Proposal 7: RAN4 to discuss how to incorporate the US4.9-specific OBUE regulation for gNBs.

Proposal 8: There is no need for UEs to support inter-band CA between n79 and US4.9.

Proposal 9: Intra-band CA in US4.9 is not enabled for Rel-20.

# 4.	References

[1] 	RP-251887, ‘New WID on Introduction of NR TDD 4.9GHz Band for US Operation’, RAN#108.

[2]		Federal Communications Commission. (2024). Eighth Report and Order: Amendment of Part 90 of the Commission’s Rules (FCC 24-114). link

[3]		47 CFR § 90.1213 – Band plan, eCFR :: 47 CFR 90.1213 -- Band plan.

[4]		47 CFR § 90.1215 – Power limits, eCFR :: 47 CFR 90.1215 -- Power limits.

[5]		47 CFR § 90.210 – Emission Masks, eCFR :: 47 CFR 90.210 -- Emission masks.

[6]		47 CFR § 90.210 – Emission Masks, eCFR :: 47 CFR 90.1217 -- 4.9 GHz Band Manager.