3GPP TSG-RAN WG4 Meeting #116	R4-2510412

Bengaluru, India, Aug. 2025

Source: 	Qualcomm

Title: 	On adjacent spectrum protection requirements for the 4940-4990 MHz band

Agenda Item:	6.14.2.1

Document for:	Approval

# Introduction

A new Rel-20 WI [1] was created to ‘allow for public safety operation in line with the framework identified by the FCC within the frequency range, 4940-4990 MHz, within 5G NR’, for which ‘a new NR TDD operating band needs to be defined’. In this contribution, we share our views on the adjacent spectrum protection for the US 4.9GHz operation.

# Discussion

## Background

Since 2002, the 4940–4990 MHz band (‘US4.9’) has been reserved in the US for public safety, primarily broadband applications that are related to the protection of life, health or property. To enhance the utilization of the US4.9 band, FCC is facilitating the integration of the latest commercially available technologies, including 5G to the band [2]. Accordingly, the scope of the work in [1] is to define a new NR TDD operating band in the 4940-4990 MHz to be deployed in the US. It is noteworthy to mention that the proposed band US4.9 is a subset of the already defined C-band n79 4400-5000 MHz.

## Coexistence with non-3GPP incumbents

3GPP relies on regional regulation for coexistence compliance with non-3GPP networks or systems. Studying coexistence with non-3GPP incumbents is outside the scope of 3GPP. In a more practical consideration, how would RAN4 decide what level of interference to incumbents (different tech.) is acceptable and what is not? This type of determination is typically conducted by the regulatory body.

As such, no further 3GPP work is required beyond adhering to local regulation. For US4.9,

Note that there may be amendments to the regulation in the future, and RAN4 would need to adapt to stay consistent. It is also useful to note that the FCC themselves have instituted a ‘band manager’ with broad jurisdiction including geographical coordination [2] for the express purpose of facilitating US4.9 non-public-safety-operations deployments.

Proposal 1: No 3GPP work beyond incorporating local regulation is required to address co-existence with non 3GPP out-of-band incumbents.

# 3. Conclusion

In this paper, we have shared our preliminary views on the new WI to introduce new 4.9GHZ NR TDD band for US operation. In summary, our observations and proposals can be summarized as follows:

Proposal 1: No 3GPP work beyond incorporating local regulation is required to address co-existence with non 3GPP out-of-band incumbents.

# 4.	References

[1] 	RP-251887, ‘New WID on Introduction of NR TDD 4.9GHz Band for US Operation’, RAN#108.

[2]		Federal Communications Commission. (2024). Eighth Report and Order: Amendment of Part 90 of the Commission’s Rules (FCC 24-114). link