3GPP TSG-RAN4 Meeting #116	R4-2510415

# Bengaluru, India 25th August 2025- 29th August 2025

Title:	Views on BS demodulation requirements for MMSE-IRC

Source:	Qualcomm Incorporated

Agenda item:	7.13.4

Document for:	Approval

# 1.	Introduction

During RAN#103 plenary a new study item on BS advanced receiver performance was agreed in [1]. One of the objectives of the WI is as follows:

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In [2], a WF was agreed regarding simulation parameters, scenarios and that are the basis for the demodulation requirements to be defined in this work item. In this contribution, we provide our views on the open items captured in [2].

# 2.1	General

## 2.1.1	MMSE-IRC receiver assumptions

In [2], the following was captured:

MMSE-IRC receiver assumption - estimation of interference covariance matrix - RAN4 agree that MMSE-IRC receiver assumption is not going to be captured in the RAN4 spec - There is no consensus whether to modify the previously agreed MMSE-IRC receiver assumption.

Based on the discussion during RAN4#115, companies had different views on how to construct and estimate the interference covariance matrix for the MMSE-IRC receiver. It has been generally discussed that potential power mismatch between data and DMRS REs might exist, and the main proposal was to revise the MMSE-IRC baseline assumption to enforce the estimation of the interference covariance matrix based on the DMRS symbol (data and DMRS REs). However, several companies objected to have such restriction/ assumption agreed when deriving the BS demodulation requirement for two reasons. First, RAN4 has already agreed that the MMSE-IRC assumptions and algorithm will not be captured in the spec, but this is to ensure aligned simulation assumptions. Secondly, several companies echoed that this is an implementation-detailed aspect, and it is better to leave it up to companies’ preferences.

Observation 1: Given that no consensus is reached on the estimation of the interference covariance matrix, it is a viable way forward to leave it up to companies’ preference.

### 2.1.2	Manufacturer declarations and applicability rules

The following aspects were agreed in [2]:

Whether to consider additional applicability rule on how the test is performed - Option 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped - Option 2: Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

In our views, given that additional applicability rules were not considered for LTE, it is well justified the necessity to introduce such additional rules to NR.  Accordingly, option 2 would be our preference in case no concerns were raised during RAN4#116.

Proposal 1: Preference to not consider additional applicability rule for different test scenarios or BS classes.

# Conclusion

In this contribution we provide our views on the remaining issues of BS demodulation aspects for MMSE-IRC receiver. Our observations and proposals can be summarized as follows:

Observation 1: Given that no consensus is reached on the estimation of the interference covariance matrix, it is a viable way forward to leave it up to companies’ preference.

Proposal 1: Preference to not consider additional applicability rule for different test scenarios or BS classes.

# Reference

[1]		RP-240831, “New WID: WI resulting from discussion on Demodulation performance requirements enhancements”, Vivo, RAN#103, Maastricht, Netherlands.

[2] 	R4-2508618, “Way Forward for [115][322] NR_demod_Ph5_Part1_General_BS”, RAN4#115, China Telecom.