3GPP TSG-RAN4 Meeting #116	R4-2510419

# Bengaluru, India 25th – 29th August 2025

Title:	Views on UE demodulation requirements for SBFD

Source:	Qualcomm Incorporated

Agenda item:	7.21.6

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. The objective of the performance part of the Rel-19 work item is as follows:

- Specify necessary BS and/or UE demodulation performance requirements to support SBFD operation.

In [2], a WF was agreed capturing agreements and potential future directions for BS and UE demod aspects for Rel-19 NR duplex operation WI. In this contribution we share our views based on agreements in [2] on the demodulation requirements for both UE and base station components for the Rel-19 SBFD WI within RAN4.

# UE demodulation requirements for SBFD-aware UE

In [2], the following was agreed:

- Not to introduce UE demodulation test related with inter-UE CLI for Rel-19 SBFD-aware UEs - Not to introduce UE CSI reporting requirement for Rel-19 SBFD-aware UEs - Do not introduce UE demodulation requirement with repetition transmission cases for SBFD-aware UE

In addition, the following issue was captured as to be further discussed:

Issue 2-1-3 whether to introduce UE demodulation with SBFD carrier (with DL sub-band and UL sub-suband configured) - Way forward - Option 1: Define limited number of additional SBFD cases - Other options are not precluded

It is noteworthy to highlight that an SBFD-aware UE (i.e., UE only doing half-duplex operation in Rel-19 Duplex evolution WI) is simply a legacy TDD UE that is capable to transmit and receive on UL and DL subbands, respectively, from its SBFD-capable Rel-19 BS. In other words, legacy UE demodulation requirements apply and from a demodulation  point of view, no additional requirements are envisioned to be set for such a device. In contrast to the SBFD-capable BS, where additional sources of interference are present (e.g., self-interference, inter-sector interference), it is expected that new demodulation requirements are proposed in Rel-19 to ensure that a given level of demodulation performance is maintained.

Observation 1: From a demodulation point of view, no difference exists between SBFD-aware UE and legacy TDD UE.

In this regard, we do not see the necessity to introduce new test cases for SBFD-aware UEs or with SBFD carrier(i.e.,  SBFD operation) for legacy UEs, as it provides no additional insights compared to existing tests for legacy TDD UE demodulation requirements.

Proposal 1: RAN4 not to introduce UE demodulation requirements or new test cases for SBFD operation for SBFD-aware UEs.

# Conclusion

In this contribution, we provided our views on UE demodulation requirements with the following observation and proposal:

Observation 1: From a demodulation point of view, no difference exists between SBFD-aware UE and legacy TDD UE.

Proposal 1: RAN4 not to introduce UE demodulation requirements or new test cases for SBFD operation for SBFD-aware UEs.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

[2]       R4-2508723, “Way Forward for [115][330] NR_duplex_evo_demod”, Samsung, RAN4#115.