3GPP TSG-RAN WG4 Meeting #116	R4-2510422

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

Agenda item:	7.13.3

Document for:	Discussion

Introduction

In the last RAN4 meeting, several agreements were made resulting in good progress. The remaining issues to be discussed are

- The definition of MMSE-IRC receiver capability for 8Rx UE

- The configuration for CQI requirements.

In this contribution we will further discuss the remaining open issues and provide our observations and proposals.

Discussion

General

Definition of MMSE-IRC receiver capability for 8Rx UE Options: - Option 1: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 2: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 3: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 4: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 5: Re-use existing definition in Rel-18.

For reference we will recite the existing receiver capability definition in 38.101-4 [2] which is represented in Option 5 here:

Baseline SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. Simplified SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

In the discussions, the main issue has been the indication of “SU-MIMO” in the definition, as the definition was made for 8Rx without interference in Rel-18 as well as suggestions to specify it is a MMSE-IRC receiver.

We would like to giver our view on what the term SU-MIMO receiver means:

- Only handles intra-user inter-layer interference rejection and not inter-user interference.

Based on above we see that keeping SU-MIMO would be the most straight forward solution also for 8Rx interference requirements introduced in this WID.

Effectively this topic handles only inter-user inter-layer interference rejection and not inter-user interference, hence using the term SU-MIMO still applies.

Also, we do not see the need to specifically indicate it is an MMSE-IRC receiver, as it is to our understanding the view from companies, that a receiver without interference rejection will not be able to pass the requirements which are agreed.

As the MMSE-IRC receiver will likely be the minimum receiver type capable of passing the requirements, we see no need to include MMSE-IRC in the definitions.

Based on the above observations we now do not see the need to change the existing definition as it fully covers the receiver in question.

Re-use existing definition from Rel-18 in Rel-19, i.e. no change is needed (option 5)

Wideband CQI reporting requirements with inter-cell interference

In RAN4#115 it was decided to define one set of requirements for CQI reporting. In addition, it was agreed to use -2dB SINR for requirement definition [1]:

Configuration for CQI requirements Agreement: - Use [-2dB] SINR for requirement definition. - TBD for the Gamma value. Companies are encouraged to propose the value for Gamma in the next meeting.

Based on our simulation results provided in [2] where we see a TP gain of 2.83 @ SINR=-2dBm we find it reasonable to define the Gamma value for wideband CQI reporting requirements to gamma=2.2.

Define wideband CQI reporting requirements with SINR = -2dB, Gamma = 2.2

Conclusion

The following Observations and Proposals were made:

Definition of MMSE-IRC receiver capability for 8Rx UE

- Effectively this topic handles only inter-user inter-layer interference rejection and not inter-user interference, hence using the term SU-MIMO still applies.

- As the MMSE-IRC receiver will likely be the minimum receiver type capable of passing the requirements, we see no need to include MMSE-IRC in the definitions.

- Re-use existing definition from Rel-18 in Rel-19, i.e. no change is needed (option 5)

Configuration for CQI requirements

Define wideband CQI reporting requirements with SINR = -2dB, Gamma = 2.2

- References

- R4-2508620 - WF for [115][323] NR_demod_Ph5_Part2_UE

- R4-2510423 - Simulation results for UE demodulation performance req. for 8Rx with MMSE-IRC