3GPP TSG-RAN WG4 Meeting #116	R4-2510434

Bangalore, India, 25th – 29rd August, 2025

Agenda item:	7.1.4

Source:	Samsung

Title:	Discussion on UE demodulation performance requirements for 6Rx

Document for:	Discussion

- Introduction

In the RAN#103 meeting, the WID [1] for “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” has been approved, in which “6Rx for handheld and FWA” is an important objective for both core and performance parts. This is an enhancement for receiver in addition to Rel-15 4Rx and Rel-18 8Rx improvements (which are specified for fixed wireless access (FWA) types of devices). The WID for performance part has been updated as below [2] according latest RAN4#114 meeting agreements [3].

The objectives of performance part for UE RF enhancements for NR FR1 and FR2 Phase 4 include: 6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - 6 MIMO layers performance requirements will be only defined for FWA. - 4 MIMO layers performance requirements will be defined and will apply to FWA and handheld UE devices. - Specify the SDR requirements to support 6Rx General for all the areas - Specify release independence requirements for performance part in TS 38.307/36.307, if needed and feasible.

In RAN4#114bis and RAN4#115 meetings, work plan, general scopes and some test assumptions were agreed [4][5]. In this contribution, we provide our further analysis on the open issues of 6Rx PDSCH demodulation performance and CSI requirements, and 6Rx SDR requirements.

- Discussion

  - General

In last meeting, three open issues for general part as below [5].

Issue 1-1: Test applicability rule for 6Rx tests - Proposals - Option 1: Introduce UE declaration and design proper test applicability rule in the specification as per the objective of perf part in RP-250780 if no explicit UE capability signalling to differentiate UE type introduced by RAN2. - Option 2: Introduce test applicability rule only for 6 layers cases - Option 3: Postpone the discussion after RAN2 finishes the UE capability signalling design Issue 1-2: Test applicability rule for different number Rx antenna ports - Proposals - Option 1: For UE supports 6Rx and at least one of other number of antennas, i.e., 8Rx, 4Rx, 2Rx, corresponding applicability rules should be introduced to skip some test cases with less UE antennas once the test cases with more UE antennas could pass. - Other options Issue 1-3: Specification structure - Proposals - Option 1: Add a new clause for 6Rx - Option 2: Modify the current section numbering and put 6Rx requirements between 4Rx and 8Rx in the new Rel-19 specification

Test applicability rule for different number Rx antenna ports

For UE supports 6Rx and at least one of other number of antennas, i.e., 8Rx, 4Rx, 2Rx, corresponding applicability rules should be introduced to skip some test cases with less UE antennas once the test cases with more UE antennas could pass, in order to balance the number of test cases and coverage.

Proposal 1: For UE supports 6Rx and at least one of other number of antennas, i.e., 8Rx, 4Rx, 2Rx, corresponding applicability rules should be introduced to skip some test cases with less UE antennas once the test cases with more UE antennas could pass, in order to balance the number of test cases and coverage as following.

| Supported RX antenna ports | Test type | Test list |  |
| --- | --- | --- | --- |
| UE supports 6Rx, 4Rx and 2Rx | PDSCH | All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.X (Note 2) | If UE passes tests in Clause 5.2.X, UE can skip Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6 |
| or | PDCCH | All tests in Clause 5.3.3 (Note 2,3) |  |
| UE supports both 6Rx and 4Rx | PBCH | All tests in Clause 5.4.3 (Note 1) |  |
| UE supports both 6Rx and 2Rx | PDSCH | All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.X (Note 2) |  |
|  | PDCCH | All tests in Clause 5.3.2 (Note 2,4) |  |
|  | PBCH | All tests in Clause 5.4.2 |  |
| UE supports 8Rx, 6Rx, 4Rx and 2Rx or UE supports both 8Rx, 6Rx, and 4Rx | PDSCH | All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.4 (Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6 |
|  | PDCCH | All tests in Clause 5.3.3 (Note 2,3) |  |
|  | PBCH | All tests in Clause 5.4.3 (Note 1) |  |
| UE supports both 8Rx, 6Rx and 2Rx | PDSCH | All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.4 (Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4 |
|  | PDCCH | All tests in Clause 5.3.2 (Note 2,4) |  |
|  | PBCH | All tests in Clause 5.4.2 |  |
| UE supports only 8Rx Or UE supports both 8Rx and 6Rx | PDSCH | All tests in Clause 5.2.4 (Note 2) |  |
|  | PDCCH | N/A |  |
|  | PBCH | N/A |  |
| UE supports only 6Rx | PDSCH | All tests in Clause 5.2.X (Note 2) |  |
|  | PDCCH | N/A |  |
|  | PBCH | N/A |  |
| Note 1: Requirements for PBCH with 4Rx is up to UE declaration Note 2: ‘maxMIMO-Layers-r16’ is not configured during the performance requirements testing for UE supporting Release 16 per-BWP MIMO layer adaptation. Note 3: 8Rx or 6Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx or 6 Rx with data source from system simulator, and the other 4 Rx or 2 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 4: 8Rx or 6Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx or 6 Rx with data source from system simulator, and the other 6 Rx or 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied. Note 5: Tests defined for 8Rx capable UEs according to whether the UE is a ‘simplified 8Rx receiver’ or ‘baseline 8Rx receiver’ are specified in Table 5.1.1.3-1. |  |  |  |

Specification structure

It’s no doubt a new sub-clause for 6Rx should be introduced for 6Rx performance requirements. About the position of this new sub-clause, it would be perfect if it could be between 4Rx and 8Rx in the new Rel-19 specification.

Proposal 2: Support introducing a new sub-clause for 6Rx and support putting it between 4Rx and 8Rx in the new Rel-19 specification if possible.

  - PDSCH demodulation performance requirements

The agreement for introducing 6Rx PDSCH demodulation performance requirements for FR1 TDD single carrier has been achieved. Therefore, related test assumptions and parameters should be discussed and aligned.

    - Performance requirements for 4 Layers

MIMO correlation for 4 MIMO layers for Handheld UE testing

This issue also has relationship with the number of test cases for 4 MIMO layers. If separate test cases for 4 MIMO layers for FWA and Handheld UE will be defined, we prefer to introduce certain correlation for handheld UE test. However, since this test is for 4Tx with 4 layers case, if the correlation is too high, it may be difficult to achieve peak throughput or even hard to achieve 70% maximum throughput. We evaluate the option 1 (with α = 0 and β = 0.07179), option 2 (with α = 0.3 and β = 0.07179) and legacy Medium B (with α = 0.3 and β = 0.005154) in our companion contribution [6]. From the initial simulation results, we have below observations:

Observation 1: The ideal SNR working point at 70% maximum throughput for option 1 (with α = 0 and β = 0.07179) is 18.0dB, for legacy Medium B (with α = 0.3 and β = 0.005154) is 26.6dB, and could not reach 70% maximum throughput for option 2 (α = 0.3 and β = 0.07179).

Therefore, if separate test cases for 4 MIMO layers for FWA and Handheld UE will be defined, we prefer to use ULA Low for FWA and ULA Medium (alpha =0, beta=0.07179) for Handheld UE. If only one test case will be defined for 4 MIMO layers for FWA and Handheld UE, we prefer to use ULA Low.

Proposal 3: If separate test cases for 4 MIMO layers for FWA and Handheld UE will be defined, we prefer to use ULA Low for FWA and ULA Medium option 1 with (alpha =0, beta=0.07179) for Handheld UE. If only one test case will be defined for 4 MIMO layers for FWA and Handheld UE, we prefer to use ULA Low.

Test cases for 4 MIMO layers for FWA and Handheld UE

Considering FWA and Handheld UE usually have different product forms and different typical parameters, such as different antenna correlations. We think it’s more reasonable to define two separate cases for FWA and Handheld UE with 4 MIMO layers.

Proposal 4: Prefer to define two separate test cases for 4 MIMO layers for FWA and Handheld UE.

MCS for 4 MIMO layer case

For the MCS values, agreements achieved in last RAN4#115 meeting are as following

Issue 2-2-5: MCS for 4 MIMO layer case for FWA/Handheld UE with TDLA30-10 ULA Low - Agreement: Down select the following two options in the next RAN4 meeting - Option 1: MCS 20 (Table 2) - Option 2: MCS 17 (Table 1) Issue 2-2-6: MCS for 4 MIMO layer case for Handheld UE with ULA medium (If agreed) - Agreement - MCS 13 (Table 1)

For 4 MIMO layer case with TDLA30-10 ULA Low, we provide our estimation results for MCS20 (Table 2) and MCS17 (Table 1) in our companion contribution [6]. From the initial simulation results, we have below observations:

Observation 2: The ideal SNR working point at 70% maximum throughput for MCS20 (Table 2) is 22.84dB, accordingly the requirements will be about 26dB for MCS20 (Table 2).

Considering 26dB is a little bit too high for demodulation performance requirements, MCS17 (Table 1) should be selected for 4 MIMO layer case for FWA/Handheld UE with TDLA30-10 ULA Low.

Proposal 5: For 4 MIMO layer case for FWA/Handheld UE with TDLA30-10 ULA Low, MCS17 (Table 1) should be selected for PDSCH performance requirements.

    - Performance requirements for 6 MIMO Layers for FWA

Test applicability rule for FWA

In last RAN4#115 meeting, one proposal for FWA test applicability rule raised as “FWA device can skip corresponding 4 layers test if it passes the FWA 6 layers tests”, the purpose of this proposal is to reduce the testing burden. However, for the same number of Rx antennas, the cases for different layers have different test requirements and different test assumptions, it’s hard to get the conclusion that the lower layer case could pass once the higher layer case passed. At the same time, the test burden problem could be reduced by the applicability rules for different Rx antennas.

Proposal 6: For the test applicability rule for FWA, do not prefer to introduce a new rule as “FWA device can skip corresponding 4 layers test if it passes the FWA 6 layers tests”.

DMRS configuration for 6 MIMO layers case

Per our understanding and the description of DMRS configurations described in 38.211 and 38.331, DMRS type 1 with one additional OFDM symbol for DMRS ports and maxLength=1 is the typical DMRS configuration (Rel-15 1+1 DMRS symbols) for 4 MIMO layers as Figure 2-1 (a), it is straightforward to configure maxLength=2 in order to support 6 MIMO layers as Figure 2-1 (b), this is the typical configuration for real network.  For Rel-18 1+1(6L), it only could be supported when the feature of Rel-18 enhanced DMRS is supported.

38.212, DMRS ports configurations

- Rel-15 typical 4 ports configuration

- Rel-15 typical 6 ports configuration

- Rel-18 6 ports configuration

Figure 2-1. DMRS ports configurations for Rel-15 DMRS and Rel-18 DMRS

Proposal 7: For DMRS configuration for 6 MIMO layers case, use the typical configuration Rel-15 Type 1 DMRS 2+2.

Proposal 8: For DMRS ports for 6 MIMO layers case, propose to use DRMS ports {1000-1003, 1004, 1006} for test assumption.

  - SDR requirements

Static channel and precoding for 6Rx

For the static channel matrices, except the property that no interference among any pair of layers, the power of each element or the average power of the static channel should be 1. Therefore option 1 should be selected as the static channel for 8T6R.

- For 8T6R (Fixed precoding with i1,1=i1,2=i2=0): - Option 1: - H=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11 - Option 2: - H=34*11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

Proposal 9: Option 1 should be selected as the static channel for 8T6R.

- H=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

SDR requirements for 1024QAM with 4 layers

For the SDR requirements for 1024QAM with 4 layers, one proposal is to do the feasibility study using new static channel.

Issue 3-1-2: SDR requirements for 1024QAM with 4 layers - Proposals - Option 1: Not Define SDR requirements for 1024QAM with 4 layers - Option 2: FFS SDR 1024QAM 4-layer requirements after 6Rx static channels are determined

However, for 1024QAM, current spec only defining SDR requirements up to MIMO layer 2 even for 8Rx since some contributors think 1024QAM is not feasible for layer 4 with 8Rx, hence it will be strange if we introduce SDR requirements for 1024QAM with 4 layers for 6Rx but not introduced for 8Rx. Therefore, no need to do the feasibility study of 1024QAM with 4 MIMO layers for 6Rx.

Proposal 10: For the SDR requirements of 6Rx with 1024QAM, no need to do the feasibility study of 1024QAM with 4 MIMO layers.

MCS values for 1024QAM with 2 layers

According the simulation results in our companion contribution [6], MCS24 could be achieved for 1024QAM with 2 layers and scaling factor 1. the MCS values for 1024QAM with 2 layers are proposed as below.

Proposal 11: For 1024QAM, define SDR requirements for MIMO layer 2 as below.

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 6RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |

  - CSI reporting performance requirements

CQI reporting requirements

For the CQI estimation processing, there is no UE handling difference for 6Rx and 4Rx/8Rx, hence we don’t think new CQI reporting requirements are necessary for 6Rx. In current spec, CQI requirements with rank 4 under static channel are defined for 8Rx, from this point of view, it is also acceptable to define CQI requirements with rank 4 under static channel for 6Rx. No need to define wideband CQI reporting requirements with inter-cell interference under fading conditions at all.

Proposal 12: For CQI reporting requirements, prefer not to define new CQI reporting requirements for 6Rx.

RI reporting requirements

Similar reasons as above for CQI and PMI reporting requirement, we don’t prefer to define new test requirements for RI reporting for 6Rx.

Proposal 13: For RI reporting requirements, prefer not to define new RI reporting requirements for 6Rx.

- Conclusion

In this contribution, we provide our further analysis and views on the PDSCH demodulation performance, SDR requirements and CSI reporting performance requirements on “6Rx for handheld and FWA” topic.

The observations and proposals could be summarized as:

For General

Proposal 1: For UE supports 6Rx and at least one of other number of antennas, i.e., 8Rx, 4Rx, 2Rx, corresponding applicability rules should be introduced to skip some test cases with less UE antennas once the test cases with more UE antennas could pass, in order to balance the number of test cases and coverage as following.

| Supported RX antenna ports | Test type | Test list |  |
| --- | --- | --- | --- |
| UE supports 6Rx, 4Rx and 2Rx | PDSCH | All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.X (Note 2) | If UE passes tests in Clause 5.2.X, UE can skip Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6 |
| or | PDCCH | All tests in Clause 5.3.3 (Note 2,3) |  |
| UE supports both 6Rx and 4Rx | PBCH | All tests in Clause 5.4.3 (Note 1) |  |
| UE supports both 6Rx and 2Rx | PDSCH | All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.X (Note 2) |  |
|  | PDCCH | All tests in Clause 5.3.2 (Note 2,4) |  |
|  | PBCH | All tests in Clause 5.4.2 |  |
| UE supports 8Rx, 6Rx, 4Rx and 2Rx or UE supports both 8Rx, 6Rx, and 4Rx | PDSCH | All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.4 (Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6 |
|  | PDCCH | All tests in Clause 5.3.3 (Note 2,3) |  |
|  | PBCH | All tests in Clause 5.4.3 (Note 1) |  |
| UE supports both 8Rx, 6Rx and 2Rx | PDSCH | All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.4 (Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4 |
|  | PDCCH | All tests in Clause 5.3.2 (Note 2,4) |  |
|  | PBCH | All tests in Clause 5.4.2 |  |
| UE supports only 8Rx Or UE supports both 8Rx and 6Rx | PDSCH | All tests in Clause 5.2.4 (Note 2) |  |
|  | PDCCH | N/A |  |
|  | PBCH | N/A |  |
| UE supports only 6Rx | PDSCH | All tests in Clause 5.2.X (Note 2) |  |
|  | PDCCH | N/A |  |
|  | PBCH | N/A |  |
| Note 1: Requirements for PBCH with 4Rx is up to UE declaration Note 2: ‘maxMIMO-Layers-r16’ is not configured during the performance requirements testing for UE supporting Release 16 per-BWP MIMO layer adaptation. Note 3: 8Rx or 6Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx or 6 Rx with data source from system simulator, and the other 4 Rx or 2 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 4: 8Rx or 6Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx or 6 Rx with data source from system simulator, and the other 6 Rx or 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied. Note 5: Tests defined for 8Rx capable UEs according to whether the UE is a ‘simplified 8Rx receiver’ or ‘baseline 8Rx receiver’ are specified in Table 5.1.1.3-1. |  |  |  |

Proposal 2: Support introducing a new sub-clause for 6Rx and support putting it between 4Rx and 8Rx in the new Rel-19 specification if possible.

For PDSCH demodulation performance requirements

Observation 1: The ideal SNR working point at 70% maximum throughput for option 1 (with α = 0 and β = 0.07179) is 18.0dB, for legacy Medium B (with α = 0.3 and β = 0.005154) is 26.6dB, and could not reach 70% maximum throughput for option 2 (α = 0.3 and β = 0.07179).

Proposal 3: If separate test cases for 4 MIMO layers for FWA and Handheld UE will be defined, we prefer to use ULA Low for FWA and ULA Medium option 1 with (alpha =0, beta=0.07179) for Handheld UE. If only one test case will be defined for 4 MIMO layers for FWA and Handheld UE, we prefer to use ULA Low.

Proposal 4: Prefer to define two separate test cases for 4 MIMO layers for FWA and Handheld UE.

Observation 2: The ideal SNR working point at 70% maximum throughput for MCS20 (Table 2) is 22.84dB, accordingly the requirements will be about 26dB for MCS20 (Table 2).

Proposal 5: For 4 MIMO layer case for FWA/Handheld UE with TDLA30-10 ULA Low, MCS17 (Table 1) should be selected for PDSCH performance requirements.

Proposal 6: For the test applicability rule for FWA, do not prefer to introduce a new rule as “FWA device can skip corresponding 4 layers test if it passes the FWA 6 layers tests”.

Proposal 7: For DMRS configuration for 6 MIMO layers case, use the typical configuration Rel-15 Type 1 DMRS 2+2.

Proposal 8: For DMRS ports for 6 MIMO layers case, propose to use DRMS ports {1000-1003, 1004, 1006} for test assumption.

For SDR requirements

Proposal 9: Option 1 should be selected as the static channel for 8T6R.

- H=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

Proposal 10: For the SDR requirements of 6Rx with 1024QAM, no need to do the feasibility study of 1024QAM with 4 MIMO layers.

Proposal 11: For 1024QAM, define SDR requirements for MIMO layer 2 as below.

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 6RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |

For CSI reporting performance requirements

Proposal 12: For CQI reporting requirements, prefer not to define new CQI reporting requirements for 6Rx.

Proposal 13: For RI reporting requirements, prefer not to define new RI reporting requirements for 6Rx.

- Reference

[1] RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#103 meeting, Huawei

[2] RP-250780, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#107 meeting, Huawei

[3] R4-2503014, WF on 6Rx UE requirements, RAN4#114 meeting, AT&T

[4] R4-2504750, Way Forward for [114bis][325] NR_ENDC_RF_Ph4_Demod_6Rx, RAN4#114bis meeting, Huawei, HiSilicon

[5] R4-2508727, Way Forward for Topic [115][326] NR_ENDC_RF_Ph4_Demod_6Rx, RAN4#115 meeting, Huawei, HiSilicon

[6] R4-2510435, Simulation results on UE demodulation performance requirements for 6Rx, RAN4#116 meeting, Samsung