3GPP TSG-RAN WG4 Meeting #116	R4-2510437

Bangalore, India, 25th – 29rd August, 2025

Agenda item:	7.12.3

Source:	Samsung

Title:	Discussion on spatial channel model study

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the SID [1] for “New SID: Study on spatial channel model for demodulation performance requirements” has been approved, which starts from RAN4#112 in Aug 2024 and targeting to complete by RAN#109 in Sep 2025 [2]. The objectives are as below:

The study item includes the following objectives: - Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall be for FR1 (conducted).

In RAN4#113 meeting, High level TR skeleton for 38.753 and initial work split have been agreed, and obvious progresses in RAN4#114 - RAN4#115 meeting have been achieved for test cases, parameter assumptions, CDL methodologies and AAV configurations [3][4][5].

In this contribution, we share our views and analysis on open issues and potential issues for spatial channel model, and provide our simulation results for both alignment and comparison purposes in our companion contribution [7].

- Discussion

  - TR38.753 related aspects

Necessity for merging Annex B to Chapter 5.1

Currently TR38.753 separates the description of CDL-based spatial channel modeling approach into two parts:

- Chapter 5.1 primarily introduces the definition of antenna array virtualization, the configurations of antenna panel and antenna element, parameters for angle scaling and randomness reduction, and the table for UMa-CDLC1, but lacks the concrete CDL channel modeling methodology/approach.

- Annex B provides the actual channel modeling methodology/approach details, which are critical for understanding the CLD-based spatial channel model.

Accordingly, below reasons are our motivations for proposing to merge annex B to chapter 5.1.

- Logical Consistency and Readability

The title of Chapter 5.1 “CDL Approach” and even a sub-section 5.1.4 “channel models” for CDL-based spatial channel model suggest this chapter should comprehensively cover CDL modeling method(s)/approach, yet it only defines configurations/parameters/tables without detailing the core techniques.

Readers expect modeling methodologies/approaches to appear in the main body, not an annex. Separating them disrupts logical flow and necessitates cross-referencing, reducing clarity.

- Improved Usability

Per our understanding, annexes typically host supplementary material (e.g., derivations, optional details), whereas “Modelling execution for CDL Channel Models” is fundamental to the CDL-based spatial channel model study TR.

Burying key methodologies in an annex risks them being overlooked, especially by implementers who may focus on the main chapters.

- Alignment with Standardization Conventions

Many 3GPP/ITU documents integrate core methodologies into main chapters (e.g., TR38.901’s channel model descriptions). Consistency with such structure(s) aids user familiarity.

- Streamlined Maintenance

Mering reduces redundancy (e.g., repeated definitions for antenna panel configuration) and ensures updates to modelling methods are centralized in one section.

Hence, relocating Annex B’s content into chapter 5.1 would enhance coherence, readability, and practicality of TR38.753.

Observation 1: For TR38.753, the most important context of CDL-based spatial channel model are captured in Annex B, while only antenna modeling, parameters and UMa-CDLC1 table are described on the main body CDL approach section.

Proposal 1: For TR38.753, merge Annex B to chapter 5.1 to enhance the TR logical consistency, readability  and practicality.

Necessity for merging Annex C to Chapter 5.2

Similarly, the main body of TR38.753 clearly include chapter 5.2 “TDL Approaches”, which describes the modeling for TDL-based spatial channel model. Furthermore, sub-section 5.2.2.2 especially describes cluster model example. However, the contexts for annex C which named “Modelling execution for TDL Channel Models” but actually about two TDL examples. We are considering the feasibility for merging annex C to chapter 5.2 in order to avoid cross-referencing.

Observation 2: For TR38.753, sub-section 5.2.2.2 especially describes cluster model example. However, the contexts for annex C which named “Modelling execution for TDL Channel Models” but actually about two TDL examples.

Proposal 2: For TR38.753, merge Annex C to chapter 5.2 to avoid cross-referencing.

mean angle μφ,model and power weighted mean angle μφ

For the angle scaling description in sub-section 5.1.4.1 as below, μφ,model is the mean angle of the tabulated CDL from 38.901, calculated using the definition in Annex A.2 of 38.901, and the annex A.2 of 38.901 copied as following. However, except the mean angle μφ,model, we also have a separate description for power weighted mean angle μφ in sub-section 5.1.4.1, which actually is the further description/clarification about mean angle μφ,model. Hence, we think the descriptions for mean angle μφ,model and power weighted mean angle μφ should be combined together in order to avoid ambiguous statement.

38.753 sub-section 5.1.4.1 Parameters for Angle Scaling

φn,scaled=wrap(ASdesiredASmodel.wrap(φn,model-μφ,model)+μφ,model) φn,model is the tabulated CDL ray angle from 38.901 ASmodel is the rms angular spread of the tabulated CDL from 38.901 including the offset ray angles provided below μφ,model is the mean angle of the tabulated CDL from 38.901, calculated using the definition in Annex A.2 of 38.901 ASdesired is the desired rms angular spread provided below φn,scaled is the resulting scaled ray angle. y = wrap(x) makes y to fall between -180 and +180 degrees so that y = (x+180) mod 360 - 180 The power weighted mean angle is given by μφ=arg{n=1Nm=1Mexp(jφn,m)Pn,m} where Pn,m is the power for the mth ray of the nth cluster and φn,m is the ray angle (either AOA, AOD, ZOA, ZOD) given in radians.

38.901 Annex A.2 Calculation of mean angle

The power weighted mean angle is given by (A-2) where is the power for the mth subpath of the nth path and is the subpaths angle (either AOA, AOD, ZOA, ZOD) given in radians.

In addition, the power weighted mean angle in 38.901 Annex A.2 defined based on mth subpath of the nth path , however, the CDL table UMa-CDLC1 defined in TR38.753 actually comes from power weighted cluster mean angle defined in 38.901 Annex A.4. Consequently, the description should be updated accordingly.

38.901 Annex A.4 Calculation of cluster mean angle

The power weighted cluster mean angle in degrees is given by μφ=180πargexpjπ180φLOSPLOS+n=1Nexpjπ180φnPn (A-4) where arg{z} is the angle in radians measured from the positive real axis to the line joining the origin and complex value z, Pn is the power for the nth cluster path, φn is the cluster path angle (either AOA, AOD, ZOA, ZOD) given in degrees, PLOS is the power for the LOS path, and φLOS is the LOS path angle (either AOA, AOD, ZOA, ZOD) given in degrees. If LOS path does not exist, PLOS=0 is assumed.

Observation 3: For TR38.753 sub-section 5.1.4.1, it seems the description of power weighted mean angle μφ is independent with the angle scaling formula.

Proposal3: For TR38.753 sub-section 5.1.4.1, the descriptions for power weighted mean angle μφ should be updated to power weighted cluster mean angle defined in 38.901 Annex A.4.

Proposal4: For TR38.753 sub-section 5.1.4.1, the descriptions for mean angle μφ,model and power weighted mean angle μφ should be combined together in order to avoid ambiguous statement.

  - CDL Based Methodologies

Frequency Generalisation

One open issue for frequency generalisation for CDL-based spatial channel model needs further discussion.

Options Option 1: To select one typical band for FDD and TDD respectively and utilize the centre frequency of such two typical FDD band TDD band as target frequency to derive desired angular spread and doppler. The two scaled CDL models can represent TDD and FDD scenario respectively for requirements definition. Option 2: Use 3.5GHz for all FR1 bands and duplexing, following legacy and SCM alignment.

According the descriptions in 38.901, for link level CDL model,

- The final scaled azimuth/zenith angles could be impacted by the tabulated AoA/DoA/ZoA/ZoD angles, AS desired, AS model, which are not differentiated in current spec.

- The final scaled delays can be obtained according to the equation τn,scaled=τn,model⋅DSdesired, in which τn,model are the same for FR1 FDD and FR1 TDD, and the delay scaling factor DSdesired seems could vary with different carrier frequencies. However, if we focus on the normal delay profile as below table defined in 38.901, there are no difference for FR1 FDD and FR1 TDD.

38.901 Table 7.7.3-2

Table 7.7.3-2. Scenario specific scaling factors - for information only Proposed Scaling Factor in [ns]; Frequency [GHz]. 2; 6; 15; 28; 39; 60; 70. Indoor office; Short-delay profile; 20; 16; 16; 16; 16; 16; 16. Normal-delay profile; 39; 30; 24; 20; 18; 16; 16. Long-delay profile; 59; 53; 47; 43; 41; 38; 37. UMi Street-canyon; Short-delay profile; 65; 45; 37; 32; 30; 27; 26. Normal-delay profile; 129; 93; 76; 66; 61; 55; 53. Long-delay profile; 634; 316; 307; 301; 297; 293; 291. UMa; Short-delay profile; 93; 93; 85; 80; 78; 75; 74. Normal-delay profile; 363; 363; 302; 266; 249; 228; 221. Long-delay profile; 1148; 1148; 955; 841; 786; 720; 698. RMa & RMa O2I; Short-delay profile; 32; 32; N/A; N/A; N/A; N/A; N/A. Normal-delay profile; 37; 37; N/A; N/A; N/A; N/A; N/A. Long-delay profile; 153; 153; N/A; N/A; N/A; N/A; N/A. UMi / UMa O2I; Normal-delay profile; 240. Long-delay profile; 616.

Observation 4: For the AoA/AoD/ZoA/ZoD angles, it seems tables for CDLA/CDLB/CDLC/CDLD/CDLE are suitable for all carrier frequencies according the description in 38.901 section 7.7.1.

Proposal 5: RAN4 to use the same CDL channel model/table for all FR1 FDD and FR1 TDD test assumptions.

CDL Scaling of angles

For Rel-19 CDL extension for MIMO simulation, RAN1 introduce below table as the scale factor in order to change the disturtion of the angles, and mentioned “Typical angular spreads for different scenarios can be obtained from the system-level model.”

38.901 Table 7.7.5.1-1

Table 7.7.5.1-1: Scale factor values for each CDL model CDL Type; Desired AOD Spread（°）; Scale Factor (AOD); Desired AOA Spread（°）; Scale Factor (AOA); Desired ZOA Spread（°）; Scale Factor (ZOA); Desired ZOD Spread（°）; Scale Factor (ZOD). CDL-A; 5; 0.0680; 30; 0.3531; 5; 0.0352; 1; 0.0352. 10; 0.1360; 45; 0.5268; 10; 0.1056; 3; 0.1056. 15; 0.2041; 60; 0.6981; 15; 0.1761; 5; 0.1761. 25; 0.3405. CDL-B; 5; 0.1238; 30; 0.5417; 5; 0.1940; 1; 0.1940. 10; 0.2475; 45; 0.8081; 10; 0.5822; 3; 0.5822. 15; 0.3710; 60; 1.0709; 15; 0.9705; 5; 0.9705. 25; 0.6168. CDL-C; 5; 0.1281; 30; 0.4307; 5; 0.3643; 1; 0.3643. 10; 0.2568; 45; 0.6447; 10; 1.0929; 3; 1.0929. 15; 0.3864; 60; 0.8585; 15; 1.8219; 5; 1.8219. 25; 0.6513. CDL-D; 5; 0.3231; 30; 9.8888; 5; 0.4477; 1; 0.4477. 10; 0.6652; 45; N/A; 10; 1.3469; 3; 1.3469. 15; 1.0594; 60; N/A; 15; 2.2579; 5; 2.2579. 25; 5.8637. CDL-E; 5; 0.3950; 30; 2.9733; 5; 0.9714; 1; 0.9714. 10; 0.8009; 45; N/A; 10; 2.9180; 3; 2.9180. 15; 1.2330; 60; N/A; 15; 4.8774; 5; 4.8774. 25; 2.3627. NOTE: Values of Table 7.7.5.1-1 were computed based on scaling factor calculation method described in Annex A.3..

For TR38.753, the desired AoD/AoA/ZoD/ZoA spread list as below table. Accordingly the scale factor of angles could be calculated as Table 2-1. It seems the scaling factors for Azimuth angles are larger than RAN1’s defination.

38.753 Table 5.1.4.1-1

Table 5.1.4.1-1: Angle scaling parameters AS; ASmodel / ASdesired [deg]. ASD; ASA; ZSD; ZSA. Model (827); 39.0949; 71.1175; 4.0666; 10.4245. Model (901); 37.4036; 71.4535; 4.0665; 10.4182. Desired; 25.7620; 74.1138; 4.8978; 18.2050.

Table 2-1. CDL scale factor used for 38.753

| CDL Type | Desired AOD Spread（°） | Scale Factor (AOD) | Desired AOA Spread（°） | Scale Factor (AOA) | Desired ZOA Spread（°） | Scale Factor (ZOA) | Desired ZOD Spread（°） | Scale Factor (ZOD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CDL-C | 25.7620 | 0.6888 | 74.1138 | 1.0372 | 18.2050 | 1.7474 | 4.8978 | 1.2044 |

Observation 5: The scale factors for Azimuth angles we used in 38.753 seems larger than RAN1’s defination in 38.901 Table 7.7.5.1-1.

Proposal 6: RAN4 to study if the CDL scale factors used in 38.753 should be updated.

- Conclusion

In this contribution, we provide analysis and views on SCM topic.

The observations and proposals could be summarized as:

Observation 1: For TR38.753, the most important context of CDL-based spatial channel model are captured in Annex B, while only antenna modeling, parameters and UMa-CDLC1 table are described on the main body CDL approach section.

Proposal 1: For TR38.753, merge Annex B to chapter 5.1 to enhance the TR logical consistency, readability  and practicality.

Observation 2: For TR38.753, sub-section 5.2.2.2 especially describes cluster model example. However, the contexts for annex C which named “Modelling execution for TDL Channel Models” but actually about two TDL examples.

Proposal 2: For TR38.753, merge Annex C to chapter 5.2 to avoid cross-referencing.

Observation 3: For TR38.753 sub-section 5.1.4.1, it seems the description of power weighted mean angle μφ is independent with the angle scaling formula.

Proposal3: For TR38.753 sub-section 5.1.4.1, the descriptions for power weighted mean angle μφ should be updated to power weighted cluster mean angle defined in 38.901 Annex A.4.

Proposal4: For TR38.753 sub-section 5.1.4.1, the descriptions for mean angle μφ,model and power weighted mean angle μφ should be combined together in order to avoid ambiguous statement.

Observation 4: For the AoA/AoD/ZoA/ZoD angles, it seems tables for CDLA/CDLB/CDLC/CDLD/CDLE are suitable for all carrier frequencies according the description in 38.901 section 7.7.1.

Proposal 5: RAN4 to use the same CDL channel model/table for all FR1 FDD and FR1 TDD test assumptions.

Observation 5: The scale factors for Azimuth angles we used in 38.753 seems larger than RAN1’s defination in 38.901 Table 7.7.5.1-1.

Proposal 6: RAN4 to study if the CDL scale factors used in 38.753 should be updated.

- Reference

[1] RP-241610, New SID: Study on spatial channel model for demodulation performance requirements, RAN#104 meeting, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

[2] RP-251040, Revised SID: Study on spatial channel model for demodulation performance requirements, RAN#108 meeting, Nokia, BT Plc

[3] R4-2502378, Way Forward for [114][322]NR_SCM, RAN4#114 meeting, Nokia

[4] R4-2504756, Way Forward for [114bis][320] NR_SCM, RAN4#114bis meeting, Nokia

[5] R4-2508776, Way Forward for [115][321] NR_SCM, RAN4#115 meeting, Nokia

[6] R4-2508622, Simulation assumptions for SCM, RAN4#115 meeting, Huawei, HiSilicon, MediaTek

[7] R4-2510438, Simulation results on spatial channel model, RAN4#116 meeting, Samsung