3GPP TSG-RAN WG4 Meeting #116	R4-2510442

Bangalore, India, 25th – 29rd August, 2025

Agenda item:	7.24.7

Source:	Samsung

Title:	Discussion on UE demodulation requirements for NR LP-WUS and receiver

Document for:	Discussion

- Introduction

The Rel-19 WI on “New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)” was approved in the RAN#102 meeting and revised in the RAN#105 meeting [2] along with the completion of the Rel-18 SI on low-power wake-up signal and receiver for NR [3], in which the performance part starts from RAN4#115 in May 2025 and targeting to complete by RAN#111 in March 2026. The objectives for performance part are as below:

- Specify the UE demodulation performance and test cases for LP-WUS/WUR (RAN4). - Study testability of above requirements - Specify RRM measurement performance requirement if identified, and corresponding test cases for the LP-WUS/WUR (RAN4).

In last RAN4#115 meeting, the work plan and some issues about testability aspects and test assumptions were agreed [6]. In this contribution, we share our views and analysis on the testability aspects and assumptions for demodulation requirements of NR LP-WUS and receiver.

- Testability Aspects

About testability aspects, below agreements and WF achieved in last RAN4#115 meeting [6].

Issue 2-1-1: Whether to define a test mode for LP-WUS/WUR - Agreement - Take RF approach as a starting point to test MDR for demodulation requirements. RF agreement Verify LP-WUR based on 1% MDR of LP-WUS which can be tested based on UE’s response to the NW/TE upon successfully detecting the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or other methods) - FFS whether test mode is recommended for MDR testing. Issue 2-1-2: Whether to consider shortening the test duration (if a test is introduced) - Agreement: - FFS whether to introduce FAR test and further study feasibility of introducing FAR test - FFS whether test mode is recommended for FAR testing. Issue 2-1-3: Whether to define separate test cases for idle/inactive and connected modes - WF - Option 1: Test cases should be designed separately LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode since the procedures are different. If both IDLE/INACTIVE mode and CONNECTED mode are supported by the LP-WUR, which mode is selected for test should be up to UE declaration.

test mode for LP-WUS/WUR

According RAN1’s design, no ACK/NACK for LP-WUS should be replied to network by UE, whether UE is listening to LP-WUS is transparent to network. While for MDR test, UE should counter the detection rate and this function may not be needed for commercial products. At the same time, during the MDR test, it’s better to keep the MR sleep, otherwise the test procedure will be complex and need long time.

Observation 1: For MDR test, UE should counter the detection rate and this function may not be needed for commercial products.

Observation 2: During the MDR test, it’s better to keep the MR sleep, otherwise the test procedure will be complex and need long time.

Proposal 1:  Test mode is recommended for LP-WUS MDR test considering UE should counter the detection rate and keep MR sleep during MDR test.

Whether to consider shortening the test duration

As we mentioned above, if UE support the function to counter the detection rate and keep MR sleep during MDR test, which mean a test mode should be supported. Based on the test mode, and the necessity to make sure the test confidence, it seems no need to add more additional assumptions in order to shortening the test duration.

Proposal 2:  Based on the test mode, and the necessity to make sure the test confidence, it seems no need to add more additional assumptions in order to shortening the test duration.

Whether to define separate test cases for idle/inactive and connected modes

As we analysed in [7], according to the UE feature list [8] attached in the RAN1 LS [9], UE has full flexibility in terms of its supported RRC states. UE can support IDLE/INACTIVE mode operation (FG62-1(a)) and CONNECTED mode operation (FG62-2(a)) separately, and UE can also support IDLE/INACTIVE mode operation (FG62-1(a)) and CONNECTED mode operation (FG62-2(a)) together, i.e., there could be 3 types of UE in terms of supported RRC states:

- UEs supporting IDLE/INACTIVE mode only

- UEs supporting CONNECTED mode only

- UEs supporting both IDLE/INACTIVE mode and CONNECTED mode

| Features | Index | Feature group |
| --- | --- | --- |
| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM [overlaid] sequence |
| 62. NR_LPWUS | 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal |
| 62. NR_LPWUS | 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence |

For FR1, supporting LP-WUS in RRC_Idle states showed most obvious power saving benefits [3]. Though UE may support not all 3 RRC states, the large commercialization possibility is that the FR1 LP-WUS UE would either support RRC_Idle or support RRC_Idle & RRC_Connected states. Therefore, it can be expected that test method based on RRC_Idle could cover most available UEs. It is reasonable to prioritize RRC_Idle if demod requirements for LP-WUS will be defined only for one RRC state.

Proposal 3: If RAN4 define demod test cases for two RRC states, separate test cases should be defined for idle/inactive and connected modes. If RAN4 define demod test cases for only one RRC state, RRC_Idle should be prioritized.

- Assumptions for Demodulation Requirements

FAR test

About the FAR (False Alarm Rate), RAN4 demod WF [6] as below and further discussion are needed. It is clearly that if FAR is too high, the power saving gain will be impacted. In RAN1’s study, the FAR only from noise, doesn’t include the impact of the falsely alarmed from wake-up due to the detection of a LP-WUS which is intended to wake-up/alarm the LP-WUR of another UE within the same UE group. And from RAN1’s simulation results in [3], the difference between 1% and 0.1% FAR are not obvious. RAN1 also down select FAR to 1% for RRC idle/inactive state in RAN1#117 meeting. Therefore, considering the test effort and the FAR impact, 1% seems enough.

Issue 3-1-3: Whether to introduce FAR test - WF - Option 1: Yes. - Option 2: No. Issue 3-1-5: Test metric for FAR test (if introduced) - WF - Option 1: 1% FAR - Option 2: 0.1% FAR

RAN1#117 Chairman notes

Agreement For RRC idle/inactive state, down-select among the following options for at least indicating subgroup information using LP-WUS: - Option 1: A LP-WUS indicates a bitmap with each bit corresponding to one subgroup of N subgroups for part of, one or more PO(s), e.g., N is 8~16, 24 - Number of information bits for a LP-WUS is at least N, single LP-WUS to wake up one or more subgroups - Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256 - Number of information bits for a LP-WUS is at least ceil (log2(X1)), where X1 is the number of codepoints indicating one or more subgroups. X1 is reported by companies, X1 could be smaller, equal to or larger than N. - Option 3: A LP-WUS indicates multiple codepoint values with each corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256 - Number of information bits for a LP-WUS is at least K*ceil (log2(X2)), where X2 is the number of codepoints indicating one or more subgroups. X2 is reported by companies, X2 could be smaller, equal to or larger than N. - K is the number of multiple codepoint values in a LP-WUS where K is larger than 1 - How to satisfy FAR is reported by companies, e.g., FEC/CRC - Note: multiple TDMed LP-WUSs can be used to support more subgroups for each option. - Note: Y% effective paging rate per PO is reported by companies - The followings are considered when down-select among options: - The number of supported UE subgroups per PO: M - Average network overhead to indicate the number of UE subgroups M per PO - False wake up rate due to subgroup-based indication, which will impact the power saving gain - Paging latency - Note: Coverage target shall be met under 1%BLER, 1% FAR (for false alarm from noise)

Proposal 4: RAN4 introduce FAR test to ensure the power saving gain with the same FAR definition as RAN1 (FAR only from noise, doesn’t include the impact of the falsely alarmed from wake-up due to the detection of a LP-WUS which is intended to wake-up/alarm the LP-WUR of another UE within the same UE group).

Proposal 5: RAN4 use 1% FAR as test metric for LP-WUS FAR test.

Waveform and Receiver type

In last RAN4#115 meeting, about LP-WUS waveform and receiver type, we have below WF [6]. As specified in [1], OOK (On-Off Keying) with overlaid OFDM sequences over OOK symbol was decided as the waveform of LP-WUS.

In RAN1 #116, the following agreement was adopted to support both OOK-1 and OOK-4 for the LP-WUS OOK waveform generation. The key difference between OOK-1 and OOK-4 is coming from the number of OOK symbol that can be transmitted within 1 OFDM symbol. OOK-4 can transmit M OOK pulses within 1 OFDM symbol because it is required to performs pre-processing (e.g., Discrete Fourier Transform, Least Square) in time domain before subcarrier mapping. In a case that M value is 1, there is no difference between OOK-1 and OOK-4 in the receiver side. Hence OOK-4 provides scalability and flexibility on data rate and network overhead, and OOK-1 is relatively simpler. In addition, according the feature list [8], UE could support LP-WUS operation based on OOK signal or OFDM overlaid sequence. Consequently, new UE receiver function for LP-WUS should cover both OOK-1 and OOK-4, and both OOK signal based receiver and OFDM overlaid sequence based receiver.

Issue 3-1-6: LP-WUS waveform - WF Option 1: Consider both OOK-1 and OOK-4 Issue 3-1-7: Receiver type - WF - Option 1: Consider both OOK and OFDM receivers

RAN1#116 agreement

Agreement Support both OOK-1 and OOK-4 for LP-WUS. - FFS how OOK-1 and OOK-4 are specified - For OOK-4, M<=4, FFS supported values - The SCS of a CP-OFDM symbol used for LP-WUS generation can be the same as one of the SCS(s) used for other NR transmissions in the same CP-OFDM symbol - FFS different SCS.

Proposal 6: Introduce UE demodulation requirements for both OOK-1 and OOK-4 LP-WUS waveforms.

Proposal 7: Introduce UE demodulation requirements for both OOK and OFDM LP-WUS receivers.

Frequency Range

For the frequency range of the test scope for Rel-19 LP-WUS, below WF was endorsed in [4] in order to support FR2. Thus, both FR1 and FR2 should be covered. And as a starting point, it is reasonable to prioritize FR1 considering the work load.

It is proposed to endorse the following in RAN#105 for Rel-19 LP-WUS/WUR WI regarding FR2 support - Confirm in RAN#105 that Rel-19 LP-WUS work item scope includes both FR1 and FR2. - Request RAN1 to make a progress on the FFS “Whether the above is applicable to FR2” (under RAN1#117 agreement on LP-WUS and LP-SS bandwidth) in RAN1#118bis. - Reuse the FR1 design when possible - No additional guidance in RAN#105 for RAN4 work and TU allocation

Proposal 8: Introduce UE demodulation requirements for both FR1 and FR2 for LP-WUS, and prioritize FR1 as a starting point considering the work load.

Duplex mode

For the duplex mode, RAN4#111 meeting confirmed that no specific limitation on the bands as below. Therefore, LP-WUS should be supported for both FDD and TDD.

Issue 1-1-1: Operation bands for LP-WUS feature Agreements: RAN4 confirm LP-WUS is a general feature not limited to specific example band(s)

Proposal 9: Introduce UE demodulation requirements for both FDD and TDD for LP-WUS.

Channel Bandwidth and SCS

For the demodulation requirements in RAN4, 10MHz CHBW with SCS 15KHz for FR1 FDD, 40MHz CHBW with SCS 30KHz for FR1 TDD and 100MHz CHBW with SCS 120KHz for FR2 are usually used by majority test cases.

Proposal 10: For the channel bandwidth and SCS, take below assumptions for the starting points

- 10MHz CHBW with SCS 15KHz for FR1 FDD;

- 40MHz CHBW with SCS 30KHz for FR1 TDD;

- 100MHz CHBW with SCS 120KHz for FR2

Number of LP-WUS PRBs

For the number of LP-WUS PRBs, the agreement in RAN1#117 meeting for FR1 TDD as following is 11 PRBs for LP-WUS and LP-SS.

Agreement Update agreement in last meeting as below: From RAN1 perspective, support X PRBs for LP-WUS and LP-SS with SCS 30kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5MHz - X to be down-selected between = 11 and 12 PRBs - FFS the number of PRBs for 15kHz - FFS if other number of PRBs needed, for LP-SS and LP-WUS with a channel bandwidth equal or less than 5MHz FFS: Whether the above is applicable to FR2

In addition, the same value that 11 PRBs for LP-WUS and LP-SS achieved in RAN1#118bis for FR1 FDD and FR2 as below. Therefore, the number of LP-WUS PRBs as 11 should be used for FR1 FDD, FR1 TDD and FR2.

Agreement From RAN1 perspective, support X=11 PRBs for LP-WUS and LP-SS with SCS 15kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5 MHz Agreement From RAN1 perspective, support X=11 PRBs for LP-WUS and LP-SS with SCS 60kHz and 120kHz for FR2.

Proposal 11: For the number of LP-WUS PRBs, the value 11 should be used for FR1 FDD, FR1 TDD and FR2.

Antenna configuration

For the antenna configuration, current WF 1T1R is reasonable configuration.

Proposal 12: For the number of antenna configurations, set 1T1R as a starting point.

Propagation condition

Although the transmit signal is different with legacy, the propagation condition is the same as legacy. Hence, we prefer to use the popular propagation conditions in demodulation requirements.

Proposal 13: For propagation condition assumption of LP-WUS test requirement, select one typical propagation condition for each case from TDLA30-10 and TDLC300-100.

Time/frequency error

During SI, some cases were evaluated under the assumption that LP-WUR can achieve the perfect synchronization based on LP-SS. However, from practical perspective, the potential residual error after the synchronization based on designed LP-SS should be considered.

About the Time/frequency error, below agreements agreed in RAN1#118 meeting.

Agreement To determine the binary sequences for LP-SS, the evaluation assumes the following: - Sync accuracy: timing estimation error smaller than T us for P=90 % of the time for at least SNR=-3dB and SNR=-6dB (lower priority), - T=2us(optional), 5us for OOK-1 - T=1us(optional), 2us for OOK-4 with M=2 - T=0.5us(optional), 1us for OOK-4 with M=4 - Other values of SNR, T, M are up to company report. - Additionally, companies can submit results on SINR with details on how the interference was modelled. - Assume one-shot estimation - The time error is based on 20ppm maximum frequency error for the detection of the first LP-SS - Note: Companies can assume other values inside of 20ppm maximum as long as the values are justified - RRM measurement accuracy: measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s for at least SNR=-3dB and SNR=-6dB (lower priority), X, Y, and Z is up to company report, other values of SNR are up to company report. - As a starting point, the time error is based on 5-10ppm residual frequency error. - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified - The frequency error: up to company report - Sampling rate: 3.84MHz (optional) or 7.68MHz assumed for 30kHz SCS. - Channel: AWGN and TDL-C 300ns is assumed. - Consider cross-correlation for 4 binary sequences under time and frequency error - Companies are encouraged to provide details on other additional simulation assumptions, high level description of their receiver algorithm, and assessment on power consumption Agreement For OOK-based LP-WUR, the LP-WUS design assumes the following: - As starting point, the time error is based on the 5-10ppm residual frequency error after frequency error correction without considering impact of drift. - The frequency error: up to company report. - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified Agreement As a starting point, for both time error and frequency error, the overlaid OFDM sequence design of LP-WUS assumes that the residual frequency error is 0.1-5ppm for OFDM-based LP-WUR after frequency error correction without considering impact of drift. - Note: Companies can use any value within the above range with justification on the value (including impact on power consumption)

In addition, below agreements agreed on RAN4 RF session in RAN4#112 meeting.

Issue 2-1-3: Time/frequency error Agreement: Residual frequency error: OFDM based receiver [5] ppm OOK based receiver [ 10 20] ppm

Furthermore, below agreements agreed on RAN4 RRM session in RAN4#112bis meeting.

Issue 2-1-3: Time/frequency error Agreement: Table 2: Cell-specific parameters Parameter; Unit; Cell 1; Cell 2. Frequency offset relative to UE frequency reference; Hz; OFDM based receiver [5] ppm OOK based receiver [5 10] ppm; N/A. Timing error; Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: T= 5us for OOK-1 T= 2us for OOK-4 with M=2 T= 1us for OOK-4 with M=4.

Taking these background details into account, we suggest considering the timing/frequency errors. The proposed timing error as T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4, and proposed frequency errors as OFDM based receiver [5] ppm, OOK based receiver [10] ppm.

Proposal 14: Suggest considering the timing/frequency errors as the test assumptions for LP-WUS performance requirements. The proposed timing error as T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4, and proposed frequency errors as OFDM based receiver [5] ppm, OOK based receiver [10] ppm.

Performance requirements for LP-SS

Although the performance of LP-SS estimation could impact the synchronization, the impact could be reflected by the final performance of receivers. Hence, we don’t think it’s necessary to define separate performance requirements for LP-SS.

Proposal 15: No need to define separate performance requirements for LP-SS.

- Conclusion

In this contribution, we provide analysis and views on the testability aspects and assumptions for for demodulation requirements of Rel-19 LP-WUS. The observations and proposals could be summarized as:

Observation 1: For MDR test, UE should counter the detection rate and this function may not be needed for commercial products.

Observation 2: During the MDR test, it’s better to keep the MR sleep, otherwise the test procedure will be complex and need long time.

Proposal 1:  Test mode is recommended for LP-WUS MDR test considering UE should counter the detection rate and keep MR sleep during MDR test.

Proposal 2:  Based on the test mode, and the necessity to make sure the test confidence, it seems no need to add more additional assumptions in order to shortening the test duration.

Proposal 3: If RAN4 define demod test cases for two RRC states, separate test cases should be defined for idle/inactive and connected modes. If RAN4 define demod test cases for only one RRC state, RRC_Idle should be prioritized.

Proposal 4: RAN4 introduce FAR test to ensure the power saving gain with the same FAR definition as RAN1 (FAR only from noise, doesn’t include the impact of the falsely alarmed from wake-up due to the detection of a LP-WUS which is intended to wake-up/alarm the LP-WUR of another UE within the same UE group).

Proposal 5: RAN4 use 1% FAR as test metric for LP-WUS FAR test.

Proposal 6: Introduce UE demodulation requirements for both OOK-1 and OOK-4 LP-WUS waveforms.

Proposal 7: Introduce UE demodulation requirements for both OOK and OFDM LP-WUS receivers.

Proposal 8: Introduce UE demodulation requirements for both FR1 and FR2 for LP-WUS, and prioritize FR1 as a starting point considering the work load.

Proposal 9: Introduce UE demodulation requirements for both FDD and TDD for LP-WUS.

Proposal 10: For the channel bandwidth and SCS, take below assumptions for the starting points

- 10MHz CHBW with SCS 15KHz for FR1 FDD;

- 40MHz CHBW with SCS 30KHz for FR1 TDD;

- 100MHz CHBW with SCS 120KHz for FR2

Proposal 11: For the number of LP-WUS PRBs, the value 11 should be used for FR1 FDD, FR1 TDD and FR2.

Proposal 12: For the number of antenna configurations, set 1T1R as a starting point.

Proposal 13: For propagation condition assumption of LP-WUS test requirement, select one typical propagation condition for each case from TDLA30-10 and TDLC300-100.

Proposal 14: Suggest considering the timing/frequency errors as the test assumptions for LP-WUS performance requirements. The proposed timing error as T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4, and proposed frequency errors as OFDM based receiver [5] ppm, OOK based receiver [10] ppm.

Proposal 15: No need to define separate performance requirements for LP-SS.

- Reference

- RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC (moderator, RAN VC), 3GPP TSG RAN Meeting #102, Edinburgh, GB, December 11-15, 2023.

- RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024

- TR38.869, Study on low-power wake up signal and receiver for NR, Release 18.

- RP-242361, Way Forward on LP-WUS for FR2, vivo, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024

- R4-2507936, WF for [115][136] NR_LPWUS_UERF, vivo, 3GPP TSG RAN4 Meeting #115, Malta, May 19-23, 2025

- R4-2508626, WF for [115][332] NR_LPWUS_Demod, Qualcomm, 3GPP TSG RAN4 Meeting #115, Malta, May 19-23, 2025

- R4-2506970, Discussion on testability for LP-WUR, Samsung, 3GPP TSG RAN4 Meeting #115, Malta, May 19-23, 2025

- R1-2501397, Initial RAN1 UE features list for Rel-19 NR after RAN1 #120, Moderators (AT&T, NTT DOCOMO, INC.)

- R4-2503104_R1-2501399, LS on Rel-19 RAN1 UE features list for NR after RAN1#120, RAN1