3GPP TSG-RAN WG4 Meeting # 116                                    R4-2510452

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.14.4

Source: 	ZTE Corporation, Saneships

Title: 	Discussion on the fast SCell activation for UE supporting Rel-18 EMR

Document for:	Approval

Introduction

In the RAN #103 meeting, the WID on several enhancements for NR and MR-DC RRM requirements was approval. There are two objectives need to discuss in Rel-19, one is FR2-1 SSB based L3 measurement delay reduction for connected mode, and the other is FR2-1 SSB based L3 measurement delay reduction for connected mode. In this meeting, we need to study the latter according to schedule [1].

- Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In this contribution, we would like to provide our views on fast SCell activation for UE supporting Rel-18 EMR.

Discussion

According the work item, valid early measurement report would be considered in the existing known/unknown SCell condition during SCell configuration/activation in Release 19, which could reduce delay requirement for the target SCell activation. It’s a common understanding that is feasible and beneficial to consider fast SCell activation for UE supporting Rel-18 EMR. In previous meeting, we have discussed and reached consensus on how to use R18 EMR and capture the changes in specification. And there is a remaining issue on UE capability should be discussed [2].

Issue 2-1-1: UE capability for EMR-based fast SCell activation FFS: - Option 1: Introduce a UE capability for R19 SCell activation based on EMR - Prerequisite feature groups: 39-8 or 39-9 in R18 RAN4 feature list - Need for the gNB to know if the feature is supported: No - Granularity: Per-UE - FDD/TDD differentiation: No - FR1/FR2 differentiation: Yes - Option 2: Not to introduce additional UE capability

In R18 eEMR, we discussed how to reuse the IDLE/INACTIVE mode measurement results which to be reported during RRC connection setup/resume in order to improve SCell/SCG setup delay. As the result, EMR and cell reselection measurement are considered, and two UE capabilities measValidationReportEMR-r18 and measValidationReportReselectionMeasurements-r18 are introduced. Besides, R16 EMR is supported by UE capability idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16.

According to the discussion about applicability, fast SCell activation delay requirements apply to following cases:

-	UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or,

-	UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or,

-	UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

Comparing the UE capability introduced in Release 18 with the capabilities required by Release 19, we find that the existing UE features (including R16 and R18) can meet all the requirements defined in Release 19 for fast SCell activation for UE supporting Rel-18 EMR. In our understanding, there is no need to discuss UE capability for fast SCell activation with Rel-18 eEMR report because existing UE capabilities are enough.

However, in the last meeting, some company proposed option1 and showed the view from UE prespective. If the capability of R19 is not introduced, the UE may not be able to distinguish between the requirements of R18 and R19, and there may be situations where the R18 UE cannot complete the R19 requirements or the R19 UE only completes the R18 requirements. Based on this, UE capability is introduced and it is only to indicate that the UE is an R18 or R19 UE to avoid mismatches. I am curious if there may be confusion between UE release and requirement release. Can UE not determine which release it belongs to based on other existing information? If it is not possible to distinguish whether the UE belongs to R18 or R19 based on existing information, a compromise can be made to introduce the UE capability to indicate.

Proposal 1: Prefer not to introduce additional UE capability for fast SCell activation with Rel-18 eEMR report.

Conclusions

In this paper, we have some discussion on fast SCell activation for UE supporting Rel-18 EMR. And we have the following proposals:

Proposal 1: Prefer not to introduce additional UE capability for fast SCell activation with Rel-18 eEMR report.

References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT

- R4-2508266, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT