3GPP TSG- WG4 Meeting # 116	R4-2510468

# Bengaluru, India, 25th – 29th August 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on simultaneous Rx-Tx

Agenda Item:	6.11.2

Document for:	Approval

Introduction

In the approved revised WID on simultaneous Rx-Tx [1], the analysis of CA_n5A-n8A is included.

In this paper, we provide considerations on CA_n5A-n8A based on the WF [2] in the last meeting.

<Way forward>: For CA_n5-n8, further consider the following solution with side conditions to support both 1UL/2DL and 2UL/2DL CA with non-concurrent n5 DL and n8 UL in Rel-19: Support case 1 and case 2 via RRC reconfiguration. - Case 1: 2UL/2DL configuration, where NW shall keep all the four carriers activated, allocate resource allocation for a dedicated user only to n5 UL, n8 UL and n8 DL (no n5 DL resource allocation to a dedicated user) and both two bands are collocated. - Case 2: 1UL/2DL configuration Companies are encouraged to study necessity of additional RRM spec or additional requirement(s) of other working groups. If there is impact on RRM or other WGs beyond the introduction of UE capability, the solution is not considered in Rel-19. Whether new capability is needed or not would be further discussed.

Besides, this paper also further analyze based on the WF [3] about how to handle the exceptional cases for FDD-FDD and FDD-SDL.

Discussion

Current state for simultaneous Rx-Tx of inter-band CA in spec:

For TDD-TDD and TDD-FDD inter-band CA, taking CA_nA-nB for example, the capability simultaneousRxTx does not differentiate the two cases. Just both supported or neither.

- simultaneous (nAUL, nBDL)

- simultaneous (nBUL, nADL)

In RRM requirement Applicability of requirements for scheduling availability in TS 38.133 clause 3.6.9

The scheduling availability requirements in clause 8.1.7.1, 8.1.7.2, 8.5.7.1, 8.5.7.2, 8.5.8.1, 8.5.8.2, 9.5.6.1, 9.5.6.2, 9.8.6.1, and 9.8.6.2 assumes that the UE is not configured with simultaneous UL/DL between two FR1 bands if the UE does not have the capability of supporting simultaneousRxTxInterBandCA.

For FDD-FDD inter-band CA, it was concluded that simultaneous Rx-Tx is mandatory without signalling. The exceptions should be considered. The frequency range of UL in band n8 and DL in band n5 is overlapped, and it is impossible to implement simultaneous (n5DL, n8UL). Meanwhile, simultaneous (n5UL, n8DL) can be supported by default. In RAN4#113, RAN4 sent LS [4] to RAN2 to ask necessary modification in RAN2 spec and/or add UE capability to enable the functionality of different simultaneous Rx-Tx capability for (n5UL, n8DL) and (n8UL, n5DL) in CA_n5-n8 with two FDD bands frequency range partially overlap.

Figure 1 Two possible operations illustration for CA_n5-n8

Continue discuss UL CA for CA_n5-n8 in Rel-19:

To support case 1 and case 2 via RRC reconfiguration in [2], the pre-requisite is to implement cross carrier scheduling. In Huawei network products and HiSilicon chipsets after Rel-18, the single-DCI feature is supported. The feature is to enable single DCI scheduling multiple PDSCHs or multiple PUSCHs over multi-carriers.

Observation 1: The implementation of Single-DCI feature enables the cross carrier scheduling to support case 1 in the scheme of WF in the last meeting.

RAN4 conducted studies in Rel-18 on enhancements for 700/800/900MHz band combinations. The studies concluded that three options are feasible for implementing the CA_n5-n8 band combination, as detailed in TR 38.872.:

1) Full band n5 and n8 RF filters implementation with option 1 and option2: - Option 1: Only support 1UL/2DL CA. Single UL in n5 - Option 2: Support both 1UL/2DL and 2UL/2DL CA. Non-concurrent n5 DL and n8 UL 2) Dedicated RF filters implementation with partial frequency range - Option 3: Support both 1UL/2DL and 2UL/2DL CA. Dedicated filter to allow simultaneous n5 DL and n8 UL without any scheduling restriction on both bands.

In [5], it was pointed out that standardizing Option 2 in Rel-19 may require a future-proof signalling design to support that the capabilities for the simultaneous operation in case more similar NR FDD-FDD inter-band CA band combinations where UL and DL from different FDD bands partially overlap are introduced, as requested in the original LS [4]. The only workload to RAN2 would be to introduce a capability differentiating simultaneous Rx-Tx capability for (nA UL, nB DL) and (nA DL, nB UL). If some day, the dedicated filter is implemented and enable the simultaneous (nA UL, nB DL). The forward-compatibility should be considered. Without simultaneous Rx-Tx between n5 DL and n8 UL, the scheduling restriction would be applied according to RRM spec. No switching behaviour is required in Rel-19.

Proposal 1: To support UL CA for CA_n5-n8 in Rel-19, a future proof capability differentiating simultaneous Rx-Tx capability for (nA UL, nB DL) and (nA DL, nB UL), is required.

After the introduction of the new capability to differentiate simultaneous Rx-Tx capability for (nA UL, nB DL) and (nA DL, nB UL), the scheduling availability in RRM spec TS 38.133 can be updated accordingly. No other RRM spec impact is observed.

Proposal 2: Scheduling availability in RRM spec TS 38.133 can be updated according to the new capability for simultaneous Rx-Tx to differentiate (nA UL, nB DL) and (nA DL, nB UL). No other RRM spec impact is observed.

However, CA_n5-n8 would not be the only FDD-FDD BC that hardly supports simultaneous Rx-Tx. For band combination without sufficient separation between DL carrier in one band and UL carrier in the other band, simultaneous DL-UL operation is difficult. In [6], two operators in North America identify two sub-1GHz to be aggregated, CA_n26-n106 with 2MHz isolation between n26 DL and n106 UL. How to design the duplexer for the BC is questionable.

Observation 2: The FDD-FDD band combination CA_nA-nB without sufficient frequency separation between band nA UL and band nB DL can hardly support simultaneous operation of (nA UL, nB DL).

For FDD-SDL inter-band CA, it was concluded that simultaneous Rx-Tx is mandatory without signalling. For Rel-19 low band carrier aggregation via switching [7], there is no simultaneous Rx-Tx between FDD UL and SDL. From the duplexer design perspective, the frequency separation between the bands does not pose a feasibility challenge in general. Things are quite similar to that for FDD-FDD without sufficient isolation between nA UL and nB DL in CA_n12-n29.

Observation 3: FDD-SDL band combination in Rel-19 low band carrier aggregation via switching without sufficient frequency separation results in duplexer design difficulty, and makes it hard to implement simultaneous Rx-Tx for FDD UL and SDL.

In [6], CA_n28-n67 was also proposed for CA via switching, n67 DL (738-758MHz) overlaps with n28 UL (703-748MHz). The reason for non-simultaneous Rx-Tx is same as that for CA_n5-n8 (n5DL, n8UL).

Conclusion

In this contribution we further the rules for simultaneous RxTx for FR1, according to the analysis, we have the following observations and proposals:

Observation 1: The implementation of Single-DCI feature enables the cross carrier scheduling to support case 1 in the scheme of WF in the last meeting.

Observation 2: The FDD-FDD band combination CA_nA-nB without sufficient frequency separation between band nA UL and band nB DL can hardly support simultaneous operation of (nA UL, nB DL).

Observation 3: FDD-SDL band combination in Rel-19 low band carrier aggregation via switching without sufficient frequency separation results in duplexer design difficulty, and makes it hard to implement simultaneous Rx-Tx for FDD UL and SDL.

Proposal 1: To support UL CA for CA_n5-n8 in Rel-19, a future proof capability differentiating simultaneous Rx-Tx capability for (nA UL, nB DL) and (nA DL, nB UL), is required.

Proposal 2: Scheduling availability in RRM spec TS 38.133 can be updated according to the new capability for simultaneous Rx-Tx to differentiate (nA UL, nB DL) and (nA DL, nB UL). No other RRM spec impact is observed.

- References

[1] RP-251536, Revised WID Simultaneous RxTx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-19, RAN#108, June, 2025.

[2] R4-2507910, WF on simultaneous Rx/Tx requirements, RAN4#115, May, 2025

[3] R4-2502840, WF on simultaneous Rx-Tx, RAN4#114, February, 2025.

[4] R4-2420410, LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx, RAN4#113

[5] R4-2505124, Reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx, RAN4#114bis, April, 2025.

[6] R4-2411176, Motivation for a new work item on LB+LB CA based on switching in Rel-19, Apple, Skyworks Solutions Inc., BT plc, TELUS, Bell Mobility, Anterix, Southern Linc, RAN4#112.

[7] RP-243317, New WID on low band carrier aggregation via switching, RAN#106