3GPP TSG-RAN WG4 Meeting # 116	R4-2510474

# Bengaluru, India, 25th – 29th August 2025

Source:	Huawei, HiSilicon

Title:	TP for TR 38.719-00-00: CA_n5_n78-n89

Agenda item:	6.3.6

Document for:	Approval

1 Background

This contribution provides text proposal on the inter-band CA with SUL band combination CA_n5_n78-n89.

2 Text Proposal

---Start of changes---

5.X	CA_n5_n78-n89

5.X.1	Operating bands

Table 5.X.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1 from TS 38.101-1) |
| --- | --- |
| CA_n5_n78-n89 | n5, n78, n89 |

5.X.2	Configuration

Table 5.X.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A_n78A-n89A | SUL_n78A-n89A | n5 | See n5 channel bandwidths in Table 5.3.5-1 for each carrier | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 for each carrier |  |
|  |  | n89 | See n89 channel bandwidths in Table 5.3.5-1 for each carrier |  |
| CA_n5A_n78C-n89A | SUL_n78A-n89A CA_n78C-n89A | n5 | See n5 channel bandwidths in Table 5.3.5-1 for each carrier | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 for each carrier |  |
|  |  | n89 | See n89 channel bandwidths in Table 5.3.5-1 for each carrier |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1 from TS 38.101-1. |  |  |  |  |

5.X.3	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

5.X.4	UE co-existence studies for 1 band UL

Table 5.X.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n5_n78-n89.

Table 5.X.4-1: Harmonic/Harmonic mixing analysis

| UL/DL harmonics |  | n89 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | fLow |  | 824 |  | 1648 |  | 2472 |  | 3296 |  | 4120 |  |  |  |
| n78 |  | fLow | fHigh |  | 849 |  | 1698 |  | 2547 |  | 3396 |  | 4245 |  |  |  |
| DL1 |  | 3300 | 3800 |  | N/A |  | - |  | - |  | D |  | - |  | UL harmonic |  |
| DL23 |  | 6600 | 7600 |  | - |  | N/A |  | - |  | N/A |  | N/A |  | Harmonic mixing |  |
| DL34 |  | 9900 | 11400 |  | - |  | - |  | N/A |  | - |  | N/A |  |  |  |
| DL4 |  | 13200 | 15200 |  | - |  | N/A |  | N/A |  | N/A |  | N/A |  |  |  |
| DL54 |  | 16500 | 19000 |  | - |  | - |  | N/A |  | N/A |  | N/A |  |  |  |
| Analysis |  |  | Direct hit n89 UL4 / n78 DL1, and no near miss n89 ULx / n78 DLy (5MHz) |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to n*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.X.4-2 summarizes frequency ranges where cross band isolation may occur for CA_n5_n78-n89.

Table 5.X.4-2: Cross-band isolation analysis

| Bands3 | n78 |  | n89 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 3300 | 3800 | 824 | 849 |
| fDL (MHz) | 3300 | 3800 | NA | NA |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 10 | 100 | 5 | 20 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 3200 | 3900 | 804 | 869 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 3100 | 4000 | 784 | 889 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 3000 | 4100 | 764 | 909 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 2900 | 4200 | 744 | 929 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 2800 | 4300 | 724 | 949 |
| Analysis | There is no cross band isolation issue for this combo when UL band is band n78. |  | There is no cross band isolation issue for this combo when UL band is band n89. |  |
| Note 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1. - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible Note 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.X.5	REFSENS requirements

The 4th harmonic MSD requirements for DL band n78 affected by UL band n89 has been specified for SUL_n78-n89.

5.X.6	∆TIB and ∆RIB values

Table 5.x.6-1: ΔTIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n5_n78-n89 | 0.6 | 0.8 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

Table 5.x.6-2: ΔRIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n5_n78-n89 | 0.2 | 0.5 | - |
| NOTE *: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE **: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

5.X.7	Out-of-band blocking exception

There is no Out-of-band blocking exception for CA_n5_n78-n89.

---End of changes---