3GPP TSG RAN WG4 Meeting #116	R4-2510486

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.19.2

Source:	vivo

Title:	Discussion on general aspects for AI mobility

Document for:	Discussion

- Introduction

In RAN#105 meeting, the revised SID [1] was approved and RAN4 work is copied as:

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

Based on this SID, RAN4 had further general discussion on priorities from scenarios-level, use-cases level and sub-use -cases level in the past meetings. In this contribution, we provide our views on general aspects for AI/ML mobility based on the related progress captured in WF[2].

- Discussion

- 2.1 general aspects in AI/ML for mobility

### 6.2.1 RRM requirements for measurement prediction #### 6.2.1.1 General The impact on RRM requirements for L3 cell-level RSRP (i.e., Point C as defined in Figure 9.2.4-1 in TS 38.300) measurement prediction has been studied for scenarios 2, 3, and 4 as defined in Table 5.2.1.1-1 and the sub-use cases 1, 2, and 3 as defined in section 4.2. Editor Note: The impact of L3 beam level (Point E as defined in Figure 9.2.4-1 in TS 38.300) measurement prediction has not been studied. However, it doesn’t preclude the discussion on L3 beam level measurement prediction in the work item phase. Editor Note: From RAN4 perspective, it is not precluded that other scenarios in Table 5.2.1.1-1 will be discussed in work item. Editor Note: For scenario 3, non-co-located cases are not precluded.

For inter-frequency prediction, based on the previous meeting's TP[3], there was an editor's note clarifying that non-co-located cases are not precluded. For this case, the latest RAN#108 meeting provided further conclusions regarding the WID scope. According to the following WID[4] description, it limits frequency domain prediction to collocated cases only.

On the other hand, RAN4 did not specifically study or discuss non-collocated cases during the SI discussions. Therefore, we propose that during the SI phase, RAN4 should only consider collocated cases for frequency domain prediction. Accordingly, the above editor's note regarding non-collocated cases in the existing SID needs to be deleted.

The objectives of this work item focuses on AI/ML-assisted mobility in RRC_CONNECTED mode, wherein RRM measurement prediction and measurement event prediction could be applied. Only functionality-based LCM procedures are supported. In general, RRM measurement configuration and report framework in RRC layer are taken as baseline. The objectives are as the following: - Specify support for RRM measurement prediction for both UE and network sided models [RAN2]: - Specify signalling and protocol aspect to enable reporting inference outcome of UE sided model for RRM measurement prediction - Specify signalling and protocol aspect to enable reporting information from UE for inference operation of network sided model for RRM measurement prediction - At least the use cases studied so far in RAN2 SI are supported: Temporal domain prediction case A and case B, and Frequency domain prediction for collocated cases. Cell level predication is supported Note 1: For UE sided model, spatial domain is not supported. Can check in RAN#109 whether for network sided model spatial domain is supported or not, based on the final outcome of the SI. Note 2: Check in RAN#109 whether L3 beam level prediction is also supported, based on the final outcome of the SI. - Specify support for measurement event prediction for UE sided models [RAN2]: - Specify signaling and protocol aspect to enable reporting inference outcome of UE sided model for measurement event prediction - Measurement event A1~A6 are in the scope in this release - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach (considering further progress for AI/ML PHY) - Inference configuration - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection, for which AI/ML PHY request/configuration framework is taken as baseline Note 3: Data transfer and model delivery for UE sided model are out of scope of this WID. And model related choices can be up to UE’s implementation. - For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]: - Network sided data collection, at least - UE is configured to log RRM measurement results. - UE supports periodical logging and L3 event triggered logging. - UE can indicate logged data availability. Note 4: The above objectives to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible. - For both RRM measurement prediction and measurement event prediction [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives - Specify LCM-related core requirements to support the LCM functionality management for both UE and network sided functionalities, if any Note 5: Can check in RAN#109 whether RAN2-related objectives need further updates, based on RAN2’s further discussions. Note 6: Can check in RAN#109 whether RAN4-related objectives need further updates, based on RAN4’s further discussions.

Proposal 1: RAN4 only to consider collocated cases for frequency domain prediction. Accordingly, the above editor's note regarding non-collocated cases in the existing SID needs to be deleted.

RAN4 agreements in RAN4#112bis Issue 1-1-1: Priorities of different use cases Agreement: - RAN4 to start the discussion with RRM measurement prediction use-case from RAN4 #112bis meeting. Agreement: - RAN4 to only start the discussion of Measurement event prediction use-case and RLF use-case if sufficient progress in RAN2. - The earliest time to start the discussion for Measurement event prediction use-case is RAN4#114 meeting (Feb 2025). - The earliest time to start the discussion for RLF use-case is from RAN4#114bis meeting (Apr 2025).

As for the priority of use cases, RAN4 had the initial working plan in RAN4#112bis meeting as above.

For the RLF prediction, according to the agreement concluded in RAN2#129 meeting as follows, it will not be studied in Rel-19 due to the limited time for SI discussion. As per this working plan, the part of RRM impact study will be excluded in RAN4 accordingly.

RAN2 Agreements on priority - RLF prediction will not be studied in Rel-19 - Further RRM prediction simulations are not expected. For next specification general impact study phase we will consider spatial, temporal, and frenquency domain cell level prediction. FFS if L3 beam level needs to be considered. For now, temporal and frequency domain prediction will be considered with higher priority for detailed study (if needed). - Continue with generalization across cell configurations for RRM prediction - Continue with measurement events

Proposal 2: RAN4 not to study RRM impact on RLF prediction in Rel-19

- Summary

Proposal 1: RAN4 only to consider collocated cases for frequency domain prediction. Accordingly, the above editor's note regarding non-collocated cases in the existing SID needs to be deleted.

Proposal 2: RAN4 not to study RRM impact on RLF prediction in Rel-19

- References

- RP-242393, Revised SID on AIML for mobility in NR, OPPO

- R4-2504901, WF on RRM requirements for FS_NR_AIML_Mob, Nokia

- R4-2508419, TP on RAN4 aspects for TR 38.744, OPPO, Nokia

- RP-251864, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, CATT