3GPP TSG RAN WG4 Meeting #116	R4-2510489

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.19.4

Source:	vivo

Title:	Discussion on testability and interoperability for AI mobility

Document for:	Discussion

- Introduction

In RAN#105 meeting, the revised SID [1] was approved and RAN4 work is copied as

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In this contribution, we will further provide our views on testability and interoperability issues for AI/ML mobility based on the latest WF[2].

- Discussion

- 2.1	Testability issues in AI/ML based beam management

In AI/ML air interface discussion, many testability issues are raised and discussed in AI BM sub-case, where the key issue is whether and how to emulate a multi-beam environment with a fading channel of spatial characteristics. A potential new FR2 OTA test system for AI BM will bring high cost and complexity for the related vendors and many technical issues are raised for the test setup.

Observation 1: Whether and how to emulate a multi-beam environment with a fading channel of spatial characteristics is the key issue in AI BM discussion.

- 2.2 Potential testability issues in AI/ML for mobility

2.2.1 Predcition consistency

### Issue 1-1-1: Prediction consistency in time domain <Way forward >: - Consider the propagation condition for the validation of the RRM measurement prediction requirements before modeling different time-varying characteristics per cell/site in FR1. - Further discuss the following options on how to model different time-varying characteristics per cell/site in FR1. - Option 1: Change Tx power. - FFS: whether it is possible to change Tx power every symbol at TE using conductive test - Option 2: Change channel condition with fixed Tx power. - Option 3: Computer simulations alike test setup can be adopted. TE can transmit the signals of cell 1, 2, 3 with time-variance following a certain channel model. - Option 4: Select one reference UE trajectory model (e.g., from TR 38.843) to study prediction consistency impact

The previous meeting further discussed prediction consistency in the time domain, with the outcome as shown above.

In AI/ML based beam management case 1, Top K beams are predicted in spatial domain and multi-probe anechoic chamber test system are needed to emulate the spatial channel environment. If one-probe setup or flat channel model is used, the real performance of AI model could not be reflected. In AI mobility, time and frequency domain prediction is performed and AI model performance can be verified in a test system without any spatial characteristics. Based on this understanding, for AI mobility use case, we consider to model different time-varying characteristics by changing Tx power for one cell.

Thus, there is no need to considered the CDL channel since there no spatial characteristics needs to be reconstruced in the chamber. The AI prediction can also operate normally without spatial characterists since all scenarios are in temporal/frequency domain and AWGN/TDL channels are enough for OTA testing.

Observation 2: In AI/ML based BM case 1, all the Tx beams with spatial domain characteristics need to be emulated in OTA chamber, while the spatial characteristics are not necessarily needed in AI mobility test system.

Proposal 1: RAN4 to consider model different time-varying characteristics per cell/site by changing Tx power.

Proposal 2: RAN4 to consider AWGN/TDL channel model as the candidate propagation conditions for further discussion in WI.

2.2.2 UE trajectory

In legacy test, the dynamic characteristic are only reflected by changing the doppler for generating the channel coefficients and changing the SNR. However, the AI model may be trained under field data/UMa data in a continues time with temporal/spatial consistency and correlations considering UE is moving and rotating. If only change the dopper and SNR, the test environment may be looks randomly from UE-sided AI model perspective and mismatch may be occurred. Therefore, we proposal to study a new method to model UE trajectory, method in TR 38.762 for dynamic OTA testing can be referred to, e.g., modelling a UE trajectory with some waypoints and generate channel model in every waypoint, as shown in Fig. 2. UE could be ratoted in a 3D turnable with the reference LOS AoA direction and change the probe power weight to emulated the dynamic environment.

Fig. 2 Base station and the UE route in the TR 38.762 [3].

Proposal 3: RAN4 to consider modelling reference UE trajectory for OTA testing and to emulate the dynamic channel by changing the doppler, changing the SNR, rotate UE in turnable, and change probe power.

- Summary

Observation 1: Whether and how to emulate a multi-beam environment with a fading channel of spatial

characteristics is the key issue in AI BM discussion.

Observation 2: In AI/ML based BM case 1, all the Tx beams with spatial domain characteristics need to be emulated in OTA chamber, while the spatial characteristics are not necessarily needed in AI mobility test system.

Proposal 1: RAN4 to consider model different time-varying characteristics per cell/site by changing Tx power.

Proposal 2: RAN4 to consider AWGN/TDL channel model as the candidate propagation conditions for further discussion in WI.

Proposal 3: RAN4 to consider modelling reference UE trajectory for OTA testing and to emulate the dynamic channel by changing the doppler, changing the SNR, rotate UE in turnable, and change probe power.

- References

- RP-242393 Revised SID on AIML for mobility in NR, OPPO

- R4-2508269 WF on FS_NR_AIML_Mob_Part2, Nokia

- R4-2418755 Discussion on impacts on RAN4 requirements for AI mobility, vivo