3GPP TSG RAN WG4 Meeting #116	R4-2510490

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.27.5.1

Source:	vivo

Title:	Discussion on RRM requirements on (e)RedCap for R19 NR NTN Phase 3

Document for:	Discussion

- Introduction

In the last meeting, RAN4 further analyzed the potential RRM impacts on Rel-19 NR NTN WI. The considerations from companies and the latest progress on (e)RedCap NTN were summarized in WF [1]. In this contribution, we would like to provide our further analysis on aspects on RRM requirements on (e)RedCap NTN.

- 2.	Discussion

  - General consideration on RRM requirements

#### Issue 1-2-3: Other features Online agreement Agreement: - SMTC adjustment in idle/inactive is kept [mandatory] for (e)RedCap UEs with NTN. - From RAN4 perspective, location-based measurement report trigger is kept optional for (e)RedCap UEs with NTN.

According to the R17 design, SMTC adjustment in idle/inactive is an optional UE capability. The purpose of this issue is to make it mandatory for (e)RedCap UEs with NTN.

34-11; SMTC adjustment in idle/inactive; It's optional for UE to perform SMTC adjustment in RRC_IDLE/RRC_INACTIVE; n/a; n/a; n/a; n/a; Optional without capability signalling.

From technical perspective, since RedCap NTN UEs have weaker processing capabilities, and based on the existing conclusion that mandates support for two SMTCs, when the network (NW) alone controls the positions of the two SMTCs (i.e., the UE does not support autonomous SMTC adjustment), the UE's processing becomes less flexible if the two SMTCs are close to each other. In this case, supporting UE-autonomous adjustment of the SMTC offset offers greater flexibility.

However, from spec perspective, the existing RAN2 mechanism for UE-autonomous SMTC adjustment is reflected in the highlighted description of `smtc4list`. This implies that if the UE reports support for autonomous adjustment, it may or may not adjust the SMTC offset based on the actual PDD. If this feature is to be enhanced to mandatory support, the corresponding RAN2 specifications may also require modification. Given that this is the final meeting for the core part, we recommend aligning with the current NTN approach and not making any spec changes.

smtc4list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in this list, it is up to the UE to select which SMTCs to consider.

Proposal 1: Maintain the same principle used in NR NTN for RedCap devices: the requirements are transparent to whether the UE adjusts or not the SMTC window in IDLE/INACTIVE mode.

  - UE capability

#### Issue 1-4-1: General principle for designing the UE capabilities for (e)RedCap UE with NTN <Way Forward>: - Option 1: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). (CATT, CMCC, ZTE, Xiaomi, E///, Samsung - compromise, Apple, QC) - FFS: The Rel-19 requirement can be release independent from Rel-17. - Option 2: RAN4 to introduce new capability to declare support over RedCap over NTN. (vivo, Samsung) - Option 3: to wait for RAN1 to confirm the working assumption (UE that supports RedCap and NTN shall support RedCap over NTN). (Nokia, QC, MTK)

For flexibility considerations, we support Option 2.

For Option 1, several issues need to be discussed and resolved before agreement:

First, there are existing requirements for (e)RedCap and NTN separately, as well as for RedCap NTN. If Option 1 is followed, for R17/18 UEs declaring support for RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support for NTN (nonTerrestrialNetwork-r17), do they need to meet the requirements defined in R19 for (e)RedCap UEs with FR1-NTN bands, or do they not need to meet any requirements (including those defined in R17/R18 for RedCap and NTN respectively)? If they also need to meet the relevant R19 requirements, then the Rel-19 requirements must be release-independent from R17.

For the release-independent approach, one exceptional feature to consider is HD-FDD collision handling. According to the latest RAN1-agreed CR [2], both CollisionHandlingOfHDFDDOperation and ntnRedCapCollisionCase4UlPriority were introduced in R19. To our understanding, R17/R18 UEs do not support handling collision Case 3 and collision Case 4 for half-duplex FDD operation. Therefore, when considering release independence, how to handle this feature also needs to be discussed.

- 17.2 Half-Duplex UE in paired spectrum A half-duplex UE (HD-UE) in paired spectrum is not capable of simultaneous transmissions and receptions on a serving cell with paired spectrum. This clause is applicable for communication of a HD-UE on a serving cell with paired spectrum. Procedures for a HD-UE are same as described for a UE in all other clauses of this document unless stated otherwise. A HD-UE that operates on a non-NTN serving cell, or a HD-UE that operates on an NTN serving cell and does not indicate CollisionHandlingOfHDFDDOperation, does not expect to detect a DCI format scheduling a reception in a set of symbols and detect a DCI format scheduling a transmission in any symbol from the set of symbols. A HD-UE that operates on an NTN serving cell in the RRC_CONNECTED state, indicates CollisionHandlingOfHDFDDOperation, is scheduled to receive a PDSCH or CSI-RS in a set of symbols based on an indication by a first DCI format, and is scheduled to transmit a PUSCH, PUCCH, SRS, or PRACH that overlap with any symbol from the set of symbols based on an indication by a second DCI format, determines - to either receive the PDSCH or the CSI-RS or transmit the PRACH based on the HD-UE implementation - to either receive the PDSCH or transmit the PUSCH, PUCCH, or SRS based on the HD-UE implementation, if the first DCI format is provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set - to receive the PDSCH or the CSI-RS if the first DCI format is provided by a PDCCH the HD-UE received according to a search space set other than a Type0/0A/1/2-PDCCH CSS set, and the HD-UE is not provided ntnRedCapCollisionCase4UlPriority, and timeline conditions for cancellation of the UL transmission as defined in clause 17.2 are satisfied - to transmit the PUSCH, PUCCH, or SRS, if the first DCI format is not provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set, and - the HD-UE is provided ntnRedCapCollisionCase4UlPriority, or - the HD-UE is not provided ntnRedCapCollisionCase4UlPriority and timeline conditions for cancellation of the UL transmission as defined in clause 17.2 are not satisfied

Observation 1: Based on RAN1 specifications, the priority rules for collision handling of overlapping DL and UL in HD-FDD for RedCap-NTN apply only to Rel-19 UEs and do not include Rel-17 and Rel-18 UEs.

Proposal 2: For UE capabilities of (e)RedCap UEs with NTN, if Option 1 is followed, RAN4 needs to consider the following issues:

- For R17/18 UEs declaring support for RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support for NTN (nonTerrestrialNetwork-r17), should they:

  - Alt 1: Meet the Rel-19 requirements (which would be release-independent from Rel-17), or

  - Alt 2: Not need to meet any requirements (including those defined in R17/R18 for RedCap and NTN separately)

- If Alt 1 is adopted, how should the exceptional feature on HD-FDD collision handling (as mentioned in Observation 1) be handled?

- Summary

Proposal 1: Maintain the same principle used in NR NTN for RedCap devices: the requirements are transparent to whether the UE adjusts or not the SMTC window in IDLE/INACTIVE mode.

Observation 1: Based on RAN1 specifications, the priority rules for collision handling of overlapping DL and UL in HD-FDD for RedCap-NTN apply only to Rel-19 UEs and do not include Rel-17 and Rel-18 UEs.

Proposal 2: For UE capabilities of (e)RedCap UEs with NTN, if Option 1 is followed, RAN4 needs to consider the following issues:

- For R17/18 UEs declaring support for RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support for NTN (nonTerrestrialNetwork-r17), should they:

  - Alt 1: Meet the Rel-19 requirements (which would be release-independent from Rel-17), or

  - Alt 2: Not need to meet any requirements (including those defined in R17/R18 for RedCap and NTN separately)

- If Alt 1 is adopted, how should the exceptional feature on HD-FDD collision handling (as mentioned in Observation 1) be handled?

- References

- R4-2508287, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT

- R1-2504975, Introduction of Non-Terrestrial Networks (NTN) for NR Phase 3, Samsung