3GPP TSG RAN WG4 Meeting #116	R4-2510491

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.27.5.2

Source:	vivo

Title:	Discussion on RRM impacts on DL coverage for R19 NR NTN Phase 3

Document for:	Discussion

- Introduction

In the last meeting, RAN4 further analyzed the potential RRM impacts on the objectives of downlink coverage enhancements and the discussion on RAN2 LS [2]. The latest progress was summarized in WF [1], and the corresponding reply LS is shown in [3]. In this contribution, we would like to provide further analysis of the potential RRM impacts on DL coverage.

- 2.	Discussion

## Issue 0: LS from RAN2 on multiple SMTCs with different periodicities Agreement: (Ad-hoc) From RAN4 perspective, it is feasible to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. - In the reply LS (to be prepared by Xiaomi), the following information highlighted in yellow for both intra- and inter-frequency will be added for reference: - FFS on the exact RRM requirement change. - FFS whether to limit the number of SMTCs to no more than 4 is desirable in an earth-moving cell deployment

For the enhancement of supporting configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs, the specific RRM requirement, we understand that this has already been covered by the existing RAN4 spec as highlighted part shown. From the perspective of RAN4 requirements, since R17 has already discussed whether and how to define corresponding delay requirements, we prefer to maintain the existing RAN4 spec without further discussion on whether additional definition or enhancement of requirements is needed for this scenario.

Proposal 1: The existing spec already covers this case (supporting configuration of more than 4 SMTCs per frequency, e.g., 6, for idle/inactive UEs), and no additional RRM requirements change is needed.

For supporting the configuration of two different SMTC periodicities (with different offsets), the exact RRM spec impacts include updates to RRC Re-establishment, RRC Connection Release with Redirection, and Handover regarding the definition of Trs/TSMTC. After enabling two different SMTC periodicities with different offsets, measurements may involve two distinct SMTC periods. For target cell measurements, the larger of the two SMTC periods should be taken as the default value.

Proposal 2: On the enhancement for supporting configuring two different SMTC periodicities (with different offsets), RAN4 to update the Trs/TSMTC assumption defined in RRC Connection Release with Redirection and handover as maximum periodicity of the SMTC occasion

- Summary

Proposal 1: The existing spec already covers this case (supporting configuration of more than 4 SMTCs per frequency, e.g., 6, for idle/inactive UEs), and no additional RRM requirements change is needed.

Proposal 2: On the enhancement for supporting configuring two different SMTC periodicities (with different

offsets), RAN4 to update the Trs/TSMTC assumption defined in RRC Connection Release with Redirection and handover as maximum periodicity of the SMTC occasion

- References

- R4-2508288, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm Incorporated

- R2-2503068, LS on SMTC enhancements, xiaomi

- R4-2508433, Reply LS on SMTC enhancements, xiaomi