3GPP TSG-RAN4 Meeting #116	R4-2510502

# Bengaluru, India, 25th August 2025 – 29th August 2025

Source:			Ericsson

Title:			NTN Ku bands – General aspects

Agenda item:		7.7.2

Document for:		Approval

# Introduction

In last RAN#107, the NTN KU-band WI was revised ([1]) including support for 15 kHz SCS.

The regulation study for the NTN bands n247 and n508 was only done for certain countries in Region 3, not in Region 1 and 2. This contribution is further discussing this aspect.

# Discussion

## NTN Ku bands

US and CEPT regulations were studied in the context of NTN Ku bands n248 and n509.

Until last RAN4#115 meeting, only the Taiwanese regulation was studied for NTN bands n247 and n508 and companies were encouraged to study other regulations for the 13.75-14.0 GHz bands, as captured in the last WF ([2]):

As we already stated in our other contribution ([3]), looking at published ECC Decisions, published ETSI Harmonized Standards and FCC 25.202 (see below), the frequency range 13.75-14.0 GHz has not been allocated to ESIM operation.

Considering this WI’s scope is restricted to Mobile VSAT (i.e. NTN VSAT type 5), this frequency range should then not be considered in the NTN Ku-bands definition for those countries.

Considering we are supposed to finalize the Core part of the NTN Ku-bands WI in this meeting, if no company has contributed in this meeting to share any other regulation study for the NTN bands n247 and n508, RAN4 shall clarify those bands are only applicable to Region 3 and to NTN VSAT type 5.

Observation: If no company as contributed to RAN4#116 meeting and share any other regulatory study for the NTN bands n247 and n508, 2 NTN bands would only be applicable in Region 3 for NTN Mobile VSAT.

And, from this observation, to our understanding, we make the following proposal to clarify the NTN Ku bands context (Japan is highlighted in yellow and should be confirmed by proponent).

Proposal: Agree with the following table capturing which regulations have been studied for each of the new NTN Ku-bands bands, clarifying their applicability.

| NTN Ku-bands | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| n247 / n508 | CEPT countries: No ECC Decision, no EN for ESIM/ESOMP | FCC: Not allocated to ESIM | Taiwan (NS_207N, NS_208N) |
| n248 / n509 | CEPT countries (NS_205/NS_206) | FCC (NS_203N, NS_204N) | Taiwan (NS_207N, NS_208N) Japan (NS_205/NS_206)? |

# Conclusion

In this contribution, we discussed the applicability of NTN bands n248 and n508 considering RAN4 contribution until RAN4#115 meeting. We made the following observation and proposal:

Observation: If no company as contributed to RAN4#116 meeting and share any other regulatory study for the NTN bands n247 and n508, clarify those 2 NTN bands are only applicable in Region 3 for NTN Mobile VSAT.

Proposal: Agree with the following table capturing which regulations have been studied for each of the new NTN Ku-bands bands, clarifying their applicability.

| NTN Ku-bands | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| n247 / n508 | CEPT countries: No ECC Decision, no EN for ESIM/ESOMP | FCC: Not allocated to ESIM | Taiwan (NS_207N, NS_208N) |
| n248 / n509 | CEPT countries (NS_205/NS_206) | FCC (NS_203N, NS_204N) | Taiwan (NS_207N, NS_208N) Japan (NS_205/NS_206)? |

# References

- RP-250799, Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT

- R4-2508778, Way Forward for [115][314] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL

- R4-2501355, NTN Ku bands – system parameters, Ericsson