3GPP TSG-RAN4 Meeting #116	R4-2510509

# Bengaluru, India, 25th August 2025 – 29th August 2025

Source:			Ericsson

Title:			Nominated bandwidth

Agenda item:		7.29.5

Document for:		Approval

# Introduction

In last RAN4#115 meeting, it was proposed to introduce the concept of “nominated bandwidth” which is used in some ETSI hENs, e.g. EN 301 442 ([1]), and RAN4 had some initial discussion on this topic.

In this contribution, we are sharing our view on this concept and how this could be used in RAN4.

# Discussion

## Background

ETSI EN 301 442 ([1]) has introduced the concept of “nominated bandwidth” to help device to comply with the specified unwanted emission mask.

This concept consists of “extending” the channel bandwidth on each side of it (as shown in Figure 1) and make the unwanted emission mask applicable from the edge of this nominated bandwidth. The extension is declared by the manufacturer.

Figure 1: Nominated Bandwidth

EN 301 442 also gives some recommendation on the declaration of these parameters “a” and “b” to optimize the operating frequency range, indicating the maximum values for those parameters. It should be noted that “a” and “b” could have different value or not.

During last RAN4#115 meeting, it was proposed to introduce this concept to help device supporting requirements and reduce the needed A-MPR values to support SEM/other ETSI requirements (or even remove the need for A-MPR).

## Discussion

### Nominated bandwidth in 3GPP

This nominated bandwidth concept helps device supporting some tough requirements, reducing then the device complexity/cost. In fact, this introduces a guard band between the channel bandwidth edge and frequency point where the specified limits apply.

Moreover, in the NTN context, it will improve the link budget by reducing the need for A-MPR. This would be beneficial especially in the IoT NTN context. If RAN4 decides to adopt such concept, RAN4 should further discuss the scope of application, not limiting it to one band operating IoT TDD NTN.

Observation1: The “nominated bandwidth” concept would help device supporting stringent requirements, reducing cost and complexity. It would improve the link budget by reducing the A-MPR need), which would be beneficial for IoT NTN, not only for one band operating IoT TDD NTN.

As the limits only apply at the edge of this nominated bandwidth (no limit is specified inside it), this nominated bandwidth shall be used as the actual bandwidth when deploying such device, this to avoid any interference that would be generated by this device.

Observation2: If introduced in 3GPP, the network deployment and channel allocation shall consider the “nominated bandwidth” and not the channel bandwidth.

This would need to be clarified in the 3GPP specification.

As it was already highlighted in last RAN4#115 meeting’s online discussion, the nominated bandwidth shall not be based on manufacturer declaration and the “a” and “b” parameters should be equal, with a value specified in the 3GPP specification. If not, it would be difficult to thoughtfully planned network deployment and its channel allocation. This will also ensure the UE ecosystem remains homogeneous.

Proposal1: If introduced in 3GPP, the parameters “a” and “b” from the nominated bandwidth’s concept shall be equal and shall have a unique value specified in 3GPP specifications.

To enable a smoother deployment of this nominated bandwidth’s concept, preserving the full flexibility of the channel allocation and network deployment, we also support the proposal made in last RAN#4 meeting to control it via network signalling (NS):

- If the NS is not broadcasted by the network, the UE shall fulfil the SEM at the bandwidth edge, as it’s specified today. This means that UE might then need high A-MPR to comply with this requirement.

- If the NS is broadcasted, the UE knows it could consider the specified nominated bandwidth, applying the SEM at the edge of this nominated bandwidth and then reducing (or even removing) the need for any A-MPR.

Proposal2: The usage of the nominated bandwidth concept should be controlled via NS signalling: the SEM will apply at the edge of the nominated bandwidth only of the new NS is broadcasted.

### ETSI TC SES LS

RAN4 received a LS from ETSI TC SES (1) sharing the progress of the NTN and IoT-NTN UE Harmonized Standards. In this LS, TC SES mentioned the “nominated bandwidth” concept, welcoming any feedback from 3GPP.

Based on the above discussion, RAN4 should suggest to TC SES the modifications listed in the above proposals, i.e.:

- The nominated bandwidth should be enabled via NS signalling.

- The “a” and “b” parameters should not be declared by the manufacturer but specified in the 3GPP specification.

Proposal3: RAN4 reply to TC SES LS suggesting the modifications listed in the above proposals:

- The “a” and “b” parameters should be equal, specified in 3GPP specifications and not declared by manufacturer.

- The nominated channel BW is enabled via NS signalling.

# Conclusion

In this contribution, we discussed the nominated bandwidth concept and how it could be applied in 3GPP . We made the following observations and proposals:

Observation1: The “nominated bandwidth” concept would help device supporting stringent requirements, reducing cost and complexity. It would improve the link budget by reducing the A-MPR need), which would be beneficial for IoT NTN, not only for one band operating IoT TDD NTN.

Observation2: If introduced in 3GPP, the network deployment and channel allocation shall consider the “nominated bandwidth” and not the channel bandwidth.

Proposal1: If introduced in 3GPP, the parameters “a” and “b” from the nominated bandwidth’s concept shall be equal and shall have a unique value specified in 3GPP specifications.

Proposal2: The usage of the nominated bandwidth concept should be controlled via NS signalling: the SEM will apply at the edge of the nominated bandwidth only of the new NS is broadcasted.

Proposal3: RAN4 reply to TC SES LS suggesting the modifications listed in the above proposals:

- The “a” and “b” parameters should be equal, specified in 3GPP specifications and not declared by manufacturer.

- The nominated channel BW is enabled via NS signalling.

# References

- EN 301 442, Harmonised Standard for NGSO Mobile Earth Stations (MES) including handheld earth stations, for Satellite Personal Communications Networks (S-PCN) operating in the 1 980 MHz to 2 010 MHz (earth-to-space) and 2 170 MHz to 2 200 MHz (space-to-earth) frequency bands under the Mobile Satellite Service (MSS) covering the essential requirements of article 3.2 of the Directive 2014/53/EU

- R4-2509023, LS on Harmonised Standard for NTN capable UE, ETSI TC SES