# 3GPP TSG-RAN WG4 Meeting #116	R4-25010520
Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.1.2.2.1

Source: 	OPPO

Title: 	Discussion on MPR reduction for single carrier

Document for:	Discussion

- Introduction

In RAN4 #114bis meeting, the power domain enhancements for single carrier were discussed and some agreements were approved in WF [1], and some issues need further discuss in this meeting.

This contribution further discusses power domain enhancement for single carrier based on the open issues.

- Discussion

Issue 1-1-1: Extension values/ratios

- Agreement in main session:

- At least ½ and ¼ are introduced as the extension ratio to apply to both single-sided and dual-sided cases

- It is FFS on the details of symmetric and asymmetric cases for dual-sided case where non-zero extension is applied for both sides.

- It is FFS on details of the single side case.

There are still two open issues for extension ratios, from our side, it’s unnecessary to define the asymmetric cases for dual-sided, since the gain is very small.

Proposal 1: don’t introduce asymmetric extension ratios for dual-sided case.

As analysis in our contribution [1], the UE will get the biggest MPR reduction region when the extension ratio gets up to 1/3 for single-sided extension as shown in figure 2-1, there is no need to extend it to 1/2. Therefore, we prefer to introduce one more extension ratio 1/3, like this, the NW can configure 1/3 single-side extension to let UE get the largest MPR reduction region, when NW haven’t up to1/2 extension ratio frequency ranges.

Figure 2-1 New inner region when single-sided extension ratio ≥ 1/3

Proposal 2: Introduce one more extension ratio 1/3.

In last meeting, it’s agreed that for the extended IBE:

- Fixed IBE used for extended CBW at each side of UE CBW as the baseline, the value is derived from the outmost allocated RB of UE CBW

- Using BPSK EVM in the IBE formula for all extended RBs

And whether this extended IBE requirement apply to the GB for extended CBW still need further confirm. we can’t see the difference between only RB based extension and both of RB based and CBW based extension, if the extended IBE requirement apply to the GB for extended CBW. We prefer to only define RB based extension, that is the extended OOB starting point should be based on the extended RB number.

Proposal 3: There is no need to introduce extended CBW and related GB for extended CBW in Spec for this feature. Only define related requirements based on extended RB.

Proposal 4: Only testing the dual-sided symmetric extension ratios is enough.

Based on above analysis, introducing follow two capabilities are enough:

- Capability 1: Indicates whether UE supports MPR reduction for single UL carrier with 1/2*[UE CBW] extension for dual-sided symmetric and single-sided case:

    - {1/2, 0}, {0, 1/2}, {1/2, 1/2}

- Capability 2: Indicates whether UE supports MPR reduction for single UL carrier with 1/X*UE CBW extension for dual-sided symmetric and single-sided case, where 1/X is a set of ratios equal and smaller than 1/2:

    - {1/2, 0}, {0, 1/2}, {1/2, 1/2}, {1/4, 0}, {0, 1/4}, {1/4, 1/4}, {1/3, 0}, {0, 1/3}, {1/3, 1/3}

Proposal 5: Introducing follow two capabilities:

- Capability 1: Indicates whether UE supports MPR reduction for single UL carrier with 1/2*[UE CBW] extension for dual-sided symmetric and single-sided case:

    - {1/2, 0}, {0, 1/2}, {1/2, 1/2}

- Capability 2: Indicates whether UE supports MPR reduction for single UL carrier with 1/X*UE CBW extension for dual-sided symmetric and single-sided case, where 1/X is a set of ratios equal and smaller than 1/2:

    - {1/2, 0}, {0, 1/2}, {1/2, 1/2}, {1/4, 0}, {0, 1/4}, {1/4, 1/4}, {1/3, 0}, {0, 1/3}, {1/3, 1/3}

- Conclusion

This paper analyzed the power enhancements by emission relaxation in the scenario of single operator in a band, and proposed:

Proposal 1: don’t introduce asymmetric extension ratios for dual-sided case.

Proposal 2: Introduce one more extension ratio 1/3.

Proposal 3: There is no need to introduce extended CBW and related GB for extended CBW in Spec for this feature. Only define related requirements based on extended RB.

Proposal 4: Only testing the dual-sided symmetric extension ratios is enough.

Proposal 5: Introducing follow two capabilities:

- Capability 1: Indicates whether UE supports MPR reduction for single UL carrier with 1/2*[UE CBW] extension for dual-sided symmetric and single-sided case:

    - {1/2, 0}, {0, 1/2}, {1/2, 1/2}

- Capability 2: Indicates whether UE supports MPR reduction for single UL carrier with 1/X*UE CBW extension for dual-sided symmetric and single-sided case, where 1/X is a set of ratios equal and smaller than 1/2:

    - {1/2, 0}, {0, 1/2}, {1/2, 1/2}, {1/4, 0}, {0, 1/4}, {1/4, 1/4}, {1/3, 0}, {0, 1/3}, {1/3, 1/3}

- Reference

[1] R4-2507319 R19 MPR reduction for single carrier, OPPO, RAN4 #115