3GPP TSG-RAN WG4 Meeting # 116		                                         R4-2510549

Bengaluru, India, 25 - 29 August 2025

Title: 					Discussion on CSSF SA mode requirements for outside gaps in R15

Source: 				Huawei, HiSilicon

Agenda item:	4.4.1

Document for:	Discussion

Introduction

In Chapter 9.1.5.1, the requirement for a CSSF value in a UE configured with different modes is specified. During the RRM phase 5 discussion of the enhanced CSSF approach, a potentially incorrect description was discovered in chapter 9.1.5.1.2.

Discussion

This paper is aimed at discussing the correction of the CSSF requirement in the SA mode. The key point of this issue is the CSSF configuration on SA and NE-DC modes. In chapter 9.1.5.2, there is one contradiction in the description of one sentence. Here is the content from TS 38.133 V15.30.0:

##### 9.1.5.1.2 SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps For UE in SA operation mode, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.2-1, which shall also be applied for a UE configured with NE-DC operation.

Tracing the Roots

To figure out why this sentence has been added in the SC mode section. The first step is to figure out when this sentence was added to the spec. Here is the NR spec (TS 38.133 V15.5.0) content:

##### 9.1.5.1.2 SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps without NR-DC operation For UE not configured with NR-DC operation in SA mode, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.2-1, which shall also be applied for a UE configured with NE-DC operation. Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor without NR-DC operation in SA mode Scenario; CSSFoutside_gap,i for FR1 PCC; CSSFoutside_gap,i for FR1 SCC; CSSFoutside_gap,i for FR2 PCC; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required. - FR1 only CA; 1; Number of configured FR1 SCell(s); N/A; N/A; N/A. - FR2 only intra band CA; N/A; N/A; 1; N/A; Number of configured FR2 SCell(s). - FR1 +FR2 CA (FR1 PCell) Note 1; 1; 2×(Number of configured SCell(s)-1); N/A; 2; 2×(Number of configured SCell(s)-1). Note 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows section 9.2.3.2.. ##### 9.1.5.1.3 SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps with NR-DC operation For UE configured with NR-DC operation in SA mode, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.3-1. Table 9.1.5.1.3-1: CSSFoutside_gap,i scaling factor with NR-DC operation in SA mode Scenario; CSSFoutside_gap,i for FR1 PCC; CSSFoutside_gap,i for FR1 SCC; CSSFoutside_gap,i for FR2 PSCC; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required. - FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell) Note 1; 1; 2×(Number of configured SCell(s)); 2; 2×(Number of configured SCell(s)). Note 1: NR-DC in Rel-15 only includes the scenarios where all serving cells in MCG in FR1 and all serving cells in SCG in FR2..

According to the Rel-15 V15.5.0 spec, the sentence “which shall also be applied for a UE configured with NE-DC operation” is to indicate that the CSSF Table 9.1.5.1.2-1 is applicable to both NE-DC CA operation and SA CA operation.

In the next version (TS 38.133 V15.6.0) of the spec, the CSSF has been designed in 3 categories, but there is no individual section for NE-DC. The description of the NE-DC using the same table as SA is reasonable.  The content captured from V15.6 spec is as follows:

##### 9.1.5.1.1 EN-DC mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps For UE configured with the E-UTRA-NR dual connectivity operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.1-1. Table 9.1.5.1.1-1: CSSFoutside_gap,i scaling factor Scenario; CSSFoutside_gap,i for FR1 PSCC; CSSFoutside_gap,i for FR1 SCC; CSSFoutside_gap,i for FR2 PSCC; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required Note 2; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required. EN-DC with FR1 only CA; 1; Number of configured FR1 SCell(s); N/A; N/A; N/A. EN-DC with FR2 only intra band CA; N/A; N/A; 1; N/A; Number of configured FR2 SCells. EN-DC with FR1 +FR2 CA (FR1 PSCell) Note 1; 1; 2×(Number of configured SCell(s)-1); N/A; 2; 2×(Number of configured SCell(s)-1). EN-DC with FR1 +FR2 CA (FR2 PSCell) Note 1; N/A; Number of configured SCell(s); 1; N/A; Number of configured SCell(s). Note 1: Only one NR FR1 operating band and one NR FR2 operating band are included for FR1+FR2 inter-band EN-DC. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2.. ##### 9.1.5.1.2 SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps without NR-DC operation For UE not configured with NR-DC operation in SA mode, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.2-1, which shall also be applied for a UE configured with NE-DC operation. Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor without NR-DC operation in SA mode Scenario; CSSFoutside_gap,i for FR1 PCC; CSSFoutside_gap,i for FR1 SCC; CSSFoutside_gap,i for FR2 PCC; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required. FR1 only CA; 1; Number of configured FR1 SCell(s); N/A; N/A; N/A. FR2 only intra band CA; N/A; N/A; 1; N/A; Number of configured FR2 SCell(s). FR1 +FR2 CA (FR1 PCell) Note 1; 1; 2×(Number of configured SCell(s)-1); N/A; 2; 2×(Number of configured SCell(s)-1). Note 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2.. ##### 9.1.5.1.3 SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps with NR-DC operation For UE configured with NR-DC operation in SA mode, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.3-1. Table 9.1.5.1.3-1: CSSFoutside_gap,i scaling factor with NR-DC operation in SA mode Scenario; CSSFoutside_gap,i for FR1 PCC; CSSFoutside_gap,i for FR1 SCC; CSSFoutside_gap,i for FR2 PSCC; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required. FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell) Note 1; 1; 2×(Number of configured SCell(s)); 2; 2×(Number of configured SCell(s)). Note 1: NR-DC in Rel-15 only includes the scenarios where all serving cells in MCG in FR1 and all serving cells in SCG in FR2..

Root Cause Confirmed

In the TS 38.133 V15.7.0 spec, the category of CSSF outside gap measurement has been redesigned. There are four individual sections for NE-DC, EN-DC, SA and NR-DC.

##### 9.1.5.1.1 EN-DC mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps For UE configured with the E-UTRA-NR dual connectivity operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.1-1. Table 9.1.5.1.1-1: CSSFoutside_gap,i scaling factor Scenario; CSSFoutside_gap,i for FR1 PSCC; CSSFoutside_gap,i for FR1 SCC; CSSFoutside_gap,i for FR2 PSCC; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required Note 2; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required. EN-DC with FR1 only CA; 1; Number of configured FR1 SCell(s); N/A; N/A; N/A. EN-DC with FR2 only intra band CA; N/A; N/A; 1; N/A; Number of configured FR2 SCells. EN-DC with FR1 +FR2 CA (FR1 PSCell) Note 1; 1; 2×(Number of configured SCell(s)-1); N/A; 2; 2×(Number of configured SCell(s)-1). EN-DC with FR1 +FR2 CA (FR2 PSCell) Note 1; N/A; Number of configured SCell(s); 1; N/A; Number of configured SCell(s). Note 1: Only one NR FR1 operating band and one NR FR2 operating band are included for FR1+FR2 inter-band EN-DC. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2.. ##### 9.1.5.1.2 SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps without NR-DC operation For UE not configured with NR-DC operation in SA mode, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.2-1, which shall also be applied for a UE configured with NE-DC operation. Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor without NR-DC operation in SA mode Scenario; CSSFoutside_gap,i for FR1 PCC; CSSFoutside_gap,i for FR1 SCC; CSSFoutside_gap,i for FR2 PCC; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required. FR1 only CA; 1; Number of configured FR1 SCell(s); N/A; N/A; N/A. FR2 only intra band CA; N/A; N/A; 1; N/A; Number of configured FR2 SCell(s). FR1 +FR2 CA (FR1 PCell) Note 1; 1; 2×(Number of configured SCell(s)-1); N/A; 2; 2×(Number of configured SCell(s)-1). Note 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2.. ##### 9.1.5.1.3 SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps with NR-DC operation For UE configured with NR-DC operation in SA mode, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.3-1. Table 9.1.5.1.3-1: CSSFoutside_gap,i scaling factor with NR-DC operation in SA mode Scenario; CSSFoutside_gap,i for FR1 PCC; CSSFoutside_gap,i for FR1 SCC; CSSFoutside_gap,i for FR2 PSCC; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required. FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell) Note 1; 1; 2×(Number of configured SCell(s)); 2; 2×(Number of configured SCell(s)). Note 1: NR-DC in Rel-15 only includes the scenarios where all serving cells in MCG in FR1 and all serving cells in SCG in FR2..

In this version, the description of the sentence, which shall also be applied for a UE configured with NE-DC operation, is not correct because the NE-DC has a corresponding section to introduce the CSSF table and requirements.

In conclusion, this sentence, which shall also be applied for a UE configured with NE-DC operation, needs to be removed from the spec. NE-DC and SA operation has separate sections to introduce the requirements, and NE-DC and SA operations are not able to share the same CSSF table.

Proposal 1: The CSSF requirements of NE-DC and SA operation should be introduced in separate sections, and NE-DC and SA operations do not share the same CSSF table or requirements.

- Conclusions

Proposal 1: The CSSF requirements of NE-DC and SA operation should be introduced in separate sections, and NE-DC and SA operations do not share the same CSSF table or requirements.

References

- 3GPP TS 38.133 V15.4.0 (2018-12)

- 3GPP TS 38.133 V15.5.0 (2019-03)

- 3GPP TS 38.133 V15.6.0 (2019-06)

- 3GPP TS 38.133 V15.7.0 (2019-09)