3GPP TSG-RAN WG4 Meeting # 116	R4-2510550

Bengaluru, India, 25 - 29 August 2025

Agenda item:	7.20.4.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 MIMO on other aspects

Document for:	Discussion

- Introduction

The Rel-19 MIMO WI is approved in [1]. In this contribution, we further provide our views regarding the RAN4 impacts for the WI.

- Discussion

UE reporting enhancement for CJT calibration

In the last meeting, the CJTC reporting has a conclusion that defines the DO and FO core requirements. The updated info captured from WF [2] is shown below:

Issue 1-1: How to define measurement period requirements for delay and frequency offset reporting for CJT calibration? Agreement: - RAN4 to discuss the achievable accuracy in terms of delay and frequency offsets based on 1 sample and [3 or 5] samples in the next meeting. - RAN4 to conclude the delay requirement for delay and frequency offset reporting in the next meeting. The delay requirement for phase offset is deprioritized in Rel-19.

In this paper, we provide our analysis of how many samples are needed for the Frequency offset and Delay offset. Since RAN4 #114 meeting, companies have provided simulation results on CJTC reporting. Based on the simulations that companies provided, the estimation error levels for FO/DO in different samples showed different performance.

There is no target accuracy for the network to verify whether the CJT estimation results work well. This approach to estimating the CJT performance is essential, but it seems there is no more time for the core part discussion. We would like to provide one method to verify the selection of the sample amount. To figure out the improvement of different samples, the calculation of phase difference is able to be set as the scale. Using the simulation result below as an example.

| Delay offset: 0.5 CP SCS: 15k Hz |  |  |  |
| --- | --- | --- | --- |
| SNR | 1 Sample | 3 Samples | 5 Samples |
|  | Error (us) | Error (us) | Error (us) |
| -3 | 0.5 | 0.36 | 0.3 |

The phase shift caused by estimation error, when transitioning from using 1 sample to 3 samples, decreased by 0.75 degrees, and from using 1 sample to 5 samples, it is reduced by 1.08 degrees. This shows that the increase in the sample shows a slight improvement in phase shift on the network side.

Observation 1: The increase in the sample shows a slight improvement in phase shift.

About the error accuracy of the CJT estimate from the UE side, the network allows the UE to report a delay offset that should not be larger than CP, and the frequency offset report result should not be larger than 0.2 ppm. In our view, the error tolerances of the CJT estimation results are UE performance and network implementation. We have no recommended target accuracy for delay offset and frequency offset. The UE final results of delay and frequency offset are supposed to be less than AD and AFO.

Observation 2: The error tolerances of the CJT estimation results are UE performance and network implementation. We have no recommended target accuracy for delay offset and frequency offset. The UE final results of delay and frequency offset are supposed to be less than AD and AFO.

In terms of the agreement from the last meeting, at least to define the accuracy for 1 sample. Based on the simulation results in the R4-2508292, the accuracy for delay and frequency offset estimation in different conditions can be defined as:

| Frequency offset |  |  |  |
| --- | --- | --- | --- |
| Accuracy | Conditions |  |  |
| Max Estimation error | SNR | SCS | Sample |
| level | dB | KHz |  |
| 42 | 0 | 15 | 1 |
| 35 | +3 |  |  |
| 108 | 0 | 30 |  |
| 70 | +3 |  |  |

| Delay offset |  |  |  |
| --- | --- | --- | --- |
| Accuracy | Conditions |  |  |
| Max Estimation error | SNR | SCS | Sample |
| level | dB | KHz |  |
| 20 | 0 | 15 | 1 |
| 15 | +3 |  |  |
| 18 | 0 | 30 |  |
| 16 | +3 |  |  |

These accuracies are defined with a little tolerance, and is able to be further discussed.

Proposal 1: One optional accuracy requirement for delay and frequency offset estimation in different conditions can be defined as:

| Frequency offset |  |  |  |
| --- | --- | --- | --- |
| Accuracy | Conditions |  |  |
| Normal condition | SNR | SCS | Sample |
| level | dB | KHz |  |
| 42 | 0 | 15 | 1 |
| 35 | +3 | 15 | 1 |
| 108 | 0 | 30 | 1 |
| 70 | +3 | 30 | 1 |

| Delay offset |  |  |  |
| --- | --- | --- | --- |
| Accuracy | Conditions |  |  |
| Normal condition | SNR | SCS | Sample |
| level | dB | KHz |  |
| 20 | 0 | 15 | 1 |
| 15 | +3 | 15 | 1 |
| 18 | 0 | 30 | 1 |
| 16 | +3 | 30 | 1 |

These accuracy values are defined with a little tolerance, and is able to be further discussed.

Enhancement for asymmetric DL sTRP/UL mTRP Scenarios

At the recent RAN4 meeting, the requirements for DL sTRP/UL mTRP Scenarios 1 and 2 were discussed. In this paper, we will provide our views on the remaining issue in detail.

Scenario 1

Here is the last meeting’s WF[1] for Scenario 1.

Issue 2-1: For Scenario#1, whether to update RACH requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Way forward: FFS Issue 2-2: For Scenario#1, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Way forward: - Option 1: Gradual timing adjustment applies for each TAG and shall be met for each TAG. - Option 2: The gradual timing adjustment requirements are only applicable to TRP with both UL and DL. Issue 2-3: For Scenario#1, how to define MRTD and MTTD requirements? Way forward: FFS

The PRACH requirements are related to the downlink reference timing. In Scenario 1, the two TRPs maintain one single downlink reference timing. So the timing for PRACH is able to take place (NTA + NTA offset) x Tc before the reception of the downlink signal of one DL/UL TRP. The UL TRP downlink reference signal is associated with the DL or Joint TCI state. Therefore, the PRACH requirements for Scenario 1 can be updated as:

The uplink transmission(s) timing for PRACH takes place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signal(s) of the reference cell associated with DL or Joint TCI state.

Proposal 2: The PRACH requirements for Scenario 1 can be defined as:

The uplink transmission(s) timing for PRACH takes place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signal(s) of the reference cell associated with DL or Joint TCI state.

According to the two TA requirement for Scenario 1, the gradual timing adjustment requirement also needs to be updated. Due to there are two TAGs in Scenario 1, the timing adjustment is supposed to apply to each TAG. For Scenario 1, for both TAGs, the gradual timing adjustment applies to each TAG and shall be met for each TAG.

Proposal 3: For Scenario 1, for both TAGs, the gradual timing adjustment applies to each TAG and shall be met for each TAG.

The MRTD and MTTD for multi-TRP are the Rx timing difference between the two DL reference timings. For Scenario 1, the UE maintains only one reference timing, so there is no need to define MRTD or MTTD for Scenario 1.

Proposal 4: There is no need to define MRTD or MTTD for Scenario 1.

Scenario 2

In response to Scenario #2, RAN1 LS has provided its official reply in document R1-2502956. The RAN4 spec should match the RAN1 LS wording:

- For the Scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs: - The UE maintains two downlink reference timings; - Baseline assumption for this feature is that Rx timing difference between two DL reference timings is no larger than one CP length, while it is subject to optional UE capability that the Rx timing difference between two DL reference timings can be assumed to be larger than CP length. - The reference point for PRACH transmission is indicated as follows: - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG

Observation 3: In the RAN1 LS wording, TCI state in the requirements of Scenario 2 using the DL TCI stete(s) only.

In terms of the RAN1 LS, UE maintains two reference timings and expects to receive SSB means this TRP will be a traditional UL/DL TRP when UE is not configured with PL offset. That means UE will and is able to receive the downlink signal on each TRP, so introducing the UL TCI state in Scenario 2 is redundant and not correct. Therefore, reference timing requirements for PUCCH/PUSCH/SRS/PRACH/PRACH in Scenario 2 should be defined as follows:

------------------------------------------------------------Text proposal-------------------------------------------------------------------

- For asymmetric DL sTRP/UL mTRP deployment with two TAs, for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), for the case UE is configured with SSB-MTC-additionalPCI, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState TS 38.331 [2] having the same TAG as the uplink signal.

- For  asymmetric DL sTRP/UL mTRP deployment with two TAs, for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), for the case UE is configured with SSB-MTC-additionalPCI, the reference point for PRACH transmission triggered by PDCCH order,

-	if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

-	if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL or TCI state(s) of the reference cell associated with the second TAG.

------------------------------------------------------------Text proposal-------------------------------------------------------------------

Proposal 5: According to RAN1 LS content, the reference timing requirements for Scenario 2 is supposed to be defined as follows:

------------------------------------------------------------Text proposal-------------------------------------------------------------------

- For asymmetric DL sTRP/UL mTRP deployment with two TAs, for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), for the case UE is configured with SSB-MTC-additionalPCI, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState TS 38.331 [2] having the same TAG as the uplink signal.

- For  asymmetric DL sTRP/UL mTRP deployment with two TAs, for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), for the case UE is configured with SSB-MTC-additionalPCI, the reference point for PRACH transmission triggered by PDCCH order,

-	if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

-	if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL or TCI state(s) of the reference cell associated with the second TAG.

------------------------------------------------------------Text proposal-------------------------------------------------------------------

- Conclusions

Observation 1: The increase in the sample shows a slight improvement in phase shift.

Observation 2: The error tolerances of the CJT estimation results are UE performance and network implementation. We have no recommended target accuracy for delay offset and frequency offset. The UE final results of delay and frequency offset are supposed to be less than AD and AFO.

Proposal 1: One optional accuracy requirement for delay and frequency offset estimation in different conditions can be defined as:

| Frequency offset |  |  |  |
| --- | --- | --- | --- |
| Accuracy | Conditions |  |  |
| Normal condition | SNR | SCS | Sample |
| level | dB | KHz |  |
| 42 | 0 | 15 | 1 |
| 35 | +3 | 15 | 1 |
| 108 | 0 | 30 | 1 |
| 70 | +3 | 30 | 1 |

| Delay offset |  |  |  |
| --- | --- | --- | --- |
| Accuracy | Conditions |  |  |
| Normal condition | SNR | SCS | Sample |
| level | dB | KHz |  |
| 20 | 0 | 15 | 1 |
| 15 | +3 | 15 | 1 |
| 18 | 0 | 30 | 1 |
| 16 | +3 | 30 | 1 |

These accuracy values are defined with a little tolerance, and is able to be further discussed.

Proposal 2: The PRACH requirements for Scenario 1 can be defined as:

The uplink transmission(s) timing for PRACH takes place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signal(s) of the reference cell associated with DL or Joint TCI state.

Proposal 3: For Scenario 1, for both TAGs, the gradual timing adjustment applies to each TAG and shall be met for each TAG.

Proposal 4: There is no need to define MRTD or MTTD for Scenario 1.

Observation 3: In the RAN1 LS wording, TCI state in the requirements of Scenario 2 using the DL TCI stete(s) only.

Proposal 5: According to RAN1 LS content, the reference timing requirements for Scenario 2 is supposed to be defined as follows:

------------------------------------------------------------Text proposal-------------------------------------------------------------------

- For asymmetric DL sTRP/UL mTRP deployment with two TAs, for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), for the case UE is configured with SSB-MTC-additionalPCI, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState TS 38.331 [2] having the same TAG as the uplink signal.

- For  asymmetric DL sTRP/UL mTRP deployment with two TAs, for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), for the case UE is configured with SSB-MTC-additionalPCI, the reference point for PRACH transmission triggered by PDCCH order,

-	if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

-	if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL or TCI state(s) of the reference cell associated with the second TAG.

------------------------------------------------------------Text proposal-------------------------------------------------------------------

4. References

[1] RP-242394 WID: NR MIMO Phase 5

[2] R4-2508272 WF on RRM requirements for NR_MIMO_Ph5_Part2