3GPP TSG-RAN4 Meeting #116	R4-2510570

# Bengaluru, India, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5868 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_unlic-Perf) CR on maintenance for NR SCell under CCA performance tests R19 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Perf |  |  |  |  |  | Date: |  |  | 2025-09-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In test cases of NR Cell under CCA, several tables show the parameter with the incorrect band group notations. Based on the clause in chapter 3.5.2, some band group notations are missing in these tables. Here are all the band group notations for NR Cell in chapter 3.5.2. Table 3.5.2-1: NR frequency band groups for FR1 Group; NR FDD; NR TDD; NR SDL; NR CCA10. Band group notation; Operating bands; Band group notation; Operating bands; Band group notation; Operating bands; Band group notation; Operating bands. A; NR_FDD_FR1_A; n1, n18, n24, n70, n744, n91, n92, n93, n94, n100, n109; NR_TDD_FR1_A; n34, n389, n39, n40, n50, n51, n53, n54, n101; NR_SDL_FR1_A; n67, n75, n76; NR_CCA_FR1_A; -. B; NR_FDD_FR1_B; n65, n66, n743; NR_TDD_FR1_B; n387; NR_SDL_FR1_B; -; NR_CCA_FR1_B; -. C; NR_FDD_FR1_C; n30; NR_TDD_FR1_C; n48, n771, n78, n79; NR_SDL_FR1_C; -; NR_CCA_FR1_C; -. D; NR_FDD_FR1_D; n28; NR_TDD_FR1_D; n772; NR_SDL_FR1_D; -; NR_CCA_FR1_D; -. E; NR_FDD_FR1_E; n2, n5, n7; NR_TDD_FR1_E; n41, n90; NR_SDL_FR1_E; -; NR_CCA_FR1_E; -. F; NR_FDD_FR1_F; n266; NR_TDD_FR1_F; -; NR_SDL_FR1_F; -; NR_CCA_FR1_F; -. G; NR_FDD_FR1_G; n3, n8, n12, n13, n1412,, n20, n71, n85, n10511, n106; NR_TDD_FR1_G; n104; NR_SDL_FR1_G; n29; NR_CCA_FR1_G; -. H; NR_FDD_FR1_H; n25; NR_TDD_FR1_H; -; NR_SDL_FR1_H; -; NR_CCA_FR1_H; -. I; NR_FDD_FR1_I; -; NR_TDD_FR1_I; NR_SDL_FR1_I; -; NR_CCA_FR1_I; n46. J; NR_FDD_FR1_J; -; NR_TDD_FR1_J; n478,12; NR_SDL_FR1_J; -; NR_CCA_FR1_J; n96, n102. K; NR_FDD_FR1_K; -; NR_TDD_FR1_K; -; NR_SDL_FR1_K; -; NR_CCA_FR1_K; -. L; NR_FDD_FR1_L; -; NR_TDD_FR1_L; -; NR_SDL_FR1_L; -; NR_CCA_FR1_L; -. M; NR_FDD_FR1_M; -; NR_TDD_FR1_M; -; NR_SDL_FR1_M; -; NR_CCA_FR1_M; -. N; NR_FDD_FR1_N; n31, n72; NR_TDD_FR1_N; -; NR_SDL_FR1_N; -; NR_CCA_FR1_N; -. NOTE 1: Except 3.8 GHz to 4.2 GHz. NOTE 2: Only 3.8 GHz to 4.2 GHz. NOTE 3: Except 1475.9 MHz to 1510.9 MHz. NOTE 4: Only when the band is confined in 1475.9 MHz to 1510.9 MHz. NOTE 5: These bands are used only in NR carrier aggregation with other NR bands according to NR CA band combinations specified in TS 38.101-1 [18] and TS 38.101-3 [20]. NOTE 6: The minimum Io condition is reduced by 0.5 dB when the carrier frequency of the assigned NR channel bandwidth is within 865-894 MHz. NOTE 7: When this band is used for V2X SL service, the band is exclusively used for NR V2X in particular regions [18]. NOTE 8: This band is unlicensed band used for V2X service. There is no expected network deployment in this band. [18]. NOTE 9: When this band is only used for WAN service. NOTE 10: Operating bands where operation on carrier frequencies with CCA is supported. NOTE 11: The minimum Io condition is reduced by 0.5 dB when the downlink channel overlap the 612-617 MHz frequency range and the channel bandwidth is 5 MHz. NOTE 12: This band can be used for V2X SL operation.. The band group for CCA is n46 and n96. The following band combination of CA is related to the CCA band that was captured in the spec 38.101-1 Chapter 5.2A. The band combinations that include the NR cell and NR_U cell are shown as follows: NR_CCA_FR1_I: CA_n1-n46 CA_n7-n46 CA_n25-n46 CA_n28-n46 CA_n46-n48 CA_n46-n66 CA_n46-n77 CA_n46-n78 CA_n46-n96 CA_n46-n102 NR_CCA_FR1_J: CA_n48-n96 CA_n1-n102 CA_n3-n102 CA_n7-n102 CA_n28-n102Therefore, the test cases for NR cell under CCA band group notations in parameters should match these CA band combinations. |
| Summary of change: |  | The changes are in following part: - Modify the band group notations in the tables. - Remove the redundant band group notations in the tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | All the band group related test cases are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.11.6.2; A.11.6.3; A.13.4.2; A.13.4.3; A.13.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1

<Skipped unchanged part>

A.11.6	Measurement performance

<Skipped unchanged part>

A.11.6.2	SS-RSRQ

A.11.6.2.1	Intra-frequency measurement accuracy

A.11.6.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.29.1.1.

A.11.6.2.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in table A.11.6.2.1.2-1. The absolute accuracy of SS-RSRQ intra-frequency measurement is tested by using the parameters in table A.11.6.2.1.2-2. In all test cases, Cell 1 is the PCell with CCA and Cell 2 is the target cell with CCA. Three sub-tests (Test 1, Test 2, and Test 3) are provided different Noc on Cells 1 and 2.

Table A.11.6.2.1.2-1: SS-RSRQ Intra frequency SS-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.6.2.1.2-2: SS-RSRQ Intra frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  |  | freq1 |  |  |  |
| DL CCA model |  | Config 1 |  | As specified in clause A.3.26.2.1 |  |  |  |
| UL CCA model |  | Config 1 |  | As specified in clause A.3.26.2.2 |  |  |  |
| DL CCA probability for semi-static channel access Note 7, 8 |  | PCCA_UL |  | 1.0 | - | 1.0 | - |
|  |  | PCCA_DL |  | 0.9375 | - | 0.9375 | - |
| DL CCA probability for dynamic channel access Note 8, 9 |  | PCCA_DL_1 |  | 0.75 | - | 0.75 | - |
|  |  | PCCA_DL_2 |  | 0.75 | - | 0.75 | - |
| Duplex mode |  | Config 1 |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |  |  |
| BWchannel |  | Config 1 | MHz | 40: NRB,c = 106 |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 CCA |  | SR.1.1 CCA |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA |  | CR.1.1 CCA |  |
| Control Channel RMC |  | Config 1 |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  |
| TRS configuration |  | Config 1 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OP. 1 |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |
| Time offset with Cell 1 |  | Config 1 | s | - | 3 | - | 3 |
| DBT Window Configuration |  | Config 1 |  | DBT.1 (As defined in A.3.28.1) |  |  |  |
| SSB configuration |  | Config 1 |  | SSB.1 CCA for semi-static channel access SSB.2 CCA for dynamic channel access |  |  |  |
| SMTC configuration |  | Config 1 |  | SMTC.1 |  |  |  |
| PDSCH/PDCCH |  | Config 1 | kHz | 30kHz |  |  |  |
| subcarrier spacing |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 | Config1 | NR_CCA_FR1_I | dBm/15kHz | -91 |  | -110 |  |
|  |  | NR_CCA_FR1_J |  |  |  | -109.5 |  |
| Note2 | Config 1 | NR_CCA_FR1_I | dBm/SCS | -88 |  | -107 |  |
|  |  | NR_CCA_FR1_J |  |  |  | -106.5 |  |
|  |  |  | dB | -1.76 |  | -5.46 | -5.46 |
|  |  |  | dB | 3 | 3 | -4 | -4 |
| SS-RSRPNote3 | Config 1 | NR_CCA_FR1_I | dBm/SCS | -85 | -85 | -111 | -111 |
|  |  | NR_CCA_FR1_J |  |  |  | -110.5 | -110.5 |
| SS-RSRQ Note3 |  | NR_CCA_FR1_I | dB | -14.77 | -14.77 | -17.34 | -17.34 |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |
| IoNote3 | Config 1 | NR_CCA_FR1_I | dBm/ 38.16MHz | -50 |  | -73.4 | -73.4 |
|  |  | NR_CCA_FR1_J |  |  |  | -72.9 | -72.9 |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  |  |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. Note 7: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 8: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 9: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |

A.11.6.2.1.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1.29.1.1.

A.11.6.2.2	Inter-frequency measurement accuracy

A.11.6.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.30.1.1 and 10.1.30.1.2.

A.11.6.2.2.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in table A.11.6.2.2.2-1. Both absolute accuracy and relative accuracy requirements of SS-RSRQ inter-frequency measurement are tested by using test parameters in table A.11.6.2.2.2-2. In all test cases, Cell 1 is the PCell with CCA and Cell 2 is target cell with CCA. Three sub-tests (Test 1, Test 2, and Test 3) are provided different Noc on Cells 1 and 2.

Table A.11.6.2.2.2-1: SS-RSRQ Inter frequency SS-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.6.2.2.2-2: SS-RSRQ Inter frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |
| SSB ARFCN |  |  |  | freq1 | freq2 | freq1 | freq2 |  | freq1 | freq2 |
| DL CCA model |  | Config 1 |  | As specified in clause A.3.26.2.1 |  |  |  |  |  |  |
| UL CCA model |  | Config 1 |  | As specified in clause A.3.26.2.2 |  |  |  |  |  |  |
| UL CCA probability |  | PCCA_UL |  | 1.0 | - | 1.0 | - | 1.0 |  | - |
| DL CCA probability for semi-static channel access Note 7, 8 |  | PCCA_DL |  | 0.9375 | - | 0.9375 | - | 0.9375 |  | - |
| DL CCA probability for dynamic channel access Note 8, 9 |  | PCCA_DL_1 |  | 0.75 | - | 0.75 | - | 0.75 |  | - |
|  |  | PCCA_DL_2 |  | 0.75 | - | 0.75 | - | 0.75 |  | - |
| Duplex mode |  | Config 1 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 40: NPRB,c = 106 |  |  |  |  |  |  |
| Gap pattern ID |  | Config 1 |  | 0 |  |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 CCA |  | SR.1.1 CCA |  |  | SR.1.1 CCA |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA |  | CR.1.1 CCA |  |  | CR.1.1 CCA |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  |  | CCR.1.1 CCA |  |
| TRS Configuration |  | Config 1 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1 | s | - | 3 | - | 3 | - |  | 3 |
| DBT Window configuration |  | Config 1 |  | DBT.1 |  |  |  |  |  |  |
| SSB configuration |  | Config 1 |  | SSB.1 CCA for semi-static channel access SSB.2 CCA for dynamic channel access |  |  |  |  |  |  |
| SMTC configuration |  | Config 1 |  | SMTC.1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |  | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | NR_CCA_FR1_I | dBm/15 kHz | -86.27 |  | -113 |  |  | -112 |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  | -111.5 |  |
| Note2 | Config 1 | NR_CCA_FR1_I | dBm/SCS | -83.27 |  | -110 |  |  | -109 |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  | -108.5 |  |
|  |  |  | dB | -1.75 |  | -1.75 |  |  | 3 | -1.75 |
|  |  |  | dB | -1.75 |  | -1.75 |  |  | 3 | -1.75 |
| SS-RSRPNote3 | Config 1 | NR_CCA_FR1_I |  | -85.02 | -85.02 | -111.75 | -111.75 |  | -106 | -110.75 |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  | -105.5 | -110-.25 |
| SS-RSRQNote3 |  | NR_CCA_FR1_I | dB | -14.77 | -14.77 | -40.59 | -40.59 |  | 12.56 | 14.76 |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1 | NR_CCA_FR1_I | dBm/SCS | -50 |  | -76.73 |  |  | -73.19 | -75.73 |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  | -72.69 | -75.23 |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN | AWGN |  | AWGN | AWGN |
| Antenna configuration |  |  |  | 1x2 | 1x2 | 1x2 | 1x2 |  | 1x2 | 1x2 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: NR operating band groups are as defined in clause 3.5.2. NOTE 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configuration. NOTE 7: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 8: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 9: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |  |  |

A.11.6.2.2.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1.30.1.1 and 10.1.30.1.2.

A.11.6.2.3	Intra-frequency measurement accuracy on SCC

A.11.6.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.29.1.1.

A.11.6.2.3.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in table A.11.6.2.3.2-1. The absolute accuracy of SS-RSRQ intra-frequency measurement is tested by using the parameters in table A.11.6.2.3.2-2. In all test cases, Cell 1 is the PCell with CCA, Cell 2 is the SCell with CCA, and Cell 3 is the target cell with CCA. Three sub-tests (Test 1, Test 2, and Test 3) are provided different Noc on Cells 1, 2, and 3.

Table A.11.6.2.3.2-1: SS-RSRQ Intra frequency SS-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.6.2.3.2-2: SS-RSRQ Intra frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 / Cell 2 | Cell 3 | Cell 1 / Cell 2 | Cell 3 |
| SSB ARFCN |  |  |  | freq1 for Cell 1 freq2 for Cell 2 | freq2 | freq1 for Cell 1 freq2 for Cell 2 | freq2 |
| DL CCA model |  | Config 1 |  | As specified in clause A.3.26.2.1 |  |  |  |
| UL CCA model |  | Config 1 |  | As specified in clause A.3.26.2.2 |  |  |  |
| DL CCA probability for semi-static channel access Note 7, 8 |  | PCCA_UL |  | 1.0 | - | 1.0 | - |
|  |  | PCCA_DL |  | 0.9375 | - | 0.9375 | - |
| DL CCA probability for dynamic channel access Note 8, 9 |  | PCCA_DL_1 |  | 0.75 | - | 0.75 | - |
|  |  | PCCA_DL_2 |  | 0.75 | - | 0.75 | - |
| Duplex mode |  | Config 1 |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |  |  |
| BWchannel |  | Config 1 | MHz | 40: NRB,c = 106 |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 CCA |  | SR.1.1 CCA |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA |  | CR.1.1 CCA |  |
| Control Channel RMC |  | Config 1 |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  |
| TRS configuration |  | Config 1 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OP. 1 |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |
| Time offset with Cell 1 |  | Config 1 | s | - | 3 | - | 3 |
| DBT Window Configuration |  | Config 1 |  | DBT.1 (As defined in A.3.28.1) |  |  |  |
| SSB configuration |  | Config 1 |  | SSB.1 CCA for semi-static channel access SSB.2 CCA for dynamic channel access |  |  |  |
| SMTC configuration |  | Config 1 |  | SMTC.1 |  |  |  |
| CSI-RS for tracking |  | Config 1 |  | TRS.1.2 TDD |  |  |  |
| PDSCH/PDCCH |  | Config 1 | kHz | 30kHz |  |  |  |
| subcarrier spacing |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 | Config1 | NR_CCA_FR1_I | dBm/15kHz | -91 |  | -110 |  |
|  |  | NR_CCA_FR1_J |  |  |  | -109.5 |  |
| Note2 | Config 1 | NR_CCA_FR1_I | dBm/SCS | -88 |  | -107 |  |
|  |  | NR_CCA_FR1_J |  |  |  | -106.5 |  |
|  |  |  | dB | -1.76 |  | -5.46 | -5.46 |
|  |  |  | dB | 3 | 3 | -4 | -4 |
| SS-RSRPNote3 | Config 1 | NR_CCA_FR1_I | dBm/SCS | -85 | -85 | -111 | -111 |
|  |  | NR_CCA_FR1_J |  |  |  | -110.5 | -110.5 |
| SS-RSRQ Note3 |  | NR_CCA_FR1_I | dB | -14.77 | -14.77 | -17.34 | -17.34 |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |
| IoNote3 | Config 1 | NR_CCA_FR1_I | dBm/ 38.16MHz | -50 |  | -73.4 | -73.4 |
|  |  | NR_CCA_FR1_J |  |  |  | -72.9 | -72.9 |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  |  |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. Note 7: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 8: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 9: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |

A.11.6.2.3.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1.29.1.1.

A.11.6.2.4	Inter-frequency measurement accuracy

A.11.6.2.4.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.30.1.1 and 10.1.30.1.2.

A.11.6.2.4.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in table A.11.6.2.4.2-1. Both absolute accuracy and relative accuracy requirements of SS-RSRQ inter-frequency measurement are tested by using test parameters in table A.11.6.2.4.2-2 and A.11.6.2.4.2-3. In all test cases, Cell 1 is the PCell and Cell 2 is target cell with CCA. Three sub-tests (Test 1, Test 2, and Test 3) are provided different Noc on Cells 1 and 2.

Table A.11.6.2.4.2-1: SS-RSRQ Inter frequency SS-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | Without CCA: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.6.2.4.2-2: SS-RSRQ Inter frequency test parameters

| Parameter |  |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 2 | Cell 2 |
| SSB ARFCN |  |  |  | freq2 | freq2 | freq2 |
| DL CCA model |  | Config 1, 2, 3 |  | As specified in clause A.3.26.2.1 |  |  |
| UL CCA model |  | Config 1, 2, 3 |  | As specified in clause A.3.26.2.2 |  |  |
| UL CCA probability |  | PCCA_UL |  | 1.0 |  |  |
| DL CCA probability for semi-static channel access Note 7, 8 |  | PCCA_DL |  | 0.9375 |  |  |
| DL CCA probability for dynamic channel access Note 8, 9 |  | PCCA_DL_1 |  | 0.75 |  |  |
|  |  | PCCA_DL_2 |  | 0.75 |  |  |
| Duplex mode |  | Config 1, 2, 3 |  | TDD |  |  |
| TDD configuration |  | Config 1, 2, 3 |  | TDDConf.1.1 CCA |  |  |
| BWchannel |  | Config 1, 2, 3 | MHz | 40: NPRB,c = 106 |  |  |
| Gap pattern ID |  | Config 1, 2, 3 |  | 0 |  |  |
| BWP BW |  | Config 1, 2, 3 |  | 40: NRB,c = 106 |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |
| Time offset with Cell 1 |  | Config 1, 2, 3 | s | 3 |  |  |
| DBT Window configuration |  | Config 1, 2, 3 |  | DBT.1 |  |  |
| SSB configuration |  | Config 1, 2, 3 |  | SSB.1 CCA for semi-static channel access SSB.2 CCA for dynamic channel access |  |  |
| SMTC configuration |  | Config 1, 2, 3 |  | SMTC.1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2, 3 | kHz | 30 kHz |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 |  | NR_CCA_FR1_I | dBm/15 kHz | -86.27 | -113 | -112 |
|  |  | NR_CCA_FR1_J |  |  |  | -111.5 |
| Note2 | Config 1, 2, 3 | NR_CCA_FR1_I | dBm/SCS | -83.27 | -110 | -109 |
|  |  | NR_CCA_FR1_J |  |  |  | -108.5 |
|  |  |  | dB | -1.75 | -1.75 | -1.75 |
|  |  |  | dB | -1.75 |  | -1.75 |
| SS-RSRPNote3 | Config 1, 2, 3 | NR_CCA_FR1_I |  | -85.02 | -111.75 | -110.75 |
|  |  | NR_CCA_FR1_J |  |  |  | -110-.25 |
| SS-RSRQNote3 |  | NR_CCA_FR1_I | dB | -14.77 | -40.59 | 14.76 |
|  |  | NR_CCA_FR1_J |  |  |  |  |
| IoNote3 | Config 1, 2, 3 | NR_CCA_FR1_I | dBm/SCS | -50 | -76.73 | -75.73 |
|  |  | NR_CCA_FR1_J |  |  |  | -75.23 |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN |
| Antenna configuration |  |  |  | 1x2 | 1x2 | 1x2 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: NR operating band groups are as defined in clause 3.5.2. NOTE 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configuration. NOTE 7: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 8: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 9: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |

Table A.11.6.2.4.2-3: SS-RSRQ Intra frequency test parameters for NR PCell

| Parameter |  |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 1 | Cell 1 |
| SSB ARFCN |  |  |  |  | freq1 |  |
| Duplex mode |  | Config 1 |  |  | FDD |  |
|  |  | Config 2,3 |  |  | TDD |  |
| TDD configuration |  | Config 1 |  |  | Not Applicable |  |
|  |  | Config 2 |  |  | TDDConf.1.1 |  |
|  |  | Config 3 |  |  | TDDConf.2.1 |  |
| BWchannel |  | Config 1 | MHz |  | 10: NPRB,c = 52 |  |
|  |  | Config 2 |  |  | 10: NPRB,c = 52 |  |
|  |  | Config 3 |  |  | 40: NPRB,c = 106 |  |
| Gap Pattern ID |  |  |  |  | 0 |  |
| BWP configuration |  | Initial DL BWP |  |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |
|  |  | Initial UL BWP |  |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |
| DRX Cycle |  |  | ms |  | Not Applicable |  |
| PDSCH Reference measurement channel |  | Config 1 |  |  | SR.1.1 FDD |  |
|  |  | Config 2 |  |  | SR.1.1 TDD |  |
|  |  | Config 3 |  |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  |  | CR.1.1 FDD |  |
|  |  | Config 2 |  |  | CR.1.1 TDD |  |
|  |  | Config 3 |  |  | CR.2.1 TDD |  |
| Control Channel RMC |  | Config 1 |  |  | CCR.1.1 FDD |  |
|  |  | Config 2 |  |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  |  | CCR.2.1 TDD |  |
| TRS Configuration |  | Config 1 |  |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  |  | OP. 1 |  |
| SS-RSSI-Measurement |  |  |  |  | Not Applicable |  |
| SMTC configuration |  | Config 1 |  |  | SMTC.2 |  |
|  |  | Config 2,3 |  |  | SMTC.1 |  |
| SSB configuration |  | Config 1,2 |  |  | SSB.1 FR1 |  |
|  |  | Config 3 |  |  | SSB.2 FR1 |  |
| CSI-RS for tracking |  | Config 1 |  |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz |  | 15 kHz |  |
|  |  | Config 3 |  |  | 30 kHz |  |
| EPRE ratio of PSS to SSS |  |  | dB |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Noc Note2 | Config 1,2 | NR_FDD_FR1_A | dBm/15 kHz | -85 | -101 | -114 |
|  |  | NR_FDD_FR1_B |  |  |  | -113.5 |
|  |  | NR_TDD_FR1_C |  |  |  | -113 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  |  | NR_FDD_FR1_E |  |  |  | -112 |
|  |  | NR_FDD_FR1_H |  |  |  | -110.5 |
|  | Config 3 | NR_FDD_FR1_A |  | -91 | - | -114 |
|  |  | NR_FDD_FR1_B |  |  |  | -113.5 |
|  |  | NR_TDD_FR1_C |  |  |  | -113 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  |  | NR_FDD_FR1_E |  |  |  | -112 |
|  |  | NR_FDD_FR1_H |  |  |  | -110.5 |
| Noc Note2 | Config 1,2 | NR_FDD_FR1_A | dBm/SCS | -85 | -101 | -114 |
|  |  | NR_FDD_FR1_B |  |  |  | -113.5 |
|  |  | NR_TDD_FR1_C |  |  |  | -113 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  |  | NR_FDD_FR1_E |  |  |  | -112 |
|  |  | NR_FDD_FR1_H |  |  |  | -110.5 |
|  | Config 3 | NR_FDD_FR1_A |  | -88 | - | -111 |
|  |  | NR_FDD_FR1_B |  |  |  | -110.5 |
|  |  | NR_TDD_FR1_C |  |  |  | -110 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -109.5 |
|  |  | NR_FDD_FR1_E |  |  |  | -109 |
|  |  | NR_FDD_FR1_H |  |  |  | -107.5 |
|  |  |  | dB | -1.76 | -4.7 | -5.46 |
|  |  |  | dB | 3 | -2.9 | -4 |
| SS-RSRPNote3 | Config 1,2 | NR_FDD_FR1_A | dBm/SCS | -82 | -103.9 | -118 |
|  |  | NR_FDD_FR1_B |  |  |  | -117.5 |
|  |  | NR_TDD_FR1_C |  |  |  | -117 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -116.5 |
|  |  | NR_FDD_FR1_E, |  |  |  | -116 |
|  |  | NR_FDD_FR1_H |  |  |  | -114.5 |
|  | Config 3 | NR_FDD_FR1_A |  | -85 | - | -115 |
|  |  | NR_FDD_FR1_B |  |  |  | -114.5 |
|  |  | NR_TDD_FR1_C |  |  |  | -114 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -113.5 |
|  |  | NR_FDD_FR1_E |  |  |  | -113 |
|  |  | NR_FDD_FR1_H |  |  |  | -111.5 |
| SS-RSRQ Note3 |  | NR_FDD_FR1_A | dB | -14.77 | -16.76 | -17.34 |
|  |  | NR_FDD_FR1_B |  |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |
|  |  | NR_FDD_FR1_E |  |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A | dBm/ 9.36 MHz | -50 | -70 | -83.5 |
|  |  | NR_FDD_FR1_B |  |  |  | -83 |
|  |  | NR_TDD_FR1_C |  |  |  | -82.5 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -82 |
|  |  | NR_FDD_FR1_E |  |  |  | -81.5 |
|  |  | NR_FDD_FR1_H |  |  |  | -80 |
|  | Config 3 | NR_FDD_FR1_A | dBm/ 38.16 MHz | -50 | - | -77.4 |
|  |  | NR_FDD_FR1_B |  |  |  | -76.9 |
|  |  | NR_TDD_FR1_C |  |  |  | -76.4 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -75.9 |
|  |  | NR_FDD_FR1_E |  |  |  | -75.4 |
|  |  | NR_FDD_FR1_H |  |  |  | -73.9 |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN |
| Antenna configuration |  |  |  | 1x2 | 1x2 | 1x2 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |

A.11.6.2.4.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1.30.1.1 and 10.1.30.1.2.

A.11.6.3.4	Inter-frequency measurement accuracy

A.11.6.3.4.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.32.1.1 and 10.1.32.1.2.

A.11.6.3.4.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in Table A.11.6.3.4.2-1. Both absolute accuracy and relative accuracy requirements of SS-SINR inter-frequency measurement are tested by using test parameters in Table A.11.6.3.4.2-2 and Table A.11.6.3.4.2-3. In all test cases, Cell 1 is the PCell and Cell 2 is target cell with CCA. Three sub-tests (Test 1, Test 2, and Test 3) are provided different Noc on Cells 1 and 2.

Table A.11.6.3.4.2-1: SS-SINR Inter frequency SS-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | Without CCA: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.6.3.4.2-2: SS-SINR Inter frequency test parameters

| Parameter |  |  | Unit |  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 2 | Cell 2 | Cell 2 |
| SSB ARFCN |  |  |  |  | freq2 | freq2 | freq2 |
| DL CCA model |  | Config 1 |  |  | As specified in clause A.3.26.2.1 |  |  |
| UL CCA model |  | Config 1 |  |  | As specified in clause A.3.26.2.2 |  |  |
| UL CCA probability |  | PCCA_UL |  |  | 1.0 |  |  |
| DL CCA probability for semi-static channel access Note 7, 8 |  | PCCA_DL |  |  | 0.9375 |  |  |
| DL CCA probability for dynamic channel access Note 8, 9 |  | PCCA_DL_1 |  |  | 0.75 |  |  |
|  |  | PCCA_DL_2 |  |  | 0.75 |  |  |
| Duplex mode |  | Config 1 |  |  | TDD |  |  |
| TDD configuration |  | Config 1 |  |  | TDDConf.1.1 CCA |  |  |
| Downlink initial BWP configuration |  |  |  |  | DLBWP.0.1 |  |  |
| Downlink dedicated BWP configuration |  |  |  |  | DLBWP.1.1 |  |  |
| Uplink initial BWP configuration |  |  |  |  | ULBWP.0.1 |  |  |
| Uplink dedicated BWP configuration |  |  |  |  | ULBWP.1.1 |  |  |
| DRX Cycle configuration |  |  | ms |  | Not Applicable |  |  |
| Gap pattern ID |  |  |  |  | - |  |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |
| SS-RSSI-Measurement |  |  |  |  | Not Applicable |  |  |
| Time offset with Cell 1 |  | Config 1 | s |  | 3 |  |  |
| DBT Window configuration |  | Config 1 |  |  | DBT.1 |  |  |
| SSB configuration |  | Config 1 |  |  | SSB.1 CCA for semi-static channel access SSB.2 CCA for dynamic channel access |  |  |
| SMTC configuration |  | Config 1 |  |  | SMTC.1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz |  | 30 |  |  |
| EPRE ratio of PSS to SSS |  |  | dB |  | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | NR_CCA_FR1_I | dBm/15kHz |  | -88 | -108.5 | -115.5 |
|  |  | NR_CCA_FR1_J |  |  |  |  | -115 |
| Note2 | Config 1 | NR_CCA_FR1_I | dBm/SCS |  | -85 | -105.5 | -112.5 |
|  |  | NR_CCA_FR1_J |  |  |  |  | -112 |
|  |  |  | dB |  | -1.75 | 20 | -4.0 |
|  |  |  | dB |  | -1.75 | 20 | -4.0 |
| SS-RSRP Note3 | Config 1 | NR_CCA_FR1_I | dBm/SCS |  | -86.75 | -85.5 | -116.5 |
|  |  | NR_CCA_FR1_J |  |  |  |  | -116 |
| SS-SINRNote3 |  | NR_CCA_FR1_I | dB |  | -1.75 | 20 | -4.0 |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |
| IoNote3 | Config 1 | NR_CCA_FR1_I | dBm/ 38.16MHz |  | -51.73 | -54.41 | -80 |
|  |  | NR_CCA_FR1_J |  |  |  |  | -79.5 |
| Propagation condition |  |  | - |  | AWGN |  |  |
| Antenna configuration |  |  | - |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configuration. Note 7: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 8: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 9: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |

Table A.11.6.3.4.2-3: SS-SINR Inter frequency test parameters for NR PCell

| Parameter |  |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 1 | Cell 1 |
| SSB ARFCN |  |  |  | freq1 | freq1 | freq1 |
| Duplex mode |  | Config 1 |  |  | FDD |  |
|  |  | Config 2,3 |  |  | TDD |  |
| TDD configuration |  | Config 1 |  |  | Not Applicable |  |
|  |  | Config 2 |  |  | TDDConf.1.1 |  |
|  |  | Config 3 |  |  | TDDConf.2.1 |  |
| Downlink initial BWP configuration |  |  |  |  | DLBWP.0.1 |  |
| Downlink dedicated BWP configuration |  |  |  |  | DLBWP.1.1 |  |
| Uplink initial BWP configuration |  |  |  |  | ULBWP.0.1 |  |
| Uplink dedicated BWP configuration |  |  |  |  | ULBWP.1.1 |  |
| DRX Cycle configuration |  |  | ms |  | Not Applicable |  |
| Gap pattern ID |  |  |  |  | 0 |  |
| TRS configuration |  | Config 1 |  |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |
| PDSCH Reference measurement channel |  | Config 1 |  |  | SR.1.1 FDD |  |
|  |  | Config 2 |  |  | SR.1.1 TDD |  |
|  |  | Config 3 |  |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  |  | CR.1.1 FDD |  |
|  |  | Config 2 |  |  | CR.1.1 TDD |  |
|  |  | Config 3 |  |  | CR2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  |  | CCR.1.1 FDD |  |
|  |  | Config 2 |  |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  |  | CCR2.1 TDD |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |
| SS-RSSI-Measurement |  |  |  |  | Not Applicable |  |
| SMTC configuration |  | Config 1 |  |  | SMTC pattern 2 |  |
|  |  | Config 2,3 |  |  | SMTC pattern 1 |  |
| SSB configuration |  | Config 1,2 |  |  | SSB.1 FR1 |  |
|  |  | Config 3 |  |  | SSB.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz |  | 15 |  |
|  |  | Config 3 |  |  | 30 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A | dBm/15kHz | -88 | -108.5 | -119.5 |
|  |  | NR_FDD_FR1_B |  |  |  | -119 |
|  |  | NR_TDD_FR1_C |  |  |  | -118.5 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  | -118 |
|  |  | NR_FDD_FR1_E |  |  |  | -117.5 |
|  |  | NR_FDD_FR1_H |  |  |  | -116 |
| Note2 | Config 1,2 |  | dBm/SCS | -88 | -108.5 | Same as Noc for 15kHz |
|  | Config 3 | NR_FDD_FR1_A |  | -85 | -105.5 | -116.5 |
|  |  | NR_FDD_FR1_B |  |  |  | -116 |
|  |  | NR_TDD_FR1_C |  |  |  | -115.5 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  | -115 |
|  |  | NR_FDD_FR1_E |  |  |  | -114.5 |
|  |  | NR_FDD_FR1_H |  |  |  | -113 |
|  |  |  | dB | -1.75 | 20 | -4.0 |
|  |  |  | dB | -1.75 | 20 | -4.0 |
| SS-RSRP Note3 | Config 1,2 | NR_FDD_FR1_A | dBm/SCS | -89.75 | -88.5 | -123.5 |
|  |  | NR_FDD_FR1_B |  |  |  | -123 |
|  |  | NR_TDD_FR1_C |  |  |  | -122.5 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  | -122 |
|  |  | NR_FDD_FR1_E |  |  |  | -121.5 |
|  |  | NR_FDD_FR1_H |  |  |  | -120 |
|  | Config 3 | NR_FDD_FR1_A |  | -86.75 | -85.5 | -120.5 |
|  |  | NR_FDD_FR1_B |  |  |  | -120 |
|  |  | NR_TDD_FR1_C |  |  |  | -119.5 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  | -119 |
|  |  | NR_FDD_FR1_E |  |  |  | -118.5 |
|  |  | NR_FDD_FR1_H |  |  |  | -117 |
| SS-SINRNote3 |  | NR_FDD_FR1_A | dB | -1.75 | 20 | -4.0 |
|  |  | NR_FDD_FR1_B |  |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |
|  |  | NR_FDD_FR1_E |  |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A | dBm/ 9.36MHz | -57.83 | -60.5 | -90.09 |
|  |  | NR_FDD_FR1_B |  |  |  | -89.59 |
|  |  | NR_TDD_FR1_C |  |  |  | -89.09 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  | -88.59 |
|  |  | NR_FDD_FR1_E |  |  |  | -88.09 |
|  |  | NR_FDD_FR1_H |  |  |  | -86.59 |
|  | Config 3 | NR_FDD_FR1_A | dBm/ 38.16MHz | -51.73 | -54.41 | -84 |
|  |  | NR_FDD_FR1_B |  |  |  | -83.5 |
|  |  | NR_TDD_FR1_C |  |  |  | -83 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  | -82.5 |
|  |  | NR_FDD_FR1_E |  |  |  | -82 |
|  |  | NR_FDD_FR1_H |  |  |  | -80.5 |
| Propagation condition |  |  | - |  | AWGN |  |
| Antenna configuration |  |  | - |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |

A.11.6.3.4.3	Test Requirements

The SS-SINR measurement accuracy shall fulfil the requirements in clause 10.1.32.1.1 and 10.1.32.1.2.

A.11.6.3	SS-SINR

A.11.6.3.1	Intra-frequency measurement accuracy

A.11.6.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.31.1.1.

A.11.6.3.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in table A.11.6.3.1.2-1. The absolute accuracy of SS-SINR intra-frequency measurement is tested by using the parameters in table A.11.6.3.1.2-2. In all test cases, Cell 1 is the PCell with CCA and Cell 2 is the target cell with CCA. Two sub-tests (Test 1 and Test 2) are provided different Noc on Cells 1 and 2.

Table A.11.6.3.1.2-1: SS-SINR Intra frequency SS-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.6.3.1.2-2: SS-SINR Intra frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  |
| DL CCA model |  | Config 1 |  | As specified in clause A.3.26.2.1 |  |  |  |
| UL CCA model |  | Config 1 |  | As specified in clause A.3.26.2.2 |  |  |  |
| UL CCA probability |  | PCCA_UL |  | 1.0 | - | 1.0 | - |
| DL CCA probability for semi-static channel access Note 7, 8 |  | PCCA_DL |  | 0.9375 | - | 0.9375 | - |
| DL CCA probability for dynamic channel access Note 8, 9 |  | PCCA_DL_1 |  | 0.75 | - | 0.75 | - |
|  |  | PCCA_DL_2 |  | 0.75 | - | 0.75 | - |
| Duplex mode |  | Config 1 |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 CCA |  | SR.1.1 CCA |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA |  | CR.1.1 CCA |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |
| DBT Window configuration |  | Config 1 |  | DBT.1 |  |  |  |
| Time offset with Cell 1 |  | Config 1 | s | - | 3 | - | 3 |
| SSB configuration |  | Config 1 |  | SSB.1 CCA for semi-static channel access SSB.2 CCA for dynamic channel access |  |  |  |
| SMTC configuration |  | Config 1 |  | SMTC.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | NR_CCA_FR1_I | dBm/15 kHz | -93 |  | -112 |  |
|  |  | NR_CCA_FR1_J |  |  |  | -111.5 |  |
| Note2 | Config 1 | NR_CCA_FR1_I | dBm/SCS | -90 |  | -109 |  |
|  |  | NR_CCA_FR1_J |  |  |  | -108.5 |  |
|  |  |  | dB | 0 | -3.19 | -5.46 | -5.46 |
|  |  |  | dB | 4.54 | 2.66 | -4 | -4 |
| SS-RSRPNote3 | Config 1 | NR_CCA_FR1_I | dBm/SCS | -85.46 | -87.34 | -113 | -113 |
|  |  | NR_CCA_FR1_J |  |  |  | -112.5 | -112.5 |
| SS-SINR Note3 |  | NR_CCA_FR1_I | dB | 0 | -3.19 | -5.46 | -5.46 |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |
| IoNote3 | Config 1 | NR_CCA_FR1_I | dBm/ 38.16 MHz | -51.41 |  | -75.41 |  |
|  |  | NR_CCA_FR1_J |  |  |  | -74.91 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: NR operating band groups are as defined in clause 3.5.2. NOTE 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configuration. NOTE 7: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 8: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 9: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |

A.11.6.3.1.3	Test Requirements

The SS-SINR measurement accuracy shall fulfil the requirements in clause 10.1.31.1.1.

A.11.6.3.2	Inter-frequency measurement accuracy

A.11.6.3.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.32.1.1 and 10.1.32.1.2.

A.11.6.3.2.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in table A.11.6.3.2.2-1. Both absolute accuracy and relative accuracy requirements of SS-SINR inter-frequency measurement are tested by using test parameters in table A.11.6.3.2.2-2. In all test cases, Cell 1 is the PCell with CCA and Cell 2 is target cell with CCA. Three sub-tests (Test 1, Test 2, and Test 3) are provided different Noc on Cells 1 and 2.

Table A.11.6.3.2.2-1: SS-SINR Inter frequency SS-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.6.3.2.2-2: SS-SINR Inter frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |
| SSB ARFCN |  |  |  | freq1 | freq2 | freq1 | freq2 |  | freq1 | freq2 |
| DL CCA model |  | Config 1 |  | As specified in clause A.3.26.2.1 |  |  |  |  |  |  |
| UL CCA model |  | Config 1 |  | As specified in clause A.3.26.2.2 |  |  |  |  |  |  |
| UL CCA probability |  | PCCA_UL |  | 1.0 | - | 1.0 | - | 1.0 |  | - |
| DL CCA probability for semi-static channel access Note 7, 8 |  | PCCA_DL |  | 0.9375 | - | 0.9375 | - | 0.9375 |  | - |
| DL CCA probability for dynamic channel access Note 8, 9 |  | PCCA_DL_1 |  | 0.75 | - | 0.75 | - | 0.75 |  | - |
|  |  | PCCA_DL_2 |  | 0.75 | - | 0.75 | - | 0.75 |  | - |
| Duplex mode |  | Config 1 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |  |  |  |
| Gap pattern ID |  |  |  | 0 | - | 0 | - |  | 0 | - |
| TRS configuration |  | Config 1 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 CCA |  | SR.1.1 CCA |  |  | SR.1.1 CCA |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA |  | CR.1.1 CCA |  |  | CR.1.1 CCA |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  |  | CCR.1.1 CCA |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1 | s | - | 3 | - | 3 |  | - | 3 |
| DBT Window configuration |  | Config 1 |  | DBT.1 |  |  |  |  |  |  |
| SSB configuration |  | Config 1 |  | SSB.1 CCA for semi-static channel access SSB.2 CCA for dynamic channel access |  |  |  |  |  |  |
| SMTC configuration |  | Config 1 |  | SMTC.1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 30 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |  | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | NR_CCA_FR1_I | dBm/15 kHz | -88 |  | -108.5 |  |  | -115.5 |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  | -115 |  |
| Note2 | Config 1 | NR_CCA_FR1_I | dBm/SCS | -85 |  | -105.5 |  |  | -112.5 |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  | -112 |  |
|  |  |  | dB | -1.75 | -1.75 | 20 | 20 |  | -4.0 | -4.0 |
|  |  |  | dB | -1.75 |  | 20 |  |  | -4.0 |  |
| SS-RSRP Note3 | Config 1 | NR_CCA_FR1_I | dBm/SCS | -86.75 |  | -85.5 |  |  | -116.5 |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  | -116 |  |
| SS-SINRNote3 |  | NR_CCA_FR1_I | dB | -1.75 |  | 20 |  |  | -4.0 |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1 | NR_CCA_FR1_I | dBm/ 38.16 MHz | -51.73 |  | -54.41 |  |  | -80 |  |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  | -79.5 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: NR operating band groups are as defined in clause 3.5.2. NOTE 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configuration. NOTE 7: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 8: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 9: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |  |  |

A.11.6.3.2.3	Test Requirements

The SS-SINR measurement accuracy shall fulfil the requirements in clause 10.1.32.1.1 and 10.1.32.1.2.

A.11.6.3.3	Intra-frequency measurement accuracy on SCC

A.11.6.3.3.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.31.1.1.

A.11.6.3.3.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in table A.11.6.3.3.2-1. The absolute accuracy of SS-SINR intra-frequency measurement is tested by using the parameters in table A.11.6.3.3.2-2. In all test cases, Cell 1 is the PCell with CCA, Cell 2 is the SCell with CCA, and Cell 3 is the target cell with CCA. Two sub-tests (Test 1 and Test 2) are provided different Noc on Cells 1, 2, and 3.

Table A.11.6.3.3.2-1: SS-SINR Intra frequency SS-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.6.3.3.2-2: SS-SINR Intra frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 / Cell 2 | Cell 3 | Cell 1 / Cell 2 |  | Cell 3 |
| SSB ARFCN |  |  |  | freq1 for Cell 1 freq2 for Cell 2 | freq2 | freq1 for Cell 1 freq2 for Cell 2 |  | freq2 |
| DL CCA model |  | Config 1 |  | As specified in clause A.3.26.2.1 |  |  |  |  |
| UL CCA model |  | Config 1 |  | As specified in clause A.3.26.2.2 |  |  |  |  |
| UL CCA probability |  | PCCA_UL |  | 1.0 | - | 1.0 | - |  |
| DL CCA probability for semi-static channel access Note 7, 8 |  | PCCA_DL |  | 0.9375 | - | 0.9375 | - |  |
| DL CCA probability for dynamic channel access Note 8, 9 |  | PCCA_DL_1 |  | 0.75 | - | 0.75 | - |  |
|  |  | PCCA_DL_2 |  | 0.75 | - | 0.75 | - |  |
| Duplex mode |  | Config 1 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 CCA |  | SR.1.1 CCA |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA |  | CR.1.1 CCA |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |
| DBT Window configuration |  | Config 1 |  | DBT.1 |  |  |  |  |
| Time offset with Cell 1 |  | Config 1 | s | 3 (for Cell 2) | 3 | 3 (for Cell 2) |  | 3 |
| SSB configuration |  | Config 1 |  | SSB.1 CCA for semi-static channel access SSB.2 CCA for dynamic channel access |  |  |  |  |
| SMTC configuration |  | Config 1 |  | SMTC.1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 30 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | NR_CCA_FR1_I | dBm/15 kHz | -93 |  | -112 |  |  |
|  |  | NR_CCA_FR1_J |  |  |  | -111.5 |  |  |
| Note2 | Config 1 | NR_CCA_FR1_I | dBm/SCS | -90 |  | -109 |  |  |
|  |  | NR_CCA_FR1_J |  |  |  | -108.5 |  |  |
|  |  |  | dB | 0 | -3.19 | -5.46 |  | -5.46 |
|  |  |  | dB | 4.54 | 2.66 | -4 |  | -4 |
| SS-RSRPNote3 | Config 1 | NR_CCA_FR1_I | dBm/SCS | -85.46 | -87.34 | -113 |  | -113 |
|  |  | NR_CCA_FR1_J |  |  |  | -112.5 |  | -112.5 |
| SS-SINR Note3 |  | NR_CCA_FR1_I | dB | 0 | -3.19 | -5.46 |  | -5.46 |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  |
| IoNote3 | Config 1 | NR_CCA_FR1_I | dBm/ 38.16 MHz | -51.41 |  | -75.41 |  |  |
|  |  | NR_CCA_FR1_J |  |  |  | -74.91 |  |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: NR operating band groups are as defined in clause 3.5.2. NOTE 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configuration. NOTE 7: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 8: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 9: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |

A.11.6.3.3.3	Test Requirements

The SS-SINR measurement accuracy shall fulfil the requirements in clause 10.1.31.1.1.

A.11.6.3.4	Inter-frequency measurement accuracy

A.11.6.3.4.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.32.1.1 and 10.1.32.1.2.

A.11.6.3.4.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in table A.11.6.3.4.2-1. Both absolute accuracy and relative accuracy requirements of SS-SINR inter-frequency measurement are tested by using test parameters in table A.11.6.3.4.2-2 and table A.11.6.3.4.2-3. In all test cases, Cell 1 is the PCell and Cell 2 is target cell with CCA. Three sub-tests (Test 1, Test 2, and Test 3) are provided different Noc on Cells 1 and 2.

Table A.11.6.3.4.2-1: SS-SINR Inter frequency SS-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | Without CCA: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.6.3.4.2-2: SS-SINR Inter frequency test parameters

| Parameter |  |  | Unit |  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 2 | Cell 2 | Cell 2 |
| SSB ARFCN |  |  |  |  | freq2 | freq2 | freq2 |
| DL CCA model |  | Config 1 |  |  | As specified in clause A.3.26.2.1 |  |  |
| UL CCA model |  | Config 1 |  |  | As specified in clause A.3.26.2.2 |  |  |
| UL CCA probability |  | PCCA_UL |  |  | 1.0 |  |  |
| DL CCA probability for semi-static channel access Note 7, 8 |  | PCCA_DL |  |  | 0.9375 |  |  |
| DL CCA probability for dynamic channel access Note 8, 9 |  | PCCA_DL_1 |  |  | 0.75 |  |  |
|  |  | PCCA_DL_2 |  |  | 0.75 |  |  |
| Duplex mode |  | Config 1 |  |  | TDD |  |  |
| TDD configuration |  | Config 1 |  |  | TDDConf.1.1 CCA |  |  |
| Downlink initial BWP configuration |  |  |  |  | DLBWP.0.1 |  |  |
| Downlink dedicated BWP configuration |  |  |  |  | DLBWP.1.1 |  |  |
| Uplink initial BWP configuration |  |  |  |  | ULBWP.0.1 |  |  |
| Uplink dedicated BWP configuration |  |  |  |  | ULBWP.1.1 |  |  |
| DRX Cycle configuration |  |  | ms |  | Not Applicable |  |  |
| Gap pattern ID |  |  |  |  | - |  |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |
| SS-RSSI-Measurement |  |  |  |  | Not Applicable |  |  |
| Time offset with Cell 1 |  | Config 1 | s |  | 3 |  |  |
| DBT Window configuration |  | Config 1 |  |  | DBT.1 |  |  |
| SSB configuration |  | Config 1 |  |  | SSB.1 CCA for semi-static channel access SSB.2 CCA for dynamic channel access |  |  |
| SMTC configuration |  | Config 1 |  |  | SMTC.1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz |  | 30 |  |  |
| EPRE ratio of PSS to SSS |  |  | dB |  | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | NR_CCA_FR1_I | dBm/15 kHz |  | -88 | -108.5 | -115.5 |
|  |  | NR_CCA_FR1_J |  |  |  |  | -115 |
| Note2 | Config 1 | NR_CCA_FR1_I | dBm/SCS |  | -85 | -105.5 | -112.5 |
|  |  | NR_CCA_FR1_J |  |  |  |  | -112 |
|  |  |  | dB |  | -1.75 | 20 | -4.0 |
|  |  |  | dB |  | -1.75 | 20 | -4.0 |
| SS-RSRP Note3 | Config 1 | NR_CCA_FR1_I | dBm/SCS |  | -86.75 | -85.5 | -116.5 |
|  |  | NR_CCA_FR1_J |  |  |  |  | -116 |
| SS-SINRNote3 |  | NR_CCA_FR1_I | dB |  | -1.75 | 20 | -4.0 |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |
| IoNote3 | Config 1 | NR_CCA_FR1_I | dBm/ 38.16 MHz |  | -51.73 | -54.41 | -80 |
|  |  | NR_CCA_FR1_J |  |  |  |  | -79.5 |
| Propagation condition |  |  | - |  | AWGN |  |  |
| Antenna configuration |  |  | - |  | 1x2 |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: NR operating band groups are as defined in clause 3.5.2. NOTE 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configuration. NOTE 7: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 8: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 9: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |

Table A.11.6.3.4.2-3: SS-SINR Inter frequency test parameters for NR PCell

| Parameter |  |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 1 | Cell 1 |
| SSB ARFCN |  |  |  | freq1 | freq1 | freq1 |
| Duplex mode |  | Config 1 |  |  | FDD |  |
|  |  | Config 2,3 |  |  | TDD |  |
| TDD configuration |  | Config 1 |  |  | Not Applicable |  |
|  |  | Config 2 |  |  | TDDConf.1.1 |  |
|  |  | Config 3 |  |  | TDDConf.2.1 |  |
| Downlink initial BWP configuration |  |  |  |  | DLBWP.0.1 |  |
| Downlink dedicated BWP configuration |  |  |  |  | DLBWP.1.1 |  |
| Uplink initial BWP configuration |  |  |  |  | ULBWP.0.1 |  |
| Uplink dedicated BWP configuration |  |  |  |  | ULBWP.1.1 |  |
| DRX Cycle configuration |  |  | ms |  | Not Applicable |  |
| Gap pattern ID |  |  |  |  | 0 |  |
| TRS configuration |  | Config 1 |  |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |
| PDSCH Reference measurement channel |  | Config 1 |  |  | SR.1.1 FDD |  |
|  |  | Config 2 |  |  | SR.1.1 TDD |  |
|  |  | Config 3 |  |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  |  | CR.1.1 FDD |  |
|  |  | Config 2 |  |  | CR.1.1 TDD |  |
|  |  | Config 3 |  |  | CR2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  |  | CCR.1.1 FDD |  |
|  |  | Config 2 |  |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  |  | CCR2.1 TDD |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |
| SS-RSSI-Measurement |  |  |  |  | Not Applicable |  |
| SMTC configuration |  | Config 1 |  |  | SMTC pattern 2 |  |
|  |  | Config 2,3 |  |  | SMTC pattern 1 |  |
| SSB configuration |  | Config 1,2 |  |  | SSB.1 FR1 |  |
|  |  | Config 3 |  |  | SSB.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz |  | 15 |  |
|  |  | Config 3 |  |  | 30 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A | dBm/15 kHz | -88 | -108.5 | -119.5 |
|  |  | NR_FDD_FR1_B |  |  |  | -119 |
|  |  | NR_TDD_FR1_C |  |  |  | -118.5 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  | -118 |
|  |  | NR_FDD_FR1_E |  |  |  | -117.5 |
|  |  | NR_FDD_FR1_H |  |  |  | -116 |
| Note2 | Config 1,2 |  | dBm/SCS | -88 | -108.5 | Same as Noc for 15 kHz |
|  | Config 3 | NR_FDD_FR1_A |  | -85 | -105.5 | -116.5 |
|  |  | NR_FDD_FR1_B |  |  |  | -116 |
|  |  | NR_TDD_FR1_C |  |  |  | -115.5 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  | -115 |
|  |  | NR_FDD_FR1_E |  |  |  | -114.5 |
|  |  | NR_FDD_FR1_H |  |  |  | -113 |
|  |  |  | dB | -1.75 | 20 | -4.0 |
|  |  |  | dB | -1.75 | 20 | -4.0 |
| SS-RSRP Note3 | Config 1,2 | NR_FDD_FR1_A | dBm/SCS | -89.75 | -88.5 | -123.5 |
|  |  | NR_FDD_FR1_B |  |  |  | -123 |
|  |  | NR_TDD_FR1_C |  |  |  | -122.5 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  | -122 |
|  |  | NR_FDD_FR1_E |  |  |  | -121.5 |
|  |  | NR_FDD_FR1_H |  |  |  | -120 |
|  | Config 3 | NR_FDD_FR1_A |  | -86.75 | -85.5 | -120.5 |
|  |  | NR_FDD_FR1_B |  |  |  | -120 |
|  |  | NR_TDD_FR1_C |  |  |  | -119.5 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  | -119 |
|  |  | NR_FDD_FR1_E |  |  |  | -118.5 |
|  |  | NR_FDD_FR1_H |  |  |  | -117 |
| SS-SINRNote3 |  | NR_FDD_FR1_A | dB | -1.75 | 20 | -4.0 |
|  |  | NR_FDD_FR1_B |  |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |
|  |  | NR_FDD_FR1_E |  |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A | dBm/ 9.36 MHz | -57.83 | -60.5 | -90.09 |
|  |  | NR_FDD_FR1_B |  |  |  | -89.59 |
|  |  | NR_TDD_FR1_C |  |  |  | -89.09 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  | -88.59 |
|  |  | NR_FDD_FR1_E |  |  |  | -88.09 |
|  |  | NR_FDD_FR1_H |  |  |  | -86.59 |
|  | Config 3 | NR_FDD_FR1_A | dBm/ 38.16 MHz | -51.73 | -54.41 | -84 |
|  |  | NR_FDD_FR1_B |  |  |  | -83.5 |
|  |  | NR_TDD_FR1_C |  |  |  | -83 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  | -82.5 |
|  |  | NR_FDD_FR1_E |  |  |  | -82 |
|  |  | NR_FDD_FR1_H |  |  |  | -80.5 |
| Propagation condition |  |  | - |  | AWGN |  |
| Antenna configuration |  |  | - |  | 1x2 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |

A.11.6.3.4.3	Test Requirements

The SS-SINR measurement accuracy shall fulfil the requirements in clause 10.1.32.1.1 and 10.1.32.1.2.

<End of Change 1>

<Start of Change 2>

A.13		NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1

<Skipped unchanged part>

A.13.4	Measurement performance

<Skipped unchanged part>

A.13.4.2	SS-RSRQ

A.13.4.2.1	Intra-frequency measurement accuracy on SCC

A.13.4.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.29.1.1.

A.13.4.2.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in table A.13.4.2.1.2-1. The absolute accuracy of SS-RSRQ intra-frequency measurement is tested by using the parameters in table A.13.4.2.1.2-2 and table A.13.4.2.1.2-3. In all test cases, Cell 1 is the PCell, Cell 2 is the SCell with CCA, and Cell 3 is the target cell with CCA. Three sub-tests (Test 1, Test 2, and Test 3) are provided different Noc on Cells 1, 2, and 3.

Table A.13.4.2.1.2-1: SS-RSRQ Intra frequency SS-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | Without CCA: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.13.4.2.1.2-2: SS-RSRQ Intra frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| SSB ARFCN |  |  |  | freq2 | freq2 | freq2 | freq2 |
| DL CCA model |  | Config 1, 2, 3 |  | As specified in clause A.3.26.2.1 |  |  |  |
| UL CCA model |  | Config 1, 2, 3 |  | As specified in clause A.3.26.2.2 |  |  |  |
| DL CCA probability for semi-static channel access Note 7, 8 |  | PCCA_UL |  | 1.0 | - | 1.0 | - |
|  |  | PCCA_DL |  | 0.9375 | - | 0.9375 | - |
| DL CCA probability for dynamic channel access Note 8, 9 |  | PCCA_DL_1 |  | 0.75 | - | 0.75 | - |
|  |  | PCCA_DL_2 |  | 0.75 | - | 0.75 | - |
| Duplex mode |  | Config 1, 2, 3 |  | TDD |  |  |  |
| TDD configuration |  | Config 1, 2, 3 |  | TDDConf.1.1 CCA |  |  |  |
| BWchannel |  | Config 1, 2, 3 | MHz | 40: NRB,c = 106 |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1, 2, 3 |  | SR.1.1 CCA |  | SR.1.1 CCA |  |
| RMSI CORESET Reference Channel |  | Config 1, 2, 3 |  | CR.1.1 CCA |  | CR.1.1 CCA |  |
| Control Channel RMC |  | Config 1, 2, 3 |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  |
| TRS configuration |  | Config 1, 2, 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OP. 1 |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |
| Time offset with Cell 1 |  | Config 1, 2, 3 | s | 3 | 3 | 3 | 3 |
| DBT Window Configuration |  | Config 1, 2, 3 |  | DBT.1 |  |  |  |
| SSB configuration |  | Config 1, 2, 3 |  | SSB.1 CCA for semi-static channel access SSB.2 CCA for dynamic channel access |  |  |  |
| SMTC configuration |  | Config 1, 2, 3 |  | SMTC.1 |  |  |  |
| CSI-RS for tracking |  | Config 1, 2, 3 |  | TRS.1.2 TDD |  |  |  |
| PDSCH/PDCCH |  | Config 1, 2, 3 | kHz | 30kHz |  |  |  |
| subcarrier spacing |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 | Config 1, 2, 3 | NR_CCA_FR1_I | dBm/15kHz | -91 |  | -110 |  |
|  |  | NR_CCA_FR1_J |  |  |  | -109.5 |  |
| Note2 | Config 1, 2, 3 | NR_CCA_FR1_I | dBm/SCS | -88 |  | -107 |  |
|  |  | NR_CCA_FR1_J |  |  |  | -106.5 |  |
|  |  |  | dB | -1.76 |  | -5.46 | -5.46 |
|  |  |  | dB | 3 | 3 | -4 | -4 |
| SS-RSRPNote3 | Config 1, 2, 3 | NR_CCA_FR1_I | dBm/SCS | -85 | -85 | -111 | -111 |
|  |  | NR_CCA_FR1_J |  |  |  | -110.5 | -110.5 |
| SS-RSRQ Note3 |  | NR_CCA_FR1_I | dB | -14.77 | -14.77 | -17.34 | -17.34 |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |
| IoNote3 | Config 1, 2, 3 | NR_CCA_FR1_I | dBm/ 38.16MHz | -50 |  | -73.4 | -73.4 |
|  |  | NR_CCA_FR1_J |  |  |  | -72.9 | -72.9 |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  |  |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. Note 7: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 8: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 9: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |

Table A.13.4.2.1.2-3: SS-RSRQ Intra frequency test parameters for NR PCell

| Parameter |  |  | Unit | Test 1 | Test 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 1 |
| SSB ARFCN |  |  |  | freq1 | freq1 |
| Duplex mode |  | Config 1 |  | FDD | FDD |
|  |  | Config 2,3 |  | TDD | TDD |
| TDD configuration |  | Config 1 |  | Not Applicable | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | Config 2 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | Config 3 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| Gap Pattern ID |  |  |  | 0 | 0 |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 | DLBWP.0.1 |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 | DLBWP.1.1 |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 | ULBWP.0.1 |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 | ULBWP.1.1 |
| DRX Cycle configuration |  |  | ms | Not Applicable | Not Applicable |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | Config 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | Config 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  |  | OP.1 | OP.1 |
| SS-RSSI-Measurement |  |  |  | Not Applicable | Not Applicable |
| SMTC configuration |  | Config 1 Config 2,3 |  | SMTC.2 | SMTC.2 |
|  |  |  |  | SMTC.1 | SMTC.1 |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 | SSB.2 FR1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 | 15 |
|  |  | Config 3 |  | 30 | 30 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 | Config 1, 2 | NR_FDD_FR1_A | dBm/15kHz | -85 | -114 |
|  |  | NR_FDD_FR1_B |  |  | -113.5 |
|  |  | NR_TDD_FR1_C |  |  | -113 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  | -112.5 |
|  |  | NR_FDD_FR1_E |  |  | -112 |
|  |  | NR_FDD_FR1_H |  |  | -110.5 |
|  | Config 3 | NR_FDD_FR1_A |  | -91 | -114 |
|  |  | NR_FDD_FR1_B |  |  | -113.5 |
|  |  | NR_TDD_FR1_C |  |  | -113 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  | -112.5 |
|  |  | NR_FDD_FR1_E |  |  | -112 |
|  |  | NR_FDD_FR1_H |  |  | -110.5 |
| Note2 | Config 1,2 | NR_FDD_FR1_A | dBm/SCS | -85 | -114 |
|  |  | NR_FDD_FR1_B |  |  | -113.5 |
|  |  | NR_TDD_FR1_C |  |  | -113 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  | -112.5 |
|  |  | NR_FDD_FR1_E |  |  | -112 |
|  |  | NR_FDD_FR1_H |  |  | -110.5 |
|  | Config 3 | NR_FDD_FR1_A |  | -88 | -111 |
|  |  | NR_FDD_FR1_B |  |  | -110.5 |
|  |  | NR_TDD_FR1_C |  |  | -110 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  | -109.5 |
|  |  | NR_FDD_FR1_E |  |  | -109 |
|  |  | NR_FDD_FR1_H |  |  | -107.5 |
|  |  |  | dB | -1.76 | -5.46 |
|  |  |  | dB | 3 | -4 |
| SS-RSRPNote3 | Config 1,2 | NR_FDD_FR1_A | dBm/SCS | -82 | -118 |
|  |  | NR_FDD_FR1_B |  |  | -117.5 |
|  |  | NR_TDD_FR1_C |  |  | -117 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  | -116.5 |
|  |  | NR_FDD_FR1_E |  |  | -116 |
|  |  | NR_FDD_FR1_H |  |  | -114.5 |
|  | Config 3 | NR_FDD_FR1_A |  | -85 | -115 |
|  |  | NR_FDD_FR1_B |  |  | -114.5 |
|  |  | NR_TDD_FR1_C |  |  | -114 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  | -113.5 |
|  |  | NR_FDD_FR1_E |  |  | -113 |
|  |  | NR_FDD_FR1_H |  |  | -111.5 |
| SS-RSRQ Note3 |  | NR_FDD_FR1_A | dB | -14.77 | -17.34 |
|  |  | NR_FDD_FR1_B |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  |  | NR_FDD_FR1_E |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A | dBm/ 9.36MHz | -50 | -83.5 |
|  |  | NR_FDD_FR1_B |  |  | -83 |
|  |  | NR_TDD_FR1_C |  |  | -82.5 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  | -82 |
|  |  | NR_FDD_FR1_E |  |  | -81.5 |
|  |  | NR_FDD_FR1_H |  |  | -80 |
|  | Config 3 | NR_FDD_FR1_A | dBm/ 38.16MHz | -50 | -77.4 |
|  |  | NR_FDD_FR1_B |  |  | -76.9 |
|  |  | NR_TDD_FR1_C |  |  | -76.4 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  | -75.9 |
|  |  | NR_FDD_FR1_E |  |  | -75.4 |
|  |  | NR_FDD_FR1_H |  |  | -73.9 |
| Propagation condition |  |  | - | AWGN | AWGN |
| Antenna configuration |  |  | - | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |

A.13.4.2.1.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1.29.1.1.

A.13.4.3	SS-SINR

A.13.4.3.1	Intra-frequency measurement accuracy on SCC

A.13.4.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.31.1.1.

A.13.4.3.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in table A.13.4.3.1.2-1. The absolute accuracy of SS-SINR intra-frequency measurement is tested by using the parameters in table A.13.4.3.1.2-2 and table A.13.4.3.1.2-3. In all test cases, Cell 1 is the PCell, Cell 2 is the SCell with CCA, and Cell 3 is the target cell with CCA. Two sub-tests (Test 1 and Test 2) are provided different Noc on Cells 1, 2, and 3.

Table A.13.4.3.1.2-1: SS-SINR Intra frequency SS-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | Without CCA: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

A.13.4.3.1.2-2: SS-SINR Intra frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 | Cell 2 |  | Cell 3 |
| SSB ARFCN |  |  |  | freq2 | freq2 | freq2 |  | freq2 |
| DL CCA model |  | Config 1, 2, 3 |  | As specified in clause A.3.26.2.1 |  |  |  |  |
| UL CCA model |  | Config 1, 2, 3 |  | As specified in clause A.3.26.2.2 |  |  |  |  |
| UL CCA probability |  | PCCA_UL |  | 1.0 | - | 1.0 | - |  |
| DL CCA probability for semi-static channel access Note 7, 8 |  | PCCA_DL |  | 0.9375 | - | 0.9375 | - |  |
| DL CCA probability for dynamic channel access Note 8, 9 |  | PCCA_DL_1 |  | 0.75 | - | 0.75 | - |  |
|  |  | PCCA_DL_2 |  | 0.75 | - | 0.75 | - |  |
| Duplex mode |  | Config 1, 2, 3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1, 2, 3 |  | TDDConf.1.1 CCA |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |  |
| TRS configuration |  | Config 1, 2, 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |
| PDSCH Reference measurement channel |  | Config 1, 2, 3 |  | SR.1.1 CCA |  | SR.1.1 CCA |  |  |
| RMSI CORESET Reference Channel |  | Config 1, 2, 3 |  | CR.1.1 CCA |  | CR.1.1 CCA |  |  |
| Dedicated CORESET Reference Channel |  | Config 1, 2, 3 |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |
| DBT Window configuration |  | Config 1, 2, 3 |  | DBT.1 |  |  |  |  |
| Time offset with Cell 1 |  | Config 1, 2, 3 | s | 3 (for Cell 2) | 3 | 3 (for Cell 2) |  | 3 |
| SSB configuration |  | Config 1, 2, 3 |  | SSB.1 CCA for semi-static channel access SSB.2 CCA for dynamic channel access |  |  |  |  |
| SMTC configuration |  | Config 1, 2, 3 |  | SMTC.1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2, 3 | kHz | 30 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | NR_CCA_FR1_I | dBm/15 kHz | -93 |  | -112 |  |  |
|  |  | NR_CCA_FR1_J |  |  |  | -111.5 |  |  |
| Note2 | Config 1, 2, 3 | NR_CCA_FR1_I | dBm/SCS | -90 |  | -109 |  |  |
|  |  | NR_CCA_FR1_J |  |  |  | -108.5 |  |  |
|  |  |  | dB | 0 | -3.19 | -5.46 |  | -5.46 |
|  |  |  | dB | 4.54 | 2.66 | -4 |  | -4 |
| SS-RSRPNote3 | Config 1, 2, 3 | NR_CCA_FR1_I | dBm/SCS | -85.46 | -87.34 | -113 |  | -113 |
|  |  | NR_CCA_FR1_J |  |  |  | -112.5 |  | -112.5 |
| SS-SINR Note3 |  | NR_CCA_FR1_I | dB | 0 | -3.19 | -5.46 |  | -5.46 |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  |
| IoNote3 | Config 1, 2, 3 | NR_CCA_FR1_I | dBm/ 38.16 MHz | -51.41 |  | -75.41 |  |  |
|  |  | NR_CCA_FR1_J |  |  |  | -74.91 |  |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: NR operating band groups are as defined in clause 3.5.2. NOTE 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configuration. NOTE 7: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 8: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 9: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |

A.13.4.3.1.2-3: SS-SINR Intra frequency test parameters for NR PCell

| Parameter |  |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 1 |
| SSB ARFCN |  |  |  | freq1 | freq1 |
| Duplex mode |  | Config 1 |  | FDD | FDD |
|  |  | Config 2,3 |  | TDD | TDD |
| TDD configuration |  | Config 1 |  | Not Applicable | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 | DLBWP.0.1 |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 | DLBWP.1.1 |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 | ULBWP.0.1 |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 | ULBWP.1.1 |
| DRX Cycle configuration |  |  | ms | Not Applicable | Not Applicable |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | Config 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | Config 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  |  | OP.1 | OP.1 |
| SS-RSSI-Measurement |  |  |  | Not Applicable | Not Applicable |
| SMTC configuration |  | Config 1 Config 2,3 |  | SMTC.2 | SMTC.2 |
|  |  |  |  | SMTC.1 | SMTC.1 |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 | SSB.2 FR1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 | 15 |
|  |  | Config 3 |  | 30 | 30 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  | NR_FDD_FR1_A | dBm/15 kHz | -93 | -116 |
|  |  | NR_FDD_FR1_B |  |  | -115.5 |
|  |  | NR_TDD_FR1_C |  |  | -115 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  | -114.5 |
|  |  | NR_FDD_FR1_E |  |  | -114 |
|  |  | NR_FDD_FR1_H |  |  | -112.5 |
| Note2 | Config 1,2 |  | dBm/SCS | -93 | Same as Noc for 15 kHz |
|  | Config 3 | NR_FDD_FR1_A |  | -90 | -113 |
|  |  | NR_FDD_FR1_B |  |  | -112.5 |
|  |  | NR_TDD_FR1_C |  |  | -112 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  | -111.5 |
|  |  | NR_FDD_FR1_E |  |  | -111 |
|  |  | NR_FDD_FR1_H |  |  | -109.5 |
|  |  |  | dB | 0 | -5.46 |
|  |  |  | dB | 4.54 | -4 |
| SS-RSRPNote3 | Config 1,2 | NR_FDD_FR1_A | dBm/SCS | -88.46 | -120 |
|  |  | NR_FDD_FR1_B |  |  | -119.5 |
|  |  | NR_TDD_FR1_C |  |  | -119 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  | -118.5 |
|  |  | NR_FDD_FR1_E |  |  | -118 |
|  |  | NR_FDD_FR1_H |  |  | -116.5 |
|  | Config 3 | NR_FDD_FR1_A |  | -85.46 | -117 |
|  |  | NR_FDD_FR1_B |  |  | -116.5 |
|  |  | NR_TDD_FR1_C |  |  | -116 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  | -115.5 |
|  |  | NR_FDD_FR1_E |  |  | -115 |
|  |  | NR_FDD_FR1_H |  |  | -113.5 |
| SS-SINR Note3 |  | NR_FDD_FR1_A | dB | 0 | -5.46 |
|  |  | NR_FDD_FR1_B |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  |  | NR_FDD_FR1_E |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A | dBm/ 9.36 MHz | -57.5 | -85.51 |
|  |  | NR_FDD_FR1_B |  |  | -85.01 |
|  |  | NR_TDD_FR1_C |  |  | -84.51 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  | -84.01 |
|  |  | NR_FDD_FR1_E |  |  | -83.51 |
|  |  | NR_FDD_FR1_H |  |  | -82.01 |
|  | Config 3 | NR_FDD_FR1_A | dBm/ 38.16 MHz | -51.41 | -79.41 |
|  |  | NR_FDD_FR1_B |  |  | -78.91 |
|  |  | NR_TDD_FR1_C |  |  | -78.41 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  | -77.91 |
|  |  | NR_FDD_FR1_E |  |  | -77.41 |
|  |  | NR_FDD_FR1_H |  |  | -75.91 |
| Propagation condition |  |  | - | AWGN | AWGN |
| Antenna configuration |  |  | - | 1x2 | 1x2 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |

A.13.4.3.1.3	Test Requirements

The SS-SINR measurement accuracy shall fulfil the requirements in clause 10.1.31.1.1.

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell

A.13.4.4.1	SSB based L1-RSRP measurement

A.13.4.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.33.1 for L1-RSRP measurements based on SSB with the testing configurations for NR cells in table A.13.4.4.1.1-1.

Table A.13.4.4.1.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

A.13.4.4.1.2	Test parameters

In this set of test cases there are two cells in the test, PCell (Cell 1) and a SCell under CCA (Cell 2). Cell 2 operates on a carrier frequency with CCA and transmits SSBs in DBT window according to DL CCA model.

Two sub-tests (Test 1 and Test 2) are provided with different Noc  on Cell 2. The test parameters for the Cell 1 and Cell 2 are given in table A.13.4.4.1.2-1 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in table A.13.4.4.1.2-1.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There is no measurement gap configured in the test. Before the test, UE is configured one SSB resource set with two SSB resources. On Cell 2, UE is configured to perform L1-RSRP measurement based on the SSB resources 0 and 1.

Table A.13.4.4.1.2-1: FR1 SSB based L1-RSRP test parameters

| Parameter |  | Config | Unit | Test 1 |  |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |
| Active PCell/SCell Configuration |  | 1~3 |  | PCell | SCell |  | PCell | SCell |
| SSB ARFCN |  | 1~3 |  | freq1 | freq2 |  | freq1 | freq2 |
| DL CCA model |  | 1~3 |  | N/A | As specifieed in A.3.26.2.1 |  | N/A | As specifieed in A.3.26.2.1 |
| UL CCA model |  | 1~3 |  | N/A | As specified in A.3.26.2.2 |  | N/A | As specified in A.3.26.2.2 |
| Duplex mode |  | 1 |  | FDD | TDD |  | FDD | TDD |
|  |  | 2,3 |  | TDD |  |  | TDD |  |
| TDD configuration |  | 1 |  | N/A | TDDConf.1.1 CCA |  | N/A | TDDConf.1.1 CCA |
|  |  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |
|  |  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |
| BWchannel |  | 1 | MHz | 10: NPRB,c = 52 | 40: NPRB,c = 106 |  | 10: NPRB,c = 52 | 40: NPRB,c = 106 |
|  |  | 2 |  | 10: NPRB,c = 52 |  |  | 10: NPRB,c = 52 |  |
|  |  | 3 |  | 40: NPRB,c = 106 |  |  | 40: NPRB,c = 106 |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 CCA |  | SR.1.1 FDD | SR.1.1 CCA |
|  |  | 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |
|  |  | 3 |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 CCA |  | CR.1.1 FDD | CR.1.1 CCA |
|  |  | 2 |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |
|  |  | 3 |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 CCA |  | CCR.1.1 FDD | CCR.1.1 CCA |
|  |  | 2 |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |
|  |  | 3 |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |
| SSB configuration for Semi-static channel access |  | 1 |  | SSB.3 FR1 | SSB.3 CCA |  | SSB.3 FR1 | SSB.3 CCA |
|  |  | 2 |  | SSB.3 FR1 |  |  | SSB.3 FR1 |  |
|  |  | 3 |  | SSB.4 FR1 |  |  | SSB.4 FR1 |  |
| SSB configuration for Dynamic channel access |  | 1 |  | SSB.3 FR1 | SSB.4 CCA |  | SSB.3 FR1 | SSB.4 CCA |
|  |  | 2 |  | SSB.3 FR1 |  |  | SSB.3 FR1 |  |
|  |  | 3 |  | SSB.4 FR1 |  |  | SSB.4 FR1 |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.2 TDD |  | TRS.1.1 FDD | TRS.1.2 TDD |
|  |  | 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |
|  |  | 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |
| OCNG Patterns |  | 1~3 |  | OP.1 |  |  | OP.1 |  |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration |  | 1~3 |  | SMTC.1 |  | N/A | SMTC.1 | N/A |
| DBT Window Configuration |  | 1~3 |  | N/A |  | DBT.1 | N/A | DBT.1 |
| reportConfigType |  | 1~3 |  | periodic |  |  | periodic |  |
| reportQuantity |  | 1~3 |  | ssb-Index-RSRP |  |  | ssb-Index-RSRP |  |
| Number of reported RS |  | 1~3 |  | 2 |  |  | 2 |  |
| L1-RSRP reporting period |  | 1~3 |  | slot80 |  |  | slot80 |  |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A | 1~3 | dBm/15 kHz | -94.65 |  |  | -94.65 | - |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | - |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | - |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | - |
|  | NR_FDD_FR1_E |  |  |  |  |  |  | - |
|  |  |  |  |  |  |  |  | - |
|  |  |  |  |  |  |  |  | - |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | - |
|  |  |  |  |  |  |  |  | - |
|  |  |  |  |  |  |  | - |  |
| Note2 | NR_FDD_FR1_A | 1~3 | dBm/SSB SCS | -91.65 |  |  | -91.65 | - |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | - |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | - |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | - |
|  | NR_TDD_FR1_E |  |  |  |  |  |  | - |
|  |  |  |  |  |  |  |  | - |
|  |  |  |  |  |  |  |  | - |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | - |
|  |  |  |  |  |  |  |  | - |
|  |  |  |  |  |  |  | - |  |
|  |  | 1~3 | dB | 10 |  |  | 10 | -3 |
| SS-RSRPNote3 | NR_FDD_FR1_A | 1~3 | dBm/SCS | -81.65 |  |  | -81.65 | - |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | - |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | - |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | - |
|  | NR_TDD_FR1_E |  |  |  |  |  |  | - |
|  |  |  |  |  |  |  |  | - |
|  |  |  |  |  |  |  |  | - |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | - |
|  |  |  |  |  |  |  |  | - |
|  |  |  |  |  |  |  | - |  |
| IoNote3 | NR_FDD_FR1_A | 1~3 | dBm/ 38.16 MHz | -50.19 |  |  | -50.19 | - |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | - |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | - |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | - |
|  | NR_TDD_FR1_E |  |  |  |  |  |  | - |
|  |  |  |  |  |  |  |  | - |
|  |  |  |  |  |  |  |  | - |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | - |
|  |  |  |  |  |  |  |  | - |
|  |  |  |  |  |  |  | - |  |
|  |  | 1~3 | dB | 10 |  |  | 10 | -3 |
| Propagation condition |  | 1~3 |  | AWGN |  |  | AWGN |  |
| Antenna configuration |  | 1~3 |  | 1x2 |  |  | 1x2 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

A.13.4.4.1.3	Test Requirements

In both Test 1 and Test 2, the L1-RSRP measurement accuracy for SSB#0 and SSB#1 of Cell 2 shall fulfil the requirements in clauses 10.1.33.1.

<End of Change 2>