3GPP TSG-RAN WG4 Meeting # 116	R4-2510575

Bengaluru, India, August 25th – 29th, 2025

Title: 	Discussion on conditional Intra-CU LTM

Source: 	Huawei, HiSilicon

Agenda item:	7.25.3.3

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

This contribution provides analysis on conditional intra-CU LTM from RAN4 RRM requirement perspective.

Discussion

As per the latest progress, the basic framework of CLTM delay is almost settled down. In the following, we provide our views on some undecided components.

Issue 1-2-2: Tmeasure < Agreement > In C-LTM delay, Tmeasure is measurement time. It is defined from the end of TEvent_DU until UE starts TCLTM-RRC-processing to a target cell. For SSB based L1-RSRP measurement: - Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed. - Measurement latency for both serving and candidate cells need to be considered, and follow the definition of measurement delay in the event-triggered L1 reporting delay discussed under the corresponding agenda item. For SSB based L3-RSRP measurement: - Reuse Tmeasure in CHO. For CSI-RS based L1-RSRP measurement: - For known cell, Tmeasure shall include the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period. - Further discuss the conditions for TCSI-RS_SFN_intra = 0. - Further discuss whether to include unknown cell case. If included, add one more component of TPSS/SSS. For CSI-RS based L3-RSRP measurement: - RAN4 does not define CLTM requirements for the case where CSI-RS based L3-RSRP measurements are used in the execution condition evaluation.

In legacy conditional handover, Tmeasure is the time duration when the condition event is really met at UE side (i.e., UE realize the condition of CHO is met).

The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts. For conditional intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

As both L1 measurement and L3 measurement can be used to trigger conditions of C-LTM, Tmeas can be the L3 measurement period or L1 measurement period. Regarding CSI-RS based L1 RSRP measurement, to correctly receive configured CSI-RS resource, UE shall acquire coarse timing and get the information of SFN of the candidate cell.

- Regarding the CSI-RS based L1 measurement period on neighbor cell, it is under discussion in a parallel objective. It is agreed that the precondition of CSI-RS based L1 measurement is that the candidate cell is known and UE has performed L3 measurement on the cell. So the coarse timing has been acquired for the known cell and no additional time is needed.

- The time period used to acquire the SFN information is not always need. The time can be equal to 0 if the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise, the time period used to acquire the SFN information is TCSI-RS_SFN_intra which can refer to the corresponding requirements in R16 CSI-RS based L3 measurement. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.

Proposal 1: If CSI-RS based L1-RSRP measurement is used in event:

For known cell, Tmeasure shall include the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period.

- The time period used to acquire the SFN information is equal to 0 if the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise, the time period used to acquire the SFN information is TCSI-RS_SFN_intra which can refer to the corresponding requirements in R16 CSI-RS based L3 measurement. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.

Proposal 2: Don’t consider unknown case for CSI-RS based L1 RSRP measurement.

In C-LTM, upon condition is met, after UE finished preparation operations and detached from the source cell, the steps which are defined in R18 LTM cell switch delay, i.e., TLTM-processing, Tfirst-RS, TRS-proc , TLTM-IU, shall be followed.

In R18 LTM cell switch delay:

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the uncertain time in transmit the first UL transmission.

TLTM-processing, TLTM-IU reached conclusion in RAN4#115 meeting, the controversial leftover is about Tfirst-RS and TRS-proc

Endorsed draft BigCR [R4-2508461] - Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell [stated in clause 6.3.1.3]. - TRS-proc is the time for SSB processing [stated in clause 6.3.1.3]. - Editor notes: further discuss the conditions for Tfirst-RS = 0 and TRS-proc=0.

In R18 LTM, the conditions of Tfirst-RS = 0 had been discussed for a long time. We think the main principle shall be inherited to CLTM, with only some updates required. As no cell switch command is indicated from network during CLTM procedure, the legacy description of “cell switch command” can be updated as “UE starts cell switch execution to the target cell”. As CSI-RS based L1 measurement is also supported in CLTM, the measurement period of CSI-RS resources associated to target TCI is supposed to be considered as well.

Legacy requirements: Tfirst-RS = 0 and TRS-proc= 0 under the following conditions: - The target TCI state indicated in the LTM cell switch command is in the serving cell active TCI state list, or - The UE is configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and - The target TCI state in the cell switch command is in the LTM candidate cell active TCI state list, and - the time between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is at least TCI state activation delay stated in section 8.25.3 and - the time between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is not more than TCI state activation delay stated in section 8.25.3 + 160 ms, or - the measurement period of the SSB associated to target TCI state is not larger than 160 ms after the LTM candidate cell TCI state activation MAC-CE is received, or - The target cell is an FR1 cell, and the UE is not configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and - The target TCI state in the cell switch command is in the LTM candidate cell active TCI state list, and - the time between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is at least TCI state activation delay stated in section 8. 25.3, and not more than - TCI state activation delay stated in section 8. 25.3 + 480 ms for an intra-frequency target cell, or NOTE: longer L3 measurement delay may be expected for 480 ms after TCI state activation delay stated in section 8.25.3. - TCI state activation delay stated in section 8. 25.3 + 160 ms for an inter-frequency target cell, or - The time between the latest PDCCH ordered RACH preamble transmission on the target cell and the cell switch command is not more than 160 ms. Otherwise, Tfirst-RS is the time to the first SSB transmission on the target cell after Tcmd + TLTM-RRC-processing + TLTM-processing. TRS-proc = 2 ms.

Proposal 3: Tfirst-RS = 0 and TRS-proc= 0 under the following conditions:

-	The target TCI state indicated in the LTM cell switch command is in the serving cell active TCI state list, or

-	The UE is configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and

-	The target TCI state is in the LTM candidate cell active TCI state list, and

-	the time between UE receiving the LTM candidate cell TCI state activation MAC-CE and UE starting cell switch execution to the target cell is at least TCI state activation delay stated in section 8.25.3, and not more than TCI state activation delay stated in section 8.25.3 + 160 ms, or

-	the measurement period of the SSB/CSI-RS associated to target TCI state is not larger than 160 ms after the LTM candidate cell TCI state activation MAC-CE is received, or

-	The target cell is an FR1 cell, and the UE is not configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and

-	The target TCI state is in the LTM candidate cell active TCI state list, and

-	the time between UE receiving the LTM candidate cell TCI state activation MAC-CE and UE starting cell switch execution to the target cell is at least TCI state activation delay stated in section 8. 25.3, and not more than

-	TCI state activation delay stated in section 8. 25.3 + 480 ms for an intra-frequency target cell, or

NOTE:	longer L3 measurement delay may be expected for 480 ms after TCI state activation delay stated in section 8.25.3.

-	TCI state activation delay stated in section 8. 25.3 + 160 ms for an inter-frequency target cell, or

-	The time between the latest PDCCH ordered RACH preamble transmission on the target cell and UE starting cell switch execution to the target cell is not more than 160 ms.

Conclusion

The contribution provides analysis on intra-CU LTM from RAN4 RRM requirement perspective. The following proposals are proposed:

Proposal 1: If CSI-RS based L1-RSRP measurement is used in event:

For known cell, Tmeasure shall include the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period.

- The time period used to acquire the SFN information is equal to 0 if the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise, the time period used to acquire the SFN information is TCSI-RS_SFN_intra which can refer to the corresponding requirements in R16 CSI-RS based L3 measurement. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.

Proposal 2: Don’t consider unknown case for CSI-RS based L1 RSRP measurement.

Proposal 3: Tfirst-RS = 0 and TRS-proc= 0 under the following conditions:

-	The target TCI state indicated in the LTM cell switch command is in the serving cell active TCI state list, or

-	The UE is configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and

-	The target TCI state is in the LTM candidate cell active TCI state list, and

-	the time between UE receiving the LTM candidate cell TCI state activation MAC-CE and UE starting cell switch execution to the target cell is at least TCI state activation delay stated in section 8.25.3, and not more than TCI state activation delay stated in section 8.25.3 + 160 ms, or

-	the measurement period of the SSB/CSI-RS associated to target TCI state is not larger than 160 ms after the LTM candidate cell TCI state activation MAC-CE is received, or

-	The target cell is an FR1 cell, and the UE is not configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and

-	The target TCI state is in the LTM candidate cell active TCI state list, and

-	the time between UE receiving the LTM candidate cell TCI state activation MAC-CE and UE starting cell switch execution to the target cell is at least TCI state activation delay stated in section 8. 25.3, and not more than

-	TCI state activation delay stated in section 8. 25.3 + 480 ms for an intra-frequency target cell, or

NOTE:	longer L3 measurement delay may be expected for 480 ms after TCI state activation delay stated in section 8.25.3.

-	TCI state activation delay stated in section 8. 25.3 + 160 ms for an inter-frequency target cell, or

-	The time between the latest PDCCH ordered RACH preamble transmission on the target cell and UE starting cell switch execution to the target cell is not more than 160 ms.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2504910, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom