3GPP TSG-RAN WG4 Meeting # 116	R4-2510577

Bengaluru, India, August 25 – 29, 2025

Title: 	Discussion on general aspects in AIML mobility

Source: 	Huawei, HiSilicon

Agenda item:	7.19.2

Document for:	Discussion

# Introduction

The revised SID on AI/ML for mobility in NR agreed in [1]. For mobility enhancement, AI is introduced to predict the RRM measurement and event prediction. The identified use cases are AI based cell-level/beam-level measurement prediction, AI based HO failure/RLF prediction and AI based measurement event prediction. RAN4 is expected to evaluate testability, interoperability, and impacts on RRM requirements and performance.

Revised SID on AIML for mobility [1]: Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] … - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

This contribution focuses on general aspects in AI/ML mobility.

# 2. General aspects of AI mobility in RAN4

## 2.1 Interoperability and generalization

TP on RAN4 aspects for TR 38.744 [2] 6.2.4 Interoperability RAN4 has not concluded on any interoperability aspect in this study item. Editor Note: RAN4 may further discuss on this issue in RAN4#116, any further agreements will be captured in this clause. 6.2.5 Generalization RAN4 has not concluded on any generalization aspect in this study item. Editor Note: RAN4 may further discuss on this issue in RAN4#116, any further agreements will be captured in this clause.

According to TR 38.843, interoperability primarily pertains to two-sided models. Given that models are deployed on both the network side and the UE side, the two-sided models must be compatible to ensure interoperability between the UE and the NW. In contrast, AI-mobility models, which are deployed either on the UE side or the network side, are one-sided models and do not involve interoperability issues.

Proposal 1: Conclude that no issues are identified for interoperability in AI-mobility.

There are two approaches to conducting generalization testing. The first is to define multiple test cases under different test conditions. If the DUT passes all these test cases, the AI/ML functionality can be deemed to meet the generalization requirements restricted to the defined conditions. The second approach is to vary the test conditions during a single test. If the performance meets the requirements under changing conditions, the generalization requirement is satisfied.

For the second approach, procedures such as model switching or model monitoring in LCM may be involved. However, there is no need to couple model inference with other LCM procedures within a single test. Therefore, the first approach is more preferable. Detailed test conditions for generalization can be discussed on a case-by-case basis.

Proposal 2: Generalization can be ensured by defining several test conditions. Test conditions will not change during one test. Test conditions will be discussed case by case.

## 2.2 Training and testing dataset

In RAN2, the AI model is trained by system level channel model and verified in terms of the accuracy of measurement prediction under the same channel model. However, in RAN4 legacy, only link level channel model is considered for defining test cases. There are two potential testing options for RAN4 in AI RRM measurement prediction.

  - Option 1: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4.

  - Option 2: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4.

Proposal 3: RAN4 will study the following two potential testing options for defining the requirement of prediction accuracy.  Pros and Cons need to be analyzed.

  - Option 1: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4.

  - Option 2: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4.

There are two options for defining the testing and training dataset. One is using the channel model and the sampling rules. Another is that the dataset is specified or partially specified in a sample-by-sample manner.

Proposal 4: RAN4 will study the following two potential options for defining the testing and training dataset.

  - Option 1: Use the channel model and the sampling rules.

  - Option 2: Specify or partially specify the testing dataset in a sample-by-sample manner.

# 3. Conclusion

The contribution provides analysis on RRM measurement prediction in AI/ML mobility. The following proposals are proposed:

Proposal 1: Conclude that no issues are identified for interoperability in AI-mobility.

Proposal 2: Generalization can be ensured by defining several test conditions. Test conditions will not change during one test. Test conditions will be discussed case by case.

Proposal 3: RAN4 will study the following two potential testing options for defining the requirement of prediction accuracy.  Pros and Cons need to be analyzed.

  - Option 1: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4.

  - Option 2: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4.

Proposal 4: RAN4 will study the following two potential options for defining the testing and training dataset.

  - Option 1: Use the channel model and the sampling rules.

  - Option 2: Specify or partially specify the testing dataset in a sample-by-sample manner.

# 4. Reference

- RP-242393, “Revised SID on AIML for mobility in NR”, 3GPP RAN#105, September 9-12, 2024.

- R4-2508419, “TP on RAN4 aspects for TR 38.744”, 3GPP RAN4#115, May, 2025.