3GPP TSG-RAN WG4 Meeting # 116	R4-2510578

Bengaluru, India, August 25th – 29th, 2025

Title: 	Discussion on impacts for RRM measurement prediction

Source: 	Huawei, HiSilicon

Agenda item:	7.19.3.1

Document for:	Discussion

Introduction

The revised SID on AI/ML for mobility in NR agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI The study will focus on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively. Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2] - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation - NOTE 1: RAN1/3 work can be triggered via LS NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 3: Two-sided model is not included

This contribution focuses on RRM measurement prediction in AI/ML mobility.

Discussion

General aspects

In RAN2, the evaluation scenarios for measurement prediction with prioritization are listed in [TR38.744].

Table 5.2.1-1 prioritization of evaluation scenarios [TR38.744]

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

There are two study goals. The 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements. The 2nd study goal is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption) [2].

During RAN2#126 meeting, companies had sufficient discussion on the scenario priority with the target study goal. The final conclusion is to set scenario 6 (FR2 to FR2 intra-frequency spatial domain with the first goal) as the middle priority. We suggest RAN4 to follow RAN2 priority at the current stage, and then further analyse middle priority scenarios after sufficient progress on high priority scenarios. In addition, in R19 AI/ML interface, spatial domain predication is being carried out for beam management. Some kinds of methodology can be leveraged or inherited to AI/ML mobility scenario 6.

Proposal 1: RAN4 to focus on the discussion on scenario 2, 3 and 4 with high priority in this release.

Measurement prediction accuracy

Relative RSRP accuracy definition

The definition of relative RSRP accuracy was discussed for several meeting cycles and achieved below agreement [3]. There are two bullets of FFS. In the following, we provide analysis on these.

Issue 2-1: Relative RSRP accuracy definition Agreement: - For intra-frequency L3-RSRP, - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), - cell 1 and cell 2 are on the same frequency - the reported RSRP of cell2 can be measured or predicted. - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy - Relative RSRP accuracy for Beam level measurements is FFS during WI phase depending upon RAN2 progress. - For inter-frequency L3-RSRP, - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1 - the reported RSRP of cell2 can be measured or predicted. - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy

The first FFS regards the time difference between the two reported RSRPs from different cells. As per the legacy relative RSRP accuracy definition, there is no restriction on the time interval between two measured cells. From specification perspective, even the time interval between two compared cells is large, UE is still required to satisfy the relative measurement accuracy. However in practical, as the reported RSRPs from different cells are triggered by certain Event or reported periodically, the time separation between the reported RSRP would not be too long.

- 10.1.2.1.2 Relative SS-RSRP Accuracy The relative SS-RSRP accuracy is defined as the SS-RSRP measured from one cell compared to the SS-RSRP measured from another cell on the same frequency, or between any two SS-RSRP levels measured on the same cell in FR1. The accuracy requirements in this clause are also applicable when highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 is configured.

Proposal 2: Not consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy.

The second FFS is whether the relative accuracy of beam-level L3 RSRP is needed or not. To facilitate discussion, the high-level measurement model in TS38.300 without AI is duplicated as below. Regarding the RSRP accuracy for Beam level measurements, the refence point is at point E, which is defined as,

E: a measurement (i.e. beam-specific measurement) after processing in the beam filter. The reporting rate is identical to the reporting rate at point A1. This measurement is used as input for selecting the X measurements to be reported.

Figure 9.2.4-1: Measurement Model [in TS38.300]

On the one hand, as we know, the beam-level measurement report includes up to maxNrofRS-IndexesToReport SSB indexes in order of decreasing beam sorting quantity whose sorting quantity is above absThreshSS-BlocksConsolidation. Therefore the relative comparison is indeed performed among the beams from the same cell. Moreover the existing definition of relative RSRP accuracy has already included the beam level measurements (it states that “between any two SS-RSRP levels measured on the same cell”).

On the other hand, in TR38.744, 3 sub-use cases for beam level measurement prediction are introduced, however L3 filtered beam level prediction sub-use cases are lower priority in RAN2.

3 sub-use cases are considered for beam-level RRM measurement prediction: Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s) Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) Editor Note 1: L3 filtered beam level prediction sub-use cases are lower priority and simulation should focus on FR2 intra-frequency temporal domain case A

So we think due to limited time in R19 SI, Relative RSRP accuracy for Beam level measurement can be not be considered in R19.

Proposal 3: Due to limited time in SI, Relative RSRP accuracy for Beam level measurement can be not be considered in R19.

Accuracy requirements of predicted RSRP

Issue 2-2: Accuracy requirements for the predicted RSRPs - Proposals and Observations: - Proposal 1 (Ericsson): Predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurement reporting for mobility purposes. - Proposal 2 (Nokia): For temporal domain Case-B prediction, RAN4 may consider accuracy performance requirements similar to those of the legacy relative and absolute accuracy performance defined for measurement. - Proposal 3 (ZTE): RAN4 should discuss whether to define separate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases. - Recommended WF

In TR38.744, 3 sub-use cases for cell-level measurement prediction are considered.

3 sub-use cases are considered for cell-level RRM measurement prediction: Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

In sub-use case 1, the prediction error is generated from actual L1 beam level predicting L1 beam level measurement (prediction is executed at point A1).

In sub-use case 2, the prediction error is generated from actual L3 beam level predicting L3 beam level measurement (prediction is executed at point C).

In sub-use case 3, the prediction error is generated from actual L1 beam level predicting L3 beam level measurement (measurement result at point A1 predicting measurement result at Point C).

In the three cases, the inference model is used at different position in the measurement process. The AI prediction model algorithm would be various due to different input and output. As per the latest RAN2 simulation results, s majority of the companies observe that RRM sub-use case 2 demonstrates higher prediction accuracy than RRM sub-use case 1 and RRM sub-use case 3 at least with short PW length for FR2 intra-frequency temporal domain case A.

From RAN4 defining requirements perspective, the point needs to be discussed is whether RAN4 needs to define prediction accuracy for each sub-use case/scenario or RAN4 only define one cell level prediction accuracy requirement to accommodate 3 sub-use cases/scenario.

If use-case specific prediction accuracy is defined, UE needs to know which prediction accuracy is to be satisfied. The scenario and sub-use case shall be known to UE. This issue is related with RAN2, however RAN2 will not discuss it in R-19.

If it is expected to define one unified prediction accuracy, based on simulation results RAN4 can select one sub-use case which has proper or acceptable prediction accuracy, or RAN4 can determine one prediction accuracy to cover all sub-use cases. The prerequisite of both ways shall be based on simulation results.

Regarding to whether the prediction accuracy shall at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurement, it is premature to draw conclusion in R-19 SI phase.

Proposal 4: For prediction accuracy, in R19 SI phase it is premature to draw conclusion on

- define separate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases;

- whether the prediction accuracy shall at least meet the accuracy requirements that are defined for the non-AI/ML RRM measurement.

Ground truth definition for RSRP Accuracy

In RAN4#113 meeting, for AI mobility, a high-level definition of L3 RSRP predication accuracy is agreed in [4].

Issue 2-1-9a: L3 RSRP prediction accuracy definition Agreement: - For testing, Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP - FFS Ground truth definition of L3-RSRP for FR1 and FR2

It is agreed in [5] that for the ground truth of L3 RSRP in FR2, reported L3 RSRP measurement under sufficient high SNR is taken as a baseline. With the sufficient high SNR, the propagation environment can be regarded as “no noise”. Moreover the conclusion is aligned with the ground truth definition in R19 AL/ML interface WI for beam management.

Issue 2-1-2: Ground Truth Definition for RSRP accuracy Agreement: Take Option 1-1-FR2 as baseline, and further discuss any additional aspects need to be considered. Option 1-1-FR2: The ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2.

The ground truth definition for L3 RSRP accuracy in FR1 is still under discussion [5]. There are two alternatives.

Issue 2-1-2: Ground Truth Definition for RSRP accuracy - Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list

- Alternative 1: Ground truth for FR1 is based on the TE transmitted or reception power.

Channel conditions

In legacy FR1 conducted measurement accuracy test, UE antenna is directly connected to TE. The channel environment is AWGN.  As there is no channel fluctuation in time domain, TE can determine the ideal L3-RSRP result. For AI/ML based measurement prediction, to verify AI prediction capability, AWGN channel is not proper, and fading channel is to be employed.

Observation 1: Fading channel is to be employed for AI mobility measurement prediction.

The measurement time instances alignment between UE and TE

The time instance of ground truth of L3 RSRP shall be aligned with that of the predicted L3 RSRP. Then TE shall have the information on the specific time instance when UE performs prediction. As discussed above, for either intra-frequency temporal domain case B and case A, although the configuration of PW, OW and skipping pattern are known to TE, the specific position in time domain of UE obtaining a predicted result is unknown to TE.

Observation 2: For either intra-frequency temporal domain case B and case A, although the configuration of PW, OW and skipping pattern are known to TE, the specific position in time domain of UE obtaining a L3 predicted result is unknown to TE.

Number of samples of L1 measurement

In legacy L3 measurement, the measurement accuracy is specified at point A and no L1 filter is applied. For AI/ML measurement prediction, the L3 RSRP measurement is specified at point C. Then the ground truth of L3 RSRP result shall be determined by the linear average with multiple L1 measurement samples. Although the current L3 RSRP measurement period is defined based on 5 L1 samples, the actual number of L1 samples is up to UE implementation.

Observation 3: The number of L1 measurement samples is unknown to TE.

Based on the above analysis, the factors of measurement time instances alignment between UE and TE and number of samples of L1 measurement are obstacles to use ground truth definition Alt1.

Proposal 5: If the ground truth in FR1 is defined based on the TE transmitted or reception power (Alter 1), the following aspects are obstacles:

- The specific position in time domain of UE obtaining a L3 predicted result is unknown to TE.

- The number of L1 measurement samples is unknown to TE.

- Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions

In general, no big obstacles are observed if the ground truth for FR1 is defined based on UE reported value (Alt 2). One tricky point is that UE may cheat, as UE also know its reported value. The cheating issue is in parallel under discussion in R19 AI interface beam management test. We can wait more progress on R19 AI interface beam management

Proposal 6: The ground truth in FR1 can be defined based on the reported measurement value under certain conditions.

- how to avoid UE cheating can wait the progress in R19 AI interface beam management test.

Impacts from observation and prediction windows

Scenarios of intra-frequency temporal domain and inter-frequency frequency domain for measurement prediction are captured in [TR38.744].

Table 5.2.1.1-1: Prioritization of evaluation scenarios

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | intra-cell |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Agreements in RAN2#126 Chairman notes 7 To agree on methodology of Intra_F_C_T_Case A as following: Intra-frequency intra-cell temporal domain case A prediction is done by predicting measurement result(s) in prediction window based on measurement results in observation window of the same cell for both FR1_to_FR1 and FR2_to_FR2. FFS aligning the prediction window 8 To agree on methodology of Intra_F_C_T_Case B: Intra-frequency intra-cell temporal domain prediction is done by predicting sub set measurement (case B) instances in temporal domain of the same cell for both FR1_to_FR1 and FR2_to_FR2. Several measurement reduction rates should be aligned among companies. The detail values for measurement reduction rate are FFS.

We know temporal domain case A and case B have different working mechanism. For both case A and case B, OW (observation window) and PW (prediction window) are introduced [2].

Figure 5.2.1-1 Example of intra-frequency temporal domain case A [TR38.744]

In intra-cell temporal domain prediction case A, observation window is a duration UE performs actual measurement. Within an observation window, the measurement period shall follow the legacy measurement period. The prediction window is a duration UE predicts measurement. Observation window and prediction window will slide. It means that the instances in previous prediction window would become part of the observation window when UE perform measurement in predicted instances. It shall be noted that the observation window length depends on model performance, channel environment and observation length, etc.

The below observations for FR2 intra-frequency temporal domain case A are made in RAN2 [TR 38.744]:

- Higher UE speed correlates with decreased prediction accuracy

- Longer PW length correlates with decreased prediction accuracy

- Increasing the OW length can improve the prediction accuracy, especially when the OW length is relatively short. However, once the OW length exceeds a certain value, further increase of the OW length does not yield significant benefit.

To guarantee prediction accuracy for FR2 intra-frequency temporal domain case A, we think limited doppler shift (corresponds to limited UE speed), limited PW length and proper OW length shall be set.

Proposal 7: For intra-frequency temporal domain prediction case A, RSRP prediction accuracy is to be defined under the side condition of certain OW and PW length.

Proposal 8: To guarantee prediction accuracy for FR2 intra-frequency temporal domain case A, limited doppler shift, limited PW length and proper OW length shall be set in testing.

In FR1 intra-frequency temporal domain prediction case B, the measurement results in PW are predicted and these time instances are completely skipped by UE. RAN2 provide two examples duplicated as below, and example 2 is recommended as baseline for evaluation.

Figure 5.2.1-2 Example 1 of intra-frequency temporal domain case B [TR38.744]

Figure 5.2.1-3 Example 2 of intra-frequency temporal domain case B [TR38.744]

Measurement Reduction Rate in temporal domain (MRRT) is used for case B in RAN2 evaluation work [TR38.744].

MRRT = skipped measurement time instances / total measurement time instances

It is observed that,

- Under different MRRT (e.g., 1:1, 2:3), the prediction accuracy would be different. Increasing MRRT would result in decreased prediction accuracy.

- Regarding example1 in figure 5.2.1-2 and example 2 in Figure 5.2.1-3, MRRT for both examples are the same, that is 0.5. However the prediction error may be not the same, even with the same MRRT value. Taking an extreme example, following the predication strategy like example 1, using100 measurements in OW predict 100 measurements in PW, MRRT is still 0.5. However the prediction accuracy of the sample in the farthest instance in PW would be degraded. In other words, even under the same MRRT setting, different measurement skipping patterns can result in different prediction accuracy.

Therefore the accuracy requirements for intra-frequency temporal domain prediction case B shall be specified for certain MRRT with some limitations, e.g., PW length.

Proposal 9: For intra-frequency temporal domain prediction case B, RSRP prediction accuracy is to be defined under the side condition of MRRT with limited PW length.

For inter-frequency inter-cell frequency domain measurement, the idea is to leverage the RSRP result on one cell to infer the RSRP on another cell in another frequency. The prediction accuracy in frequency domain would be evaluated and defined in RAN4 RRM. With different channel correlation coefficient between two inter-frequency carriers, the measurement prediction accuracy is different. In RAN2, only co-located scenario is considered where the cell to be measured and the cell to be predicted are located in the same sector/site with the same antenna configuration. RAN2 had some simulations and observed that

- the corelation between different frequency in FR1 has high correlation, e.g., correlation coefficient is ~0.9;

- the higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy.

Proposal 10: For inter-frequency inter-cell frequency domain prediction, RSRP prediction accuracy in collocated scenario can be specified under certain channel correlation coefficient, e.g., ~0.9.

Cluster approach is under discussion in RAN2. Using cluster approach or single cell approach would impact on prediction accuracy. In RAN2, it is observed that cluster cells as input can outperform the prediction accuracy than single cell as input. From accuracy definition perspective, either the AI prediction model is based on cluster approach or single cell approach is up to implementation, unless RAN2 specifies certain explicit indication and corresponding UE capability. Therefore it needs further discussion on whether cluster/single cell approach is one side condition for prediction accuracy, or specify minimum accuracy requirements to accommodate both approaches.

RAN2#128 Agreements

For inter-frequency, cluster as input (i.e. measurements from different cells as inputs )can improve the prediction accuracy than single cell as input. For temporal domain, the gains are unclear. RAN2 will focus on frequency domain for cluster based approach.

In cluster approach the model takes measurements from more than one cell as inputs.

Proposal 11: For inter-frequency inter-cell frequency domain prediction, RAN4 to discuss

- Specify RSRP prediction accuracy for cluster approach and single cell approach separately, or

- Specify minimum RSRP prediction accuracy to accommodate both cluster and single cell approach

Conclusion

The contribution provides analysis on RRM measurement prediction in AI/ML mobility. The following proposals are proposed:

Proposal 1: RAN4 to focus on the discussion on scenario 2, 3 and 4 with high priority in this release.

Proposal 2: Not consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy.

Proposal 3: Due to limited time in SI, Relative RSRP accuracy for Beam level measurement can be not be considered in R19.

Proposal 4: For prediction accuracy, in R19 SI phase it is premature to draw conclusion on

- define separate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases;

- whether the prediction accuracy shall at least meet the accuracy requirements that are defined for the non-AI/ML RRM measurement.

Observation 1: Fading channel is to be employed for AI mobility measurement prediction.

Observation 2: For either intra-frequency temporal domain case B and case A, although the configuration of PW, OW and skipping pattern are known to TE, the specific position in time domain of UE obtaining a L3 predicted result is unknown to TE.

Observation 3: The number of L1 measurement samples is unknown to TE.

Proposal 5: If the ground truth in FR1 is defined based on the TE transmitted or reception power (Alter 1), the following aspects are obstacles:

- The specific position in time domain of UE obtaining a L3 predicted result is unknown to TE.

- The number of L1 measurement samples is unknown to TE.

Proposal 6: The ground truth in FR1 can be defined based on the reported measurement value under certain conditions.

Proposal 7: For intra-frequency temporal domain prediction case A, RSRP prediction accuracy is to be defined under the side condition of certain OW and PW length.

Proposal 8: To guarantee prediction accuracy for FR2 intra-frequency temporal domain case A, limited doppler shift, limited PW length and proper OW length shall be set in testing.

Proposal 9: For intra-frequency temporal domain prediction case B, RSRP prediction accuracy is to be defined under the side condition of MRRT with limited PW length.

Proposal 10: For inter-frequency inter-cell frequency domain prediction, RSRP prediction accuracy in collocated scenario can be specified under certain channel correlation coefficient, e.g., ~0.9.

Proposal 11: For inter-frequency inter-cell frequency domain prediction, RAN4 to discuss

- Specify RSRP prediction accuracy for cluster approach and single cell approach separately, or

- Specify minimum RSRP prediction accuracy to accommodate both cluster and single cell approach

Reference

- RP-242393, Revised SID on AIML for mobility in NR, OPPO

- TR38.744

- R4-2508268, WF on AI/ML Mobility Part 1, Nokia

- R4-2420095, WF on AI/ML Mobility, Nokia

- R4-2502594, WF on AI/ML Mobility, Nokia