3GPP TSG-RAN WG4 Meeting # 116	R4-2510583

Bengaluru, India, August 25th – 29th, 2025

Title: 	Discussion on performance requirements for low band carrier aggregation via switching

Source: 	Huawei, HiSilicon

Agenda item:	7.3.5

Document for:	Discussion

Introduction

WI on low band carrier aggregation via switching is approved in [1].

- 4.1 Objective of SI or Core part WI or Testing part WI Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

The contribution provides further analysis on performance requirements for low band carrier aggregation via switching.

Discussion

- SA event triggered reporting tests without gap

Although whether the scaling factor of Kp-LB due to LB-CA switching pattern shall be applied on the L3 measurement on activated SDL SCell/PCC carrier is still under discussion in core requirements, the L3 measurement requirement on PCC shall be tested, because LB-CA switching scheme is a new functionality, and the UE L3 measurement behaviour shall be test under this scenario. Accordingly, the measurement accuracy test shall be also defined.

Proposal 1: For LB-CA switching, SA event triggered reporting test without gap on PCC and accuracy test are to be specified.

As we know, in legacy the event trigger reporting test on SCC is only tested for FR2 HST CA. In R19 LB-CA switching, mobility function is based on PCC.  SCell change rarely occurs since two carrier aggregation is considered. It seems it is not typical to configure event on SDL for reporting. We also understand that companies would like to test the UE measurement behaviour. So we have no strong view on whether to define event trigger reporting test on SDL carrier.

Proposal 2: Further discussion on whether to define event trigger reporting test on SDL carrier for LB-CA switching.

- Interruption due to SDL SCell activation

The switching pattern specified in RAN1 is only applicable to activated carriers. It is agreed in RAN4 that the legacy deactivated SCell measurement delay requirements are applied for UE to perform deactivated SDL SCell measurement. As the legacy measurement requirement is reused, RAN4 not to define SA event triggered reporting test on deactivated SCC.

Moreover the interruptions on active serving cell(s) due to L3 measurement on deactivated SDL carrier is quite different with the legacy requirements. It is straight forward to specify test for interruptions during measurements on deactivated SDL carrier in FR1. In our understanding, even without defining event triggering reporting test, defining interruption test on deactivated SDL can effectively validate UE behaviour on deactivated SDL.

Proposal 3: For LB-CA switching, test case of interruption during measurements on deactivated SDL carrier is to be specified.

- SDL SCell activation delay

R18 SSB based FR1 inter-band SCell activation delay requirement can be reused to SDL SCell activation in LB CA via switching. However the interruption or scheduling restriction during SDL SCell activation may be different with the legacy. So it is straight forward to specify test case for SDL SCell activation delay.

Proposal 4: For LB-CA switching, test case of SDL SCell activation delay is to be specified.

- BFD/CBD, TCI state switch and L1-RSRP/SINR requirements for SDL SCell

Issue 1-3-10: BFD/CBD requirements for SDL SCell Agreement: - If RAN4 to define requirement for BFD/CBD for SDL SCell in LB CA via switching, the BFD/CBD on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

As we know in FR1, there are multiple transmit beams from network. That is why RAN4 agreed to define requirements for TCI state switch delay requirement and L1 RSRP measurement for SDL SCell. So regarding BFD/CBD on SDL SCell, we don’t observe the strong objection to define corresponding core requirements.

Issue 1-3-11: TCI state switch delay requirement for SDL SCell Agreement: - The L1-RSRP measurement and T/F tracking for TCI switching on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

Issue 1-3-13: L1-RSRP/SINR measurement requirements for SDL SCell Agreement in AH: - For L1-RSRP/SINR measurement requirement for SDL SCell, UE can only perform such measurement on the overlapped occasions of L1 RS and SDL SCell reception duration indicated by switching pattern.

Based on the above agreements in RAN4, the following test cases are to be specified for SDL SCell:

Test case for Beam Failure Detection and Link Recovery Test on SDL SCell (depending on conclusion of core requirements)

Test case for Active TCI state switch delay on SDL SCell

Test case for L1-RSRP measurement for beam reporting on SDL SCell

Test case for L1-SINR measurement for beam reporting on SDL SCell

Proposal 5: For LB-CA switching, the following test cases are to be specified on SDL SCell:

Test case for Beam Failure Detection and Link Recovery Test on SDL SCell (depending on conclusion of core requirements)

Test case for Active TCI state switch delay on SDL SCell

Test case for L1-RSRP measurement for beam reporting on SDL SCell

Test case for L1-SINR measurement for beam reporting on SDL SCell

In LB CA switching, the SSB/CSI-RS based L3/L1 measurement, TCI state switching, BFD/CBD evaluation on PCC are supposed to be performed on the overlapped occasions between SMTC window and PCC duration in switch pattern. Most test specified for SDL carrier shall be also verified on PCC in LB CA via switching.

Proposal 6: For LB-CA switching, the below test cases are to be specified on FDD PCell:

Test case for Beam Failure Detection and Link Recovery Test on FDD PCell

Test case for Active TCI state switch delay on FDD PCell

Test case for L1-RSRP measurement for beam reporting on SDL SCell

Test case for L1-SINR measurement for beam reporting on SDL SCell

Conclusions

This contribution provides further analysis on performance requirements for low band carrier aggregation via switching. The following proposals are provided:

Proposal 1: For LB-CA switching, SA event triggered reporting test without gap on PCC and accuracy test are to be specified.

Proposal 2: Further discussion on whether to define event trigger reporting test on SDL carrier for LB-CA switching.

Proposal 3: For LB-CA switching, test case of interruption during measurements on deactivated SDL carrier is to be specified.

Proposal 4: For LB-CA switching, test case of SDL SCell activation delay is to be specified.

Proposal 5: For LB-CA switching, the following test cases are to be specified on SDL SCell:

Test case for Beam Failure Detection and Link Recovery Test on SDL SCell (depending on conclusion of core requirements)

Test case for Active TCI state switch delay on SDL SCell

Test case for L1-RSRP measurement for beam reporting on SDL SCell

Test case for L1-SINR measurement for beam reporting on SDL SCell

Proposal 6: For LB-CA switching, the below test cases are to be specified on FDD PCell:

Test case for Beam Failure Detection and Link Recovery Test on FDD PCell

Test case for Active TCI state switch delay on FDD PCell

Test case for L1-RSRP measurement for beam reporting on SDL SCell

Test case for L1-SINR measurement for beam reporting on SDL SCell

Reference

- RP-243317, New WID on low band carrier aggregation via switching

- RP-242532, Motivation for a new work item on low band carrier aggregation via switching in Rel-19

- R4-2508259, WF on RRM requirements for NR_LBCA_Sw, Apple